3GPP TSG-SA WG1 Meeting #110 	S1-252395

Fukuoka, Japan, 19-23 May 2025	(revision of S1-252341)

Source:	Nokia

Title:	Updated Use case on end-to-end energy efficiency improvement for the network and UE

Draft TS/TR:	3GPP TR 22.870 v.0.2.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	laurent-walter.goix@nokia.com

Abstract: This use case about end-to-end energy efficiency improvement for both the network and UE is improved to consider network energy saving.

Rev 2395

- Added contact in header

- Updated PRs based on feedback from conference call

  - Removed the word “dynamically” in PR2 [vivo]

  - Generalized PR2/3 [Samsung, ZTE, NTT DOCOMO]

  - Changed UE preference to user preference in PR2 [Huawei]

  - Clarified UE and/or network energy saving in PR3 [ZTE]

---------- Use Case template ----------

## 5.6.1	Use case on end-to-end energy efficiency improvement for the network and UE

### 5.6.1.1	Description

In the ITU-R Recommendation "Framework and overall objectives of the future development of IMT for 2030 and beyond" [39], sustainability is considered as the design principles commonly applicable to all usage scenarios. Supporting end-to-end energy efficiency is an important design target for the sustainability of 6G system.

Regards to end-to-end energy efficiency of 6G system, UE energy efficiency is an important part. Power consumption of modem (including both baseband and RF) has contributed more and more to total UE power consumption due to the large number of power consuming features supported, e.g. AI and computing as well as the strong demand of high data volume transmission. Energy efficiency design for UE could further extend UE battery life while guaranteeing certain service quality. In the 6G system, it is expected that the network could assist the UE to improve energy efficiency as one of the services, e.g. UE could subscribe to energy efficiency services to realize power saving based on network assistance.

The UE could also assist the network to improve the system energy efficiency to further reduce operation cost for mobile operators. So, the UE and network could mutually benefit from the coordination for energy efficiency improvement.

Considering the above, the energy efficiency and energy saving should consider the end-to-end performance including both the network and the UE. Considering the sustainability target of the 6G system, it should greatly improve the end-to-end energy efficiency compared to the 5G system.

Figure 5.6.1.1-1: Supporting end-to-end energy efficiency and energy saving

Energy efficiency shall be a quantifiable metric of sustainability and is important for the success of IMT-2030 technology. The energy efficiency KPIs should be defined to ensure that IMT-2030 is designed in an energy efficient way. From the operator's point of view, the KPIs help the operator to decide how much the network energy efficiency is improved by applying a specific energy-saving technology. Then, based on the quantified KPIs, the network could provide energy efficiency improvement services for the subscribers. From the subscriber's point of view, the KPIs clearly show that the energy efficiency improvement services are important for them to reduce the power consumption for the data transmission, so they are more willing to subscribe to the services from the network. This would also be a new business model for the operators. In general, the energy efficiency KPIs are important and helpful to both the operators and the subscribers.

Energy efficiency should be defined taking into account both communication performance metric and energy consumption. According to the GSMA study [3] and TR 32.972 [4], several measurements means for energy efficiency are proposed by considering the energy consumption and the data transmission. Communication related performance (e.g. data volume, latency, data rate, etc.) of data transmission should be considered for the energy efficiency measurements. For example, energy saving by reducing the data rate cannot always satisfy the subscribers' requirements. It is also important for the network to realize energy saving with guaranteed QoS. However, subscribers may request or accept a service adapted by the network (e.g., a level of data rate or QoS degradation) in order to meet objectives such as UE energy savings and/or receiving incentives and/or helping network sustainability.

With regards to end-to-end energy efficiency, the energy related information collection from radio access network, e.g. at per UE, or per application granularity, should also be considered. Radio conditions would affect the energy consumption of the UE for the same amount of data volume transmission. For example, the UE location in the Cell affects the energy consumption, e.g. the UE located at the cell edge consumes more energy compared to the one located at the cell centre for the same amount of data volume transmission.

Considering the energy efficiency at the UE, it is closely related with the user experience. Based on different technologies in the 6G network to improve the energy efficiency, the operator may offer different energy efficiency improvement services to the subscribers. Based on UE's indication, the network may apply certain power saving technologies to improve energy efficiency at the UE (and/or at the network) to satisfy the user's requirement or preference on energy efficiency. This energy efficiency improvement service for the subscribers is also a new business model for the operators in 6G system. The new business model is also the driven for the 6G system deployment. The energy efficiency improvement service would further benefit the operators and subscribers to achieve the energy efficiency target in the 6G system.

### 5.6.1.2	Pre-conditions

The operator has offered a series of energy efficiency improvement services in the 6G network for the UE to save energy and extend the battery life. Different energy efficiency improvement services may be provided to the subscribers at different prices or incentives. The subscribers may order certain energy efficiency improvement services.

For example, the operator A may offer a green subscription which includes the following kinds of energy efficiency improvement services where UE device is configured with options and corresponding rules related to different energy efficiency improvement services at network registration time:

-	Energy efficiency improvement service A: Improving the energy efficiency by reducing UE energy consumption while maintaining the service experience (e.g. low latency, high throughput).

-	Energy efficiency improvement service B: Improving the energy efficiency by reducing UE energy consumption for essential service access, e.g. voice call only. The essential service list is based on the agreement between the subscriber and operator.

-	Energy efficiency improvement service C: Improving the energy efficiency by reducing both the UE energy consumption and the network energy consumption. In this case the network may choose to degrade the QoS of certain QoS flows where the level of QoS degradation is based on the agreement between the subscriber and operator or the negotiation between UE and network initiated by UE or network. This degradation results in incentives given to the subscriber.

Note that energy efficiency improvement services A and B could also reduce network energy consumption, upon to the network’s decision, to reduce its own energy consumption considering user/UE preference, as long as the service is best-effort or this does not impact QoS for services with QoS criteria.

Bob subscribes to a green subscription and buys a pair of AR glasses and wants to wear it to access AR content during the trip. Normally, the battery of his AR glasses can last for two hours. Since it is not possible to find a power source during the trip for the AR glasses, he orders the energy efficiency improvement service A for the glasses from the operator.

Bob also has a cell phone. Sometimes his cell phone is about to run out of battery, but he does not want to miss some important calls, or still wants to access certain applications (e.g. navigation map application). He prefers the network to provide network access for essential services (e.g. voice call) for his cell phone in order to improve the energy efficiency and extend the battery life. So, he orders the energy efficiency improvement service B with providing his preference on the essential service list for his cell phone.

### 5.6.1.3	Service Flows

1. When visiting the pyramids, Bob wears his AR glasses to access specific AR tour guide information. Usually, the battery of his AR glasses can last for two hours. Considering that the tour might last for four hours, in order to save power for this AR glasses, Bob indicates to the network to apply energy efficiency improvement service A to minimize the power consumption of the AR glasses while ensuring the specific QoS. Based on the indication, the network applies energy efficiency improvement service A for Bob's AR glasses.

2. In the meantime, the network may have decided to perform a network energy saving action by shutting down the RAN node which Bob’s AR glasses are connected to, and offloading Bob’s AR glasses to a neighbour RAN node. As Bob already indicated to the network to apply energy efficiency improvement service A to minimize the power consumption of the AR glasses, network searches for a neighbour RAN node for offloading Bob’s glasses that does not cause to increase energy consumption of Bob’s AR glasses (e.g., a neighbour RAN node that is not much far from Bob in comparison to the current serving RAN node). In this case, both UE and network energy efficiency can be improved.

3. After the long trip, Bob needs to take the bus to go back home. If there is no unexpected issue, it takes nearly one hour. He finds that the battery of his cell phone is below 10% and he cannot charge his cell phone on the bus. So he indicates to the network to apply energy efficiency improvement service B for his cell phone. Based on the indication, the network applies energy efficiency improvement service B for Bob's cell phone. The network only provides limited network access to the essential services (e.g. voice call) as predefined by Bob in order to reduce the UE energy consumption.

4. One hour after taking the bus, Bob has an online meeting with its colleagues. However, because of a road accident and heavy traffic, Bob is still in the bus and has to run the meeting using his cell phone. He cannot change the video call to audio call and can only turn his own camera off (e.g., because the director of their company is talking through camera or one colleague is sharing and presenting his slides to the team). He indicates to the network to apply energy efficiency improvement service C so that he can attend the meeting even if with lower quality and energy consumption, since this time he doesn’t need to present anything. This will allow him to receive some incentive.

### 5.6.1.4	Post-conditions

During the visit to the pyramids, Bob is able to watch AR tour guide information with his AR glasses with good service experience. The AR glasses is used for four hours with good service experience due to the energy efficiency improvement service A.

During the trip back home, due to energy efficiency improvement service B, although the battery of his cell phone is below 10%, Bob's cell phone could still access the basic services (e.g. voice call) for nearly one hour. This can be easily changed during the trip to accommodate his video call which was initially due at home.

### 5.6.1.5	Existing features partly or fully covering the use case functionality

The energy efficiency requirements for 5G system have been defined in clauses 6.15 and 6.15a of TS 22.261 [14], including the general requirements, requirements for energy related information as service criteria, requirements for energy states, requirements for monitoring and measurement, requirements for information exposure, requirements for network actions leveraging energy efficiency as service criteria.

The study on Energy Efficiency as service criteria Phase 2 is under study in SA1 R20. The related use cases and requirements to the 5G system are introduced in TR 22.883 [44].

Some of the procedures and network functions proposed in TR 23.700-66 [5] can be used to realize part of the 5G requirements and service flows described in TS 22.261 [14] above.

The 6G system requirements on the energy efficiency and energy saving are based on the existing requirements in the 5G.

### 5.6.1.6	Potential New Requirements needed to support the use case

### [PR 5.6.1.6-1] The 6G system shall improve energy efficiency of 6G system compared to 5G system.

### [PR 5.6.1.6-2] The 6G system shall provide means for a UE to indicate a preference in energy saving decisions of 6G system compared to 5G system.

NOTE:	Preference can e.g. be related to UE radio condition, device battery level, or user preference indicated to the UE

### [PR 5.6.1.6-3] The 6G system shall provide means for UEs to perform actions (e.g. adjust access network selection, user plane path selection) based on UE preference in energy saving decisions, which could target network and/or UE energy saving.