3GPP TSG-SA WG1 Meeting #110 	S1-252578

Fukuoka, Japan, 19-23 May 2025	(revision of S1-252562, S1-252244)

Source:		NICT

pCR Title:		Use Case on Ubiquitous Connectivity with Enhanced Multi-Access Functionality

Draft Spec:		3GPP TR22.870 v0.2.1

Agenda Item:			8.1.5

Document for:		Approval

Contact:			Hikaru Kawasaki (h_kawasaki@nict.go.jp)

Hideshi Murai (hideshi.murai@nict.go.jp)

Abstract: We propose a new use case on enhanced multi-access functionality to ensure ubiquitous connectivity.

1. Introduction

This contribution proposes a new use case on enhanced multi-access functionality to ensure ubiquitous connectivity in uncovered or scarcely covered areas, such as rural, remote, and sparsely populated regions.

2. Reason for Change

The use case lacked the gap analysis refer to SA1 requirements and SA2 specs.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.2.1.

* * * First Change * * * *

8.x	Use Case on Ubiquitous Connectivity with Enhanced Multi-Access Functionality

8.x.1	Description

The use case enables ubiquitous connectivity in rural, remote, and sparsely populated areas through enhanced multi-access functionality, supporting dynamic selection and combination of multi-access networks (TNs and NTNs composed of 3GPP and non-3GPP deployment, potentially across different operators) for applications such as IoT, telehealth, and service robot operations. Multi-access functionality includes traffic flow control (including the ability to direct traffic) across two or more simultaneous sessions to meet performance requirements like high availability, throughput, reliability, low latency, and time jitter stability, which a single operator network may not provide due to coverage limitations, bandwidth constraints, and diverse network deployments.

8.x.2	Pre-conditions

The environment consists of heterogeneous access networks (including TNs and NTNs) in rural, remote, and sparsely populated areas. Where feasible, the system supports access to networks from different operators to enhance coverage. Users are authorized to access these networks, and UE supports multi-access functionality for dynamic network selection and traffic flow control across two or more simultaneous sessions.

8.x.3	Service Flows

The UE dynamically selects and combines the multiple access networks composed of TNs and NTNs (potentially from different operators) in rural, remote, and sparsely populated areas, using multi-access functionality to apply traffic flow control across two or more simultaneous sessions to meet performance requirements such as network availability, throughput, reliability, low latency, time jitter stability. For example:

- In an agricultural IoT scenario, the UE combines networks, potentially from different operators, for reliable sensor data collection with low power consumption.

- In a telehealth scenario, the UE prioritizes networks for reliable video communication to support remote healthcare.

- In a service robot scenario, the UE maintains continuous connectivity with low latency across networks

8.x.4	Post-conditions

The UE achieves ubiquitous connectivity in rural, remote, and sparsely populated areas through enhanced multi-access functionality, including traffic flow control across multiple network sessions (TNs and NTNs, potentially from different operators where feasible). This ensures the target performance such as network availability, throughput, reliability, low latency, time jitter stability, for applications like IoT, telehealth, or service robot operations.

8.x.5	Existing features partly or fully covering the use case functionality

The 5G supports multi-access functionality, as defined in TS 22.261 [14] Clause 6.3.2 (multi-access requirements) and Clause 6.5.0 (connectivity requirements), and

TR 33.754 for Access Traffic Steering, Switching, and Splitting (ATSSS) and DualSteer.

TS 22.261 describes multi-access to 3GPP and non-3GPP networks and does not fully address connectivity in uncovered areas. The ATSSS and DualSteer enable the muti-access to a 3GPP network and a non-3GPP network and two 3GPP access networks, respectively.

These features partially cover the proposed use case but require evolution in 6G to support enhanced multi-access functionality, including traffic flow control across TNs and NTNs, non-3GPP accesses, different operators for uncovered areas.

8.x.6	Potential New Requirements needed to support the use case

[PR8.x.6-1] The 6G system shall support a UE to connect to multiple networks simultaneously, including TNs and NTNs, to enhance connectivity in rural, remote, and sparsely populated areas, which may belong to different operators.

Note: the above may enable enhanced multi-access functionality, including traffic flow control across two simultaneous sessions, including IoT, telehealth, and service robot applications.

End of CHANGES