3GPP TSG-SA WG1 Meeting #110	S1-252705

19-23 May 2025, Fukuoka, Japan	(revision of S1-252124,0922)

Source:	vivo

pCR Title:	Use case on service experience optimization in deep indoor area

Draft Spec:	3GPP TR 22.870 v0.2.1

Agenda item:	8.1.2	System and Operation Aspects

Document for:	Approval

Contact:	Hank YU, yuhang.txyjy@vivo.com

Yanchao Kang, kangyanchao@vivo.com

Xuanbing Liu, xuanbing.liu@vivo.com

Erhao Song, erhao.song@vivo.com

Abstract: this contribution described a use case related to service experience optimization when the UE is approaching the dense urban area with limited coverage.

* * * First Change * * * *(all new text)

5.x	Use case on service experience optimization in deep indoor area

5.x.1	Description

In current network deployments, most scenarios offer a smooth and reliable communication experience. However, certain deep indoor areas, in which there can be concrete walls/floors between the devices and environment, still have challenges in providing high quality communication experience. In these deep indoor area, the wireless signal quality fluctuates rapidly (e.g., when entering or exiting buildings, passing under driveway bridges, being in an underground parking lot, or using an elevator).

Here are two examples of unsatisfactory service experiences in the deep indoor areas:

- In the case of underground parking garages, the rapid change in wireless signal quality can significantly impact mobile voice communication. Issues like slow data transmission speeds and unstable connections may arise. For instance, when a user engaged in a business call enters an underground garage, there’s a high possibility that the call will drop. It will take considerable time to resume the call, causing significant inconvenience and a poor user experience.

- In the case of elevator, the network's inability to quickly recognize changes of the UE state/location may degrade entertainment services experience such as VR gaming. Imagine a user wearing a VR headset, when he/she enters an elevator, service data flow of VR gaming may be interrupted for a few seconds. Such service interruption undermines the continuity of the VR gaming experience.

The primary cause of poor service experiences in the deep indoor areas is that the wireless signal is easily blocked or reflected by obstacles in these environments (e.g., underground garages, elevators), resulting in weakened signal strength.

Consequently, in order to improve service experience in these deep indoor areas, 6G operator are anticipated to use specific approaches to minimize the service interruption when the UE is in the deep indoor area.

5.x.2	Pre-conditions

Operator X determines some deep indoor area (e.g., the underground parking garage X, elevator Y, elevators and carriages of the high-speed train) in its 6G network through field measurements or statistical information (e.g., during the deployment stage). When the 6G network knows the UE is in the deep indoor area, the network node takes actions to minimize the service interruption in order to improve users’ service experience.

Alice and Bob are subscribers of operator X.

5.x.3	Service Flows

Scenario 1—Underground parking garage

Step 1: Alice is on a business call while she enters the underground parking garage X. The network detects that Alice’s devices are approaching such a deep indoor area (i.e., underground parking garage X).

Step 2: The network takes actions for Alice’s devices to minimize service interruption of communication services in the deep indoor area.

Step 3: Alice has a better service experience with the help of network minimizing service interruption. The dropped calls that used to happen in the underground parking garage no longer occur.

Scenario 2—Elevator

Step 1: Bob is wearing a VR headset. Bob uses his VR devices to access VR content through the 3GPP network, while he is going to enter the elevator Y. The network detects that Bob’s devices are approaching such a deep indoor area (i.e., elevator Y).

Step 2: The network takes actions for Bob’s devices to minimize service interruption of VR services in the deep indoor area.

Step 3: Bob can enjoy a high-quality VR entertainment experience with the help of network minimizing service interruption.

5.x.4	Post-conditions

Both Alice and Bob can make calls to their friends, watch high-definition streaming media, and play XR games with tolerable service interruptions.

5.x.5	Existing features partly or fully covering the use case functionality

“Deep indoor” term is used in TS 22.261, which implies that there can be concrete walls / floors between the devices. It is to be places like e.g. elevators, building’s basement, underground parking lot.

Clause 7.5.2 of TS 22.261 gives some KPI requirements for highly reliable machine type communication under the service area, such as deep indoor area, which is meant to be places like e.g. elevators, building’s basement, underground parking lot. Such 5G requirements focus on High-availability IoT and are not applicable for the commercial normal UE. Besides, 5G requirements (e.g., KPIs) are applicable when the UE locates in deep indoor area, not focus on the timing of the UE approaching the deep indoor area, which may cause some serious signalling interruptions. So, service continuity insurance for the UEs (eMBB) entering the deep indoor is not supported in 5G.

In LTE, CoMP (Coordinated Multi-Point) technology enables the physical cooperation among different transmission points (normally located in different gNBs) to ensure the data transmission and reduce the cell interference. Such technology requires a good connection (e.g., fibre-optic link) among multiple points (e.g., gNBs). It can be interpreted as a traditional measurement-based handover mechanism.

Regarding the measurement-based handover procedure, it is popularly applied to ensure service continuity. The network sends a handover command to the UE according the measurement report sent by the UE. However, when the UE is approaching the deep indoor area quickly, the UE may fail to send the measurement report  to the network (e.g., due to the current signalling quality is good and the timing for the UE measurement is not triggered) or receive the handover command from the network (e.g., due to the rapidly-deteriorating radio link). In a word, legacy handover cannot fully resolve the long service interruption in the deep indoor area.

5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-1] The 6G system shall be able to provide a mechanism to minimize interruption for the UE when UE is approaching the deep indoor area.

NOTE:	“Deep indoor” term implies that there can be concrete walls/floors between the devices. It to be places like e.g. elevators, building’s basement, underground parking lot. Here, the deep indoor area is pre-determined by operator and the wireless signal quality degrades dramatically when UEs approaching such deep indoor area.

* * * End of Change * * * *