3GPP TSG- SA1 Meeting # 111	S1-253044

25-29 August 2025, Goteborg, Sweden

# Source:	SK Telecom

# Title:	New miniWID on Access Category for Unattended Data Traffic

# Document for:	Agreement

# Agenda Item:	New Work Items

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: New miniWID on Access Category for Unattended Data Traffic

Acronym: UDTraf-REQ

Unique identifier:

Potential target Release: Rel-20

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x | x |  |  |
| No | x |  |  | x |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
| x | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 660013 | Stage 1 for Application specific Congestion control for Data Communication | SA1 stage 1 work item |

Dependency on non-3GPP (draft) specification: N/A

3	Justification

While Unattended Data Traffic is generally not problematic under normal conditions, it can cause issues in congestion scenarios. In congestion scenarios such as subway stations during rush hour, it can be easily captured that excessive access attempts lead to lower RACH success rate. Also, when we investigated the types of access attempts, a significant portion of access attempts was found in live networks to originate from Unattended Data Traffic—that is, from UEs transmitting data without any active user interaction.

The 5G system provides a single unified access control (UAC) where operators control accesses based on Access Identities and Access Categories. Three approaches for controlling access attempts of Unattended Data Traffic can be considered based on the existing list of access categories, each with its inherent limitations:

- Access Category 1 (=delay-tolerant): Applicable only when the UE is configured for NAS signalling low priority. Typical UE, e.g. smartphones, do not meet this condition.

- Operator-defined Access Category via App ID: Specific applications known to generate Unattended Data Traffic are mapped to an operator-defined access category. This approach assumes that Unattended Data Traffic can be identified on a per-app basis, but unattended traffic is often context-dependent, not app-specific.

- Operator-defined Access Category via URSP Routing: With a specific URSP routing policy on Connection Capability, Background traffic (e.g., Connection Capability identifier "10101000" indicates background traffic) is routed to a dedicated slice and that slice is mapped to an operator-defined access category. This approach requires dedicated slicing per traffic type, which imposes operational burden.

Since the existing list of access categories does not cover the access attempts from Unattended Data Traffic, we propose the introduction of a new access category for Unattended Data Traffic.

4	Objective

The objective of this work is to address the following requirements for the 5G system:

- Specify a new access category for Unattended Data Traffic

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| N/A | N/A | N/A | N/A | N/A | N/A |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 22.261 | Add a new access category for Unattended Data Traffic | TSG#101 | Table 6.22.2.3-1 |

6	Work item Rapporteur(s)

Kim, Hyunjin, SK Telecom, hyunjinbrownk@sktelecom.com

7	Work item leadership

SA WG1

8	Aspects that involve other WGs

None

9	Supporting Individual Members

Supporting IM name

SK Telecom

LG Uplus

Verizon

Huawei

Hisilicon

China Unicom

China Telecom

vivo