3GPP TSG-SA WG1 Meeting #111 	S1-253300

Goteborg, Sweden, 25-29 August 2025

Title:	Update Use case 5.8.1 on end-to-end energy efficiency improvement for the network and UE

Agenda Item:	8.1.2

Source:	Nokia

Contact:	Laurent-Walter Goix

Abstract: This contribution proposes to update the use case and add new related requirements by clarifying the opportunity and target for network energy efficiency improvement and energy saving when addressing the “end-to-end” aspect, besides UE energy efficiency. In addition, mentions of existing energy saving features on UE and network side have been added. Finally, the text refering to the ongoing R20 5G study has been updated.

---------- Use Case template ----------

* * * First Change * * * *

## 5.8.1	Use case on end-to-end energy efficiency improvement for the network and UE

#### 5.8.1.1	Description

In the ITU-R Recommendation "Framework and overall objectives of the future development of IMT for 2030 and beyond" [39], sustainability is considered as the design principles commonly applicable to all usage scenarios. Supporting end-to-end energy efficiency is an important design target for the sustainability of 6G system.

Regards to end-to-end energy efficiency of 6G system, UE energy efficiency is an important part. Power consumption of modem (including both baseband and RF) has contributed more and more to total UE power consumption due to the large number of power consuming features supported, e.g. AI and computing as well as the strong demand of high data volume transmission. Energy efficiency design for UE could further extend UE battery life while guaranteeing certain service quality. In the 6G system, it is expected that the network could assist the UE to improve energy efficiency as one of the services, e.g. UE could subscribe to energy efficiency services to realize power saving based on network assistance. The network could apply energy saving technologies to improve the UE energy efficiency.

The UE could also assist the network to improve the system energy efficiency to further reduce operation cost for mobile operators. The UE could also help the network to reduce network energy consumption or improve the energy efficiency. So, the UE and network could mutually benefit from the coordination for energy efficiency improvement.

Considering the above, the energy efficiency and energy saving should consider the end-to-end performance including both the network and the UE. Considering the sustainability target of the 6G system, it should greatly improve the end-to-end energy efficiency compared to the 5G system.

Figure 5.8.1.1-1: Supporting end-to-end energy efficiency and energy saving

Energy efficiency shall be a quantifiable metric of sustainability and is important for the success of IMT-2030 technology. The energy efficiency KPIs should be defined to ensure that IMT-2030 is designed in an energy efficient way. From the operator's point of view, the KPIs help the operator to decide how much the network energy efficiency is improved by applying a specific energy-saving technology. Then, based on the quantified energy efficiency KPIs, the network could provide energy saving services for the subscribers. From the subscriber's point of view, the energy efficiency KPIs clearly show that the energy saving services are important for them to reduce the power consumption for the data transmission, so they are more willing to subscribe to the services from the network. This would also be a new business model for the operators. In general, the energy efficiency KPIs are important and helpful to both the operators and the subscribers.

Energy efficiency should be defined taking into account both communication performance metric and energy consumption. According to the GSMA study [3] and TR 32.972 [4], several measurements means for energy efficiency are proposed by considering the energy consumption and the data transmission. Communication related performance (e.g. data volume, latency, data rate, etc.) of data transmission should be considered for the energy efficiency measurements. For example, energy saving by reducing the data rate cannot always satisfy the subscribers' requirements. It is also important for the network to realize energy saving with guaranteed QoS.

With regards to end-to-end energy efficiency, the energy related information collection from radio access network, e.g. at per UE, or per application granularity, should also be considered. Radio conditions would affect the energy consumption of the UE for the same amount of data volume transmission. For example, the UE location in the Cell affects the energy consumption, e.g. the UE located at the cell edge consumes more energy compared to the one located at the cell centre for the same amount of data volume transmission.

Considering the energy efficiency at the UE, it is closely related with the user experience. Based on different technologies in the 6G network to improve the energy efficiency, the operator could offer different energy saving services to the subscribers. Based on the UE's preference or subscription, the network may apply certain power saving technologies to improve energy efficiency at the UE to satisfy the user's requirement or preference on energy efficiency. This energy saving service for the subscribers is also a new business model for the operators in 6G system. The new business model is also the driven for the 6G system deployment. The energy saving service would further benefit the operators and subscribers to achieve the energy efficiency target in the 6G system.

#### 5.8.1.2	Pre-conditions

The operator has offered a series of energy saving services in the 6G network for the UE to save energy and extend the battery life. Different energy saving services may be provided to the subscribers at different prices or incentives. The subscribers may order certain energy saving services.

For example, the operator A may offer the following two kinds of energy efficiency improvement services:

-	Energy saving service A: Improving the energy efficiency by reducing UE energy consumption while maintaining the service experience (e.g. low latency, high throughput). Energy saving service A will improve the UE energy efficiency by extending the UE battery life (e.g. the UE battery life is extended from two hours to four hours.) without compromising the quality of use service experience

-	Energy saving service B: Improving the energy efficiency by reducing UE energy consumption for essential service access, e.g. voice call only. The essential service list is based on the agreement between the subscriber and operator. Energy saving service B will extend the UE battery life with limited communication service.

Bob buys a pair of AR glasses and wants to wear it to access AR content during the trip. Normally, the battery of his AR glasses can last for two hours. Since it is not possible to charge the AR glasses during the trip, he orders the energy saving service A for the glasses from the operator.

Bob also has a cell phone. Sometimes his cell phone is about to run out of battery, but he does not want to miss some important calls, or still wants to access certain applications (e.g. navigation map application). He prefers the network to provide network access for essential services (e.g. voice call) for his cell phone in order to improve the energy efficiency and extend the battery life. So, he orders the energy saving service B from the operator and provides his preference on the essential service list for his cell phone.

In parallel, Bob’s network is running algorithms which try to improve the network energy efficiency whenever possible, by considering appropriate solutions such as bringing certain network components (e.g., RAN nodes) to energy saving state or finding a more energy efficient access types without compromising the energy efficiency of UEs, in line with the provided energy saving services A and B.

#### 5.8.1.3	Service Flows

1. When visiting the pyramids, Bob wears his AR glasses to access specific AR tour guide information. Usually, the battery of his AR glasses can last for two hours. Considering that the tour might last for four hours, in order to save power for this AR glasses, Bob indicates to the network to apply energy saving service A to minimize the power consumption of the AR glasses while ensuring the specific QoS. Based on the indication, the network applies energy saving service A for Bob's AR glasses. Based on internal energy efficiency algorithms, the network decides not to put the nodes serving Bob in some energy saving state.

2. After the long trip, Bob needs to take the bus to go back home. It takes nearly one hour. He finds that the battery of his cell phone is below 10% and he cannot charge his cell phone on the bus. So he indicates to the network to apply energy saving service B for his cell phone. Based on the indication, the network applies energy saving service B for Bob's cell phone. The network only provides limited network access to the essential services (e.g. voice call) as predefined by Bob in order to reduce the UE energy consumption. Based on internal energy efficiency algorithms, the network decides it can put the nodes serving Bob in some energy saving state, thus also reducing its own energy consumption.

#### 5.8.1.4	Post-conditions

During the visit to the pyramids, Bob is able to watch AR tour guide information with his AR glasses with good service experience. The AR glasses is used for four hours with good service experience due to the energy saving service A.

During the trip back home, due to energy saving service B, although the battery of Bob’s cell phone is below 10%, it could still access the basic services (e.g. voice call) for nearly one hour.

#### 5.8.1.5	Existing features partly or fully covering the use case functionality

The energy efficiency requirements for 5G system have been defined in clauses 6.15 and 6.15a of TS 22.261 [14], including the general requirements, requirements for energy related information as service criteria, requirements for energy states, requirements for monitoring and measurement, requirements for information exposure, requirements for network actions leveraging energy efficiency as service criteria.

R20 use cases and requirements to the 5G system have been documented in TR 22.883 [44].

Some of the procedures and network functions proposed in TR 23.700-66 [5] can be used to realize part of the 5G requirements and service flows described in TS 22.261 [14].

In general, several UE and/or network energy saving features were defined in 5G. For example:

- Network energy saving features requiring UE support (dedicated UE capabilities): on-demand SSB and SIB1, cell DTX / DTX, multi-CSI for mMIMO muting, etc.

- UE energy saving features requiring UE support (dedicated UE capabilities): (e)DRX, PDCCH monitoring reduction features such as PDCCH skipping and Search Space Set Switching, RRC Inactive, Paging Early Indication and Radio and CN-based Paging Subgrouping, PSM, MICO

However, their applicability to trigger energy savings on both UE and network sides is limited due to the limited implementation of such features in 5G UEs and networks.

The 6G system requirements on energy efficiency and energy saving are based on the existing requirements in the 5G.

The 5G system only specified the monitoring and measurement for energy consumption in 6.15a.4 of TS 22.261 [14] as following:

Subject to operator's policy, the 5G network shall support energy consumption monitoring at per network slice and per subscriber granularity.

Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to monitor energy consumption for serving this 3rd party.

Subject to operator policy and regulatory requirements, the 5G system shall be able to monitor the energy consumption for serving the 3rd party, together with the network performance statistic information for the services provided by that network, related to same time interval e.g. hourly or daily.

Note the last requirement above only covers monitoring of ‘the energy consumption for serving the 3rd party’ and ‘network performance statistic information’, which are too coarse to estimate the energy efficiency of the 6G system.

The estimation of energy efficiency related information needs to be improved in 6G.

#### 5.8.1.6	Potential New Requirements needed to support the use case

[PR 5.8.1.6-1] The 6G system shall improve energy efficiency of 6G system compared to 5G system.

[PR 5.8.1.6-2] The 6G system shall be able to support mechanisms to improve UE and/or network energy efficiency when providing services to the subscribers.

[PR 5.8.1.6-3] The 6G system shall provide means for a user to provide a preference for UE and/or network energy efficiency/saving.

NOTE 1:	The 6G system takes into account this preference when improving UE and/or network energy efficiency. Examples of preference are user preference on prioritizing service performance, prioritizing UE energy efficiency, or prioritizing essential service (e.g. voice call) while improving UE energy efficiency.

[PR 5.8.1.6-4] The 6G system shall provide means to identify specific target UEs (e.g. persistently causing low network efficiency, based on preference) to whom adjust the provided service in order to optimize network energy saving.

[PR 5.8.1.6-5] The 6G system shall provide means for UEs to perform actions (e.g. adjust access network selection, user plane path selection, conditional handover) which could target UE and/or network energy efficiency, based on UE and/or network energy-related characteristics.

[PR 5.8.1.6-6] The 6G system shall provide means to support end-to-end (i.e., UE and network) energy saving with mandatory UE support.

NOTE 2:	This could be achieved by reusing (some) 5G-defined capabilities and/or by defining a small set of new capabilities to reduce complexity with respect to 5G.