3GPP TSG-SA WG1 Meeting #111 	S1-253616

25-29 August 2025, Goteborg, Sweden	(revision of S1-253102r1)

Title:	Use case on service robots for power grid

Agenda Item:	8.1.3

Source:	ZTE, China Telecom, China Unicom

Contact:	Lin Chen (chen.lin23@zte.com.cn)

Xu Xia (xiaxu@chinatelecom.cn)

Menghan Yu (yumh1@chinatelecom.cn)

Qun Wei (weiqun5@chinaunicom.cn)

Abstract: <provide a short description of the content>

This contribution proposes a use case about service robots, which enables the 6G network to provide communication service for service robots. Moreover, 6G network can provide LLM/VLM inference via service hosting environment for service robots’ advanced autonomy control.

1. Introduction

This contribution proposes a use case about service robots, which enables the 6G network to provide multi-modal communication service for service robots and provide LLM/VLM inference via service hosting environment for service robots’ advanced autonomy control.

- Reason for Change

S1-253102

- clarify the KPI requirement for end-to-end latency as well as AI inference latency

- remove the original PR-2

S1-253102r1

- add in note 4 to clarify that the E2E latency is achievable via the service hosting environment provided by 6G network.

- remove the further clarification of communication latency.

S1-253616

- The same as S1-253102r1, remove unnecessary revision marks

3. Proposal

It is proposed to agree the following new use cases to 3GPP TR 22.870 v0.3.1.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[5.1X]	QIAN LI et all, “6G Cloud-Native System: Vision, Challenges, Architecture Framework and Enabling Technologies” in IEEE Access, Received 19 August 2022, accepted 1 September 2022, date of publication 8 September 2022, date of current version 19 September 2022.

[6.1x]	International Federation of Robotics (IFR) (2022), Position Paper: Artificial Intelligence in Robotics

[6.1z]	Sven Behnke, etc, “Predicting away robot control latency” in Proceeding of 7th Robo Cup international Symposium, 2003.

[6.1m]	Yiyang Huang, etc, “Software-hardware co-design for embodied AI robots” in International Symposium on Computer Architecture (ISAC), 2025.

* * * Second Change * * * *

This is a new use case.

6.x	Use case on service robot for power grid

6.x.1	 Description

Service robot offers greater versatility by performing complex tasks that require dexterity, adaptability and interaction with human-centric environment. Thanks to major breakthroughs in artificial intelligence (AI), machine learning and advanced materials, service robots (e.g. humanoid robot) are poised for considerable growth over the next decade and present a massive investment opportunity.

According to a report released by Markets and Markets in January 2025, the global humanoid robot market is expected to grow from US$2.03 billion in 2024 to US$13.25 billion in 2029. Meanwhile, Goldman Sachs predicted in February 2024 that the total addressable market for humanoid robots could reach US$38 billion by 2035. Since service robots are more flexible and capable of adapting to complex terrain, they are particularly appealing for tasks that are “dangerous, dirty, and dull”, for example, mining, disaster rescue, grid inspection, nuclear reactor maintenance, and chemicals manufacturing, etc. Customers are willing to pay a higher price for robots that can do dangerous jobs that people are reluctant to do.

For example, the service robots can be deployed in power grids, enabling inspection, maintenance and emergency response operations in extreme environment. As shown in Figure 1, it is the high-altitude ultra-high voltage AC project in southwest China, which covers 1316 kilometers of 1000kV lines and four 1000kV substations. With the high altitude (e.g. 3450m) and mountainous terrain, it is hard for human technicians to go to the site. The service robots(e.g. humanoid robot) may mimic human movements and navigate the existing power grid infrastructures(e.g. narrow stairs, climb ladders), which are expected to play an important role in power grid inspection. The service robots can tolerate the hazards and continue working with the hand-operated tools, which enhances workers’ safety while ensuring service continuity.

Figure 1 The Ultra-high voltage line and 1000kV substation in southwest China

On the other hand, different levels of robot autonomy and intelligence [6.1x] have been developed. For the robots with higher level of autonomy, they are growing fast with the utilization of large AI models (e.g. LLM, VLM, SLAM). With service robot as an example, the motion control such as walking, grasping, turning, balance, posture control, reflexes and corrective actions, can be supported via embedded controller and chips for fast response. However, the large AI models for advanced autonomy are too heavy to run efficiently on embedded hardware of service robot. Offloading the computing-intensive “brain” of service robot to nearby 6G network(e.g. service hosting environment) can enable lightweight service robots with limited onboard power. To support this, the service robot needs to send multi-modal data(e.g. sensor data, LiDAR, video, voice, etc) to the 6G network. These multi-modal data can then be processed via the large AI model to generate robot control commands and natural language responses appropriately. With the support of pervasive computing capability in 6G network, the service robot for power grid inspection can respond instantly to complex situations with real-time perception-action loops as shown in Figure 2.

Figure 2 Illustration of service robot for power grid

6.x.2	 Pre-conditions

PowerGrid Corp, a leading energy provider, manages over 300,000 km of transmission lines, 600 substations, and remote renewable energy plants across mountains, forests, and coastal zones. Due to climate change, PowerGrid Corp suffers from rising maintenance costs and dangerous inspection conditions. In order to improve operation efficiency, they purchase 500 service robots for grid inspection, maintenance and emergency response.

The 6G network is operated by operator A. Operator A is responsible for the communication service provision around the power grid area. Moreover, Operator A owns sufficient computing power and support edge intelligence. The PowerGrid Corp subscribe the communication service and AI service from Operator A which provide low latency communication service as well as the large AI model inference service via service hosting environment for service robots in the field.

One day, a massive high-voltage power grid runs across snowy peaks and rugged valleys. Ice storms, falling rocks, and wildlife make this terrain not only remote but dangerous for human technicians. To ensure uninterrupted power delivery to millions of people, an service robot named Voltus is deployed to inspect, maintain, and report anomalies in the grid infrastructure.

6.x.3	 Service Flows

- Voltus is airlifted by drone to a remote grid substation. Upon landing, it connects to the 6G terrestrial network for communication. Meanwhile, it activates its simultaneous localization and mapping system to map the area using LiDAR and vision.

- Voltus walks along the grid lines. A human technician in intelligent control center connects remotely and speaks to the service robot:“Voltus, inspect the substation.”

- Voltus uses thermal imaging to detect overheating in transformers. Meanwhile, it takes video of the substation. The multi-modal data (e.g. the thermal image, video stream, sensor data and LiDAR data) are sent to the service hosting environment within 6G network.

- Upon receiving the multi-modal data, the video recognition model in service hosting environment analyzes the input data and detects objects based on the visual and thermal patterns match.

- On top of that, the large AI models (e.g. LLM, VLM, SLAM) in service hosting environment perform reasoning and planning, and generates the robot control commands and natural language responses. For example, the LLM generate the analysis report: two insulators show thermal anomalies and one grounding wire is partially detached. The LLM in service hosting environment generates robot control commands: 1) tighten the loosened bolt using a multi-tool; 2) replace the two insulators.

- The analysis report and robot control command are sent to Voltus. Based on the command, Voltus’s embedded system performs motion control, which executes walking, balancing and climbing a small pylon. Then Voltus replaces the cracked ceramic insulators from its utility belt.

- When tightening the bolt, Voltus further queries the large AI model in service hosting environment:“What torque should be applied to a 500kV line bolt with rust mitigation coating?” The large AI model in service hosting environment replies:“Apply 45 Nm torque using non-conductive wrench. Confirm line de-energized.” Upon receiving the reply from large AI model in service hosting environment, Voltus tightens the loosened bolt using a hand-operated tool accordingly.

- After a while, a human technician in intelligent control center speaks to Voltus:“Voltus, confirm insulator replacement, send close-up.”

- Voltus uses speech recognition and sends the annotated images to the human technician. Moreover, Voltus responses to the human technician using speech :“Insulator integrity verified. ”

- Upon receiving the responses from Voltus, the human technician checks the images and speaks to the Voltus robot:“Repair successful. Recommend re-inspection in 2 weeks.”

- Upon completion of the substation inspection, Voltus can be airlifted by drone to another grid substation for inspection.

6.x.4	 Post-conditions

Due to the service robots based grid inspection and maintenance, long term cost saving and higher operational efficiency can be achieved for the power grid.

- x.5 Existing features partly or fully covering the use case functionality

| Specifications and clause | Existing Requirements | Gap Analysis |
| --- | --- | --- |
| TS 22.104 [64] clause A.2.2.3 | A mobile robot essentially is a programmable machine able to execute multiple operations, following programmed paths to fulfil a large variety of tasks. This means, a mobile robot can perform activities like assistance in work steps, collaboration with other robots, e.g. for car assembly, and transport of goods, materials and other objects. Mobile robot systems are characterised by a maximum flexibility in mobility relative to the environment, with a certain level of autonomy and perception ability, i.e., they can sense and react with their environment. Table A.2.2.3-1: Service performance requirements for mobile robots | Compared with the mobile robot in 5G system, the service robot in 6G system is able to perform complex tasks with the LLM assistance in service hosting environment. |

6.x.6 Potential New Requirements needed to support the use case

[PR.6.x.6-1]	The 6G system shall be able to provide the communication and AI service for user with the KPI requirements summarized below:

Table 6.x.6-1: Proposed KPIs for use case on service robot

| Use case | Traffic type | Message/Frame size(Byte) | Transfer interval(ms) | Data rate(Mbps) | E2E latency(ms) | Reliability |
| --- | --- | --- | --- | --- | --- | --- |
| Service robot | UL sensor data (NOTE1) | 1250-12500 | 10 | 1-10 | 100-150ms (NOTE4) | 99.99% |
|  | UL LiDAR | 28800 | 100 | 27.6 (NOTE3) |  |  |
|  | DL Control command (NOTE2) | 625-12500 | 50 | 0.1-2 |  |  |
| NOTE1: refers to the kinematic state, environment perception, manipulation status info except LiDAR to be sent from the service robot to the network to enable effective motion planning, object interaction and navigation. NOTE2: refers to the control command towards service robot, e.g. high-level task, action plans, motion strategy, gripper command, etc. NOTE3: the data rate of LiDAR is based on frame rate 10Hz, 28800 points/frame, 12byte for one point cloud, i.e 28800*16*8*10=27.6Mbps. NOTE4: E2E latency includes two parts: the round-trip latency for communication service and the latency for AI inference within the service hosting environment. The typical robot control loops requires 100-150ms latency [6.1z] for AI inference, communication and control. For example, the communication may take about 40ms while the AI inference may take about 100ms [6.1m] for the service robot. |  |  |  |  |  |  |