3GPP TSG-SA WG1 Meeting #111 	S1-253254

Goteborg, Sweden, 25-29 August 2025

Title:	New Use case on Energy-aware Network exposure considering UE preference

Draft TS/TR:	3GPP TR 22.870 v.0.3.1

Agenda item:	8.1.2

Document for:	Approval

Source:	Nokia

Contact:	Laurent-Walter Goix

Abstract: This contribution proposes to introduce a new use case and requirements related to energy-aware Network exposure. Specifically, it introduces some evaluation of the amount of energy/carbon related to a specific API request, so it can be exposed by the provider and requested as criteria by the consumer. In addition, a UE preference is defined to temporarily avoid to impact the UE (e.g. collect data from it) when serving a service request via network exposure, which could drain the device battery.

R1

- Reworded “network API “ to “network exposure”

- Removed “involvment” and “fulfillment” wording

- Clarified description and service flows

- Clarified that the UE preference has no impact on UE beyond network exposure

---------- Use Case template ----------

FIRST CHANGE (new text)

### 5.8.x	Use case on Energy-aware Network exposure considering UE preference

#### 5.8.x.1	Description

5G provides several network services to consumers, like location services, and one of means is through Network exposure.  Several new services such as sensing, high precision positioning, data and AI-based services are envisioned in 6G, which could be exposed by the 6G network to authorized third parties.  These services may be energy intensive or trigger some interaction with the UE, e.g. to collect assistance data.

When Network exposure APIs are invoked by service consumers (e.g. AF, applications residing in the UE or enterprises/industrial applications), data from UE and different network domains (RAN, Core, OAM) may be required in order to serve the request (e.g. a Location API request might be served using one of various methods such as DL-TDOA (which requires data from UE) while UL-AoA might not require data from UE. These have a direct impact on the energy consumption of the network and the UE, especially for repeated requests over long periods of time for e.g. tracking services.

In addition, the network nowadays decides on its own whether to involve the UE or not when serving a request based on the specific API (e.g. location API depending upon the positioning method used), on the consumer’s request parameters or on the producer’s internal logic.  UE involvement may increase UE energy consumption which may not be the user preference when e.g. the UE battery is low, due to privacy concerns, roaming situation or other scenarios.

#### 5.8.x.2	Pre-conditions

John can set a preference to minimize the solicitation of its UE by network exposure services e.g.  based on its battery level or roaming status. This preference has no impact on other services (e.g. communication service) provided by the network.

#### 5.8.x.3	Service Flows

- John, based on its UE energy level e.g. low battery, indicated to the network its UE preference settings for network exposure services.

- John’s father’s family location tracking web portal requests the energy information (e.g. level of consumption, efficiency, CO2e etc) of the network exposure service that it requests

- The network considers the UE preference to select the best method for serving the service request and returns the energy information (e.g. level of consumption, efficiency, etc) of the network and UE for serving this service request. The level of energy consumption or carbon emission caused in the network can be abstracted (e.g. low, medium, high).

- If some energy related criteria are provided in the request from the web portal (e.g. the web portal does not want to use a “high” energy consuming service), the specified energy related criteria are considered by the network when selecting the method for serving the service request.

#### 5.8.x.4	Post-conditions

The  network takes into account John’s UE preference settings for network exposure services as well as energy criteria to serve network services requests related to John’s UE.

#### 5.8.x.5	Existing features partly or fully covering the use cases functionality

Network APIs offer the possibility to program the network and services to meet the needs of the end consumers.  These APIs are being standardized by e.g. 3GPP via NEF/AEF, operational APIs by TMForum [r3] and service APIs by CAMARA [r1].  Such network services may be requested via Intents too.

CAMARA is an open-source project within Linux Foundation to define, develop and test simplified APIs that allow to access network capabilities. CAMARA works in close collaboration with the GSMA Operator Platform Group [r2] to align API requirements and publish API definitions. In addition, billable concepts/events in CAMARA API from GSMA Open Gateway Business Stream are defined.

#### 5.8.x.6	Potential New Requirements needed to support the use case

[PR 5.8.x.6-1] Subject to operator’s policy, user consent and regulatory requirements, the 6G system shall be able to take into account a preference not to impact the UE when exposing network services to an authorized third party.

NOTE 1:	A user can prefer to not impact its UE in network service exposure requests of the network due to e.g., low battery, privacy concerns, roaming status etc. Based on this preference, the network determines how to serve a network service exposure request, which can result degraded or not achievable if data from the UE cannot be collected.

[PR 5.8.x.6-2] Subject to operator’s policy and regulatory requirements, the 6G system shall provide means to provide an authorized third party with  a prediction or estimation of the energy consumed and/or carbon emission equivalent generated by a network service exposure request.

NOTE 2:	The authorized third party can utilize such energy related information to adjust the service characteristics it requests through network exposure.

[PR 5.8.x.6-3] Subject to operator’s policy and regulatory requirements, the 6G system shall provide means to receive energy related preferences or criteria as part of the network service exposure request (e.g. maximum energy or CO2e) and consider these preferences or criteria when serving this request.

SECOND CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

<< skip unchanged part >>

[r1] CAMARA, https://camaraproject.org/

[r2] GSMA Open gateway API, https://www.gsma.com/solutions-and-impact/gsma-open-gateway/gsma-open-gateway-api-descriptions/

[r3] TMForum API directory, https://www.tmforum.org/oda/open-apis/directory

END OF CHANGES