3GPP TSG SA WG 1 Meeting #111 	S1-253255r1

25-29 August 2025, Goteborg, Sweden

Source:	CEWiT

pCR Title:	Pseudo-CR on New use case on Energy Efficiency beyond User Plane

Draft Spec:	3GPP TR 22.870v0.3.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Ankita Lachhwani

ankita.lachhwani@cewit.org.in

Abstract: This PCR proposes inputs to clause 5 of the TR 22.870 “Study on 6G Use Cases and Service Requirements

* * * First Change * * * *

## 5.8	Sustainability and Energy Efficiency

Editor’s Note:  Text to be provided.  This clause to address potential (new) requirements for network/system sustainability and energy efficiency.

### 5.8.X	Use case on Energy Efficiency beyond User Plane

#### 5.8.x.1	Description

The ITU-R Recommendation “Framework and overall objectives of the future development of IMT for 2030 and beyond” [39] positions sustainability as a fundamental design objective across all IMT-2030 scenarios. In support of this, 3GPP TS 22.261 highlights the growing need for energy-aware systems, especially in deployments with limited or unreliable energy supply. This drives the necessity for comprehensive energy management across the entire 6G network not just in the User Equipment (UE) or Radio Access Network (RAN), but critically within the Core Network, where signalling activity is extensive.

While Release 19 introduced initially in the network energy saving features, and later release extended these to cover more of the end-to-end system, energy saving mechanisms continue to focus primarily on the user plane. In the RAN, energy saving strategies have been considered for both the user and signalling plane. However, in the Core Network, energy saving efforts remain minimal and largely confined to the user plane, even though there are other functionalities like control plane signalling that continues to consume significant energy.

Several underlying conditions that contribute towards beyond user plane energy inefficiency:

- Always-on service behaviour: All core services are typically maintained in an active state, regardless of real-time network load or service demand, preventing dynamic energy saving.

- Static policy enforcement: Policy mechanisms do not currently factor in energy awareness, continuing to trigger signalling procedures without accounting for current energy conditions or system load.

- Frequent internal communication: Core network functions often exchange routine or periodic messages, even when no active sessions are present, contributing to idle energy consumption.

- Lack of situational awareness: The network lacks the contextual intelligence to prioritize energy-saving behaviour in sensitive scenarios (e.g., rural sites, constrained-power environments).

- Unified data framework for data intensive service like data collection, data processing, data exposure would lead to exponential increase in energy consumption. This framework is outside the scope of user plane.

- AI services involve model training, model monitoring, re-training, inferences, predictions and analytics which also consumes significant amount of energy

These situations are likely to worsen as 6G introduces next-generation services requiring high interactivity, ultra-low latency, and real-time responsiveness, such as telepresence, distributed AI, and advanced computing services. These applications will significantly increase signalling workloads within the Core Network, which without targeted energy-aware control-plane optimizations may lead to a substantial rise in energy consumption.

.

Hence there is a pressing need to extend the energy efficiency for non-user plane entities like signalling plane.

#### 5.8.x.2	Existing features partly or fully covering the use case functionality

The 3GPP TS 22.261 specifies the potential to deploy systems in areas without a reliable energy source requires new methods of managing energy consumption not only in the UEs but throughout all components of the 5G system.

The energy consumption/efficiency management is restricted to user plane network not the whole 5G system.

#### 5.8.x.3	Potential New Requirements needed to support the use case

[PR 5.8.X.3.-1] Subject to regulation, operator(s) policy the 6G system shall support the energy saving for the beyond user plane functionalities.