3GPP TSG-SA1 Meeting #111	S1-253280r2

25-29 August 2025, Goteborg, Sweden

Source:	China Telecom

pCR Title:	Use case on Predictive resilience enhancement for 6G networks

Draft Spec:	3GPP TR 22.870

Agenda item:	8.1.2

Document for:	Approval

Contact:	Huancheng Yuan, yuanhc2@chinatelecom.cn

Abstract: This use case proposes to consider a 6G capability of predictive resilience enhancement to improve network resilience.

1. Introduction

Please read the use case below.

2. Reason for Change

This is a use case.

3. Conclusions

None.

4. Proposal

It is proposed to study the use case in System and Operation Aspects.

* * *Start of Change (All new text)* * * *

5.6.x	Predictive resilience enhancement for 6G networks

5.6.x.1	Description

In current networks, resilience measures primarily depend on post-fault recovery. Failures such as equipment failures or function degradation are often detected after service disruption has occurred. This approach results in delayed response, prolonged outage duration, and degraded user experience.

This use case considers a shift in 6G network resilience from reactive repair to AI-driven predictive and preventive resilience. An AI system continuously analyses multi-dimensional indicators, as defined in 3GPP TS 22.261 (e.g., end-to-end service latency, communication service availability, service continuity), to assess overall network health in real time, forecast potential risks before faults occur, and recommend or execute proactive interventions to ensure service continuity.

5.x.2	Pre-conditions

The network is equipped with AI models trained on historical data to predict performance trends, and assess the likelihood of service degradation or faults.

The network experiences a slight increase in service processing latency, while service operations remain within acceptable performance thresholds.

5.x.3	Service Flows

1. The AI system continuously monitors and evaluates real-time service performance indicators across network functions. It detects a slight but consistent increase in processing latency on a specific network function, while overall service performance remains within acceptable thresholds.

2.The AI-based system compares the real-time indicators (e.g., end-to-end service latency, communication service availability, service continuity) with historical performance patterns and predictive baselines. Upon identifying deviations that diverge from expected behavior, it initiates anomaly analysis by aggregating indicators from the affected function and its adjacent nodes. Through correlation analysis, the system evaluates whether the anomaly indicates an early-stage fault risk and assesses the potential for further degradation.

3. Based on the evaluated risk level, An AI-based decision engine selects and triggers appropriate mitigation measures, including but not limited to adapt load balancing strategies. If the likelihood of performance degradation escalating into a fault is high, the system may take further actions such as proactively isolating the affected network function or redirecting traffic to redundant nodes.

5.x.4	Post-conditions

Service performance indicators return to normal levels. No actual fault occurs, and services remain continuously available without disruption.

5.x.5	Existing features partly or fully covering the use case functionality

Not applicable.

5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-1] The 6G network should be capable of collecting, analysing, and  predicting service performance indicators (e.g., end-to-end service latency, communication service availability, service continuity) to support early identification of potential service degradation and enable differentiated mitigation strategies.

* * *End of Change * * * *