3GPP TSG SA WG 1 Meeting #111 	S1-253347r1

Goteborg, Sweden, 25 -29 August 2025	(revision of S1-253347)

Source:	ESA, Fraunhofer, Airbus , TNO, Thales, Novamint

Title:	New use case on Sensing of space debris with NTN

Draft Spec:	3GPP TR 22.870 V0.3.1

Agenda item:	8.1.4 Integrated Sensing and Communication (FS_6G_REQ)

Document type:	pCR

Document for:	Discussion/Approval

Contact:	relja.djapic@tno.nl

Abstract: This document proposes to address integrated sensing and communication from space using NTN and introduces a new use case on sensing of space debris by NTN.

1. Introduction

Integrated Sensing and Communication (ISAC) is an important element of 6G as reflected in Chapter 7 of TR 22.870. However, the use cases so far explicitly only refer to the use of terrestrial networks (TN) for ISAC. Authors in ([X1], [X2], [X3]) prove the feasibility of ISAC based on non-terrestrial networks (NTN). In additoin, [X4] and [X5] develop satellite processing/computing payload to support NTN based ISAC.  Accordingly, it becomes apparent that the field of ISAC from space is rapidly emerging  and that non-terrestrial network (NTN) can support integrated sensing application.

This contribution proposes a new use case on on Integrated sensing of space debris by NTN, based on ISAC from space, i.e., based on the usage of NTN domain.

2. Reason for Change

Ensure that the capabilities of Integrated Sensing and Communications from space, i.e. by NTN, will be supported by 6G networks.

3. Conclusions

Add a new use case on Integrated Sensing and Communications from space (NTN) in TR 22.870.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 V0.3.1.

* * * 1st Change * * * *

# 2	References

[X1]	Matera et al, “Terrestrial and Non-Terrestrial Networks for Integrated Sensing and Communication”, International Symposium on Systems Engineering ISSSE 2024, Oct 2024.

[X2]	Y. Gu et al, "ISAC towards 6G Satellite–Terrestrial Communications: Principles, Status, and Prospects", 2024, Electronics 13, no. 7: 1369, https://doi.org/10.3390/electronics13071369.

[X3]	L. You et al, “Ubiquitous Integrated Sensing and Communications for Massive MIMO LEO Satellite Systems”. Integrating sensing and communication for ubiquitous internet of things, IEEE Internet of Things Magazine, Volume: 7, Issue: 4, July 2024

[X4]	M. Corici et al. "Repurposable payloads: concept, design, and initial results," 41st International Communications Satellite Systems Conference (ICSSC 2024), Seattle, USA, 2024, pp. 120-124, doi: 10.1049/icp.2024.4622.

[X5]	ESA Project 5GEOSiS – 5G Earth Observation Server in Space: https://connectivity.esa.int/projects/5geosis-–-5g-earth-observation-server-space

* * * 2nd Change * * * *

## 7.x	Use case on Sensing of space debris with NTN

### 7.x.1	Description

Minimization of the effects created by space debris as well as reduction thereof are critical aspects in achieving sustainability of the space economy and also for deployment and operation of Non-Terrestrial Networks. Integrated Sensing and Communication (ISAC) by the NTN space assets [X1, X2, X3, X4, X5] has the potential to detect and track space debris to ensure the continued safe operation of all space assets.

Monitoring the uplink channels of the NTN satellite on-board for any spurious reflections or disturbances would be a first step. The volume that could be surveilled would be comprised of the combination of the cones of the user link beams down to the lowest commercially used orbits (~ 300 km). Consequently, coverage is limited to the orbits below the NTN satellites. A bistatic passive radar sensing scenario using the UE uplink and also the gateway link signal can be applied for detection and tracking of space debris in various orbits. The illuminator should not be limited for transmit signal from ground. Signals from other satellites can be in principle considered for this use case.

Regarding tracking of the detected object, it is critical to have on-board intelligence for signal processing and detection of target. Depending on detailed measurement scheme, parameters of target movement such as velocity and flight direction should be estimated and shared with a satellite following in the same NGSO orbit for tracking. Therefore, such use case will require high-speed processing and on-board intelligence maybe on AI machine learning.

Estimation of debris position in orbit can be improved by involving multiple satellites in LEO, e.g., through triangulation measurements. Use of high frequency signal source like Ka-band will be beneficial for the detection of small debris (< 5cm).

### 7.x4.2	Pre-conditions

Satellite Network Operator MM provides “smart space debris detection and tracking” service with its 6G NTN component.

Satellite Network Operator MM can access its own NTN space assets and possibly also other satellites for bistatic monitoring.

Earth Observation constellation operator NN subscribes to this smart space debris detection and tracking service from Satellite Network Operator MM.

### 7.x4.3	Service Flows

- Company NN operates a constellation of Earth observation satellites.

- The space debris detection service of satellite network operator (SNO) MM detects a piece of debris in one of the orbits of company NN on a potential collision course with one of the company’s satellites and alerts company NN of the threat.

- Company NN assesses the threat and commands an avoidance maneuver for the affected satellite.

### 7.x4.4	Post-conditions

A collision of the piece of space debris with the EO satellite of company MM and potential follow-on impacts (like a Kessler syndrome for this orbit) has been avoided. The constellation of company MM continues to operate safely.

### 7.x4.5	Existing features partly or fully covering the use case functionality

None.

### 7.x4.6	Potential New Requirements needed to support the use case

[PR 7.X.6-1] Subject to operator’s policy and agreement with 3rd party, the 6G system shall support detection and tracking of space debris at altitudes between 300 and 2000km..

[PR 7.X.6-2] The 6G system shall support sensing services with KPIs as given in following table:

Table 7.x.6-1	Performance requirements sensing of space debris using NTN

| Scenario |  | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency[s] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Sensing of space debris by NTN | Large satellite (4 m x 1 m x 1 m) | Space | [99.9] | [10] NOTE 1 | [10] NOTE 1 | [100] NOTE 1 | [100] NOTE 1 | [10] | [] | [60] | [60], application configurable | [1] | [1] |
|  | Cube sat (30 cm x 10 cm x 10 cm) plus | Space | [95] | [10] NOTE 1 | [10] NOTE 1 | [100] NOTE 1 | [100] NOTE 1 | [10] | [] | [60] | [60] | [5] | [5] |
|  | Space debris (2 cm x 2 cm x 1 cm) NOTE 2 | Space | [90] | [10] | [10] | [100] | [100] | [10] | [] | [60] | [60] | [10] | [5] |
| NOTE 1 : The objective of space debris measurements is a precise determination of the orbit, therefore position and velocity are correlated and accuracies can be traded off against each other. Multiple measurements on the same piece of space debris can significantly enhance precision. NOTE 2: Detecting and tracking space debris that small is challenging, but nevertheless necessary. |  |  |  |  |  |  |  |  |  |  |  |  |  |