3GPP TSG SA WG 1 Meeting #112 Ad Hoc - e	S1-260042

12-16 January 2026, electronic meeting	(revision of S1-26xxxx)

Source:	Telefonica (Moderator)

pCR Title:	Consolidation of KPI requirements on ISAC

Draft Spec:	3GPP TR 22.870 1.0.1 r1

Agenda item:	1.4

Document for:	Approval

Contact:	Jesus Martin, jesusmaria dot martingarcia at telefonica dot com

Abstract: Update of the KPI table of Consolidated performance requirements for ISAC

1. Introduction

Based on the approved pCRs from SA1#112, this contribution provides an update of the KPI table of Consolidated performance requirements for Integrated Sensing and Communication. Another KPI table of Consolidated performance requirements is included for the scenario of Network assisted transportation.

2. Reason for Change

As requested by the SA1 Chairman for KPIs consolidation, moderators need to update their KPI tables with new input from SA1#112

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to approve the following changes to 3GPP TR 22.870.

* * * First Change * * * *

All new text

## 14.2.2	Consolidated performance requirements for Integrated Sensing and Communication

The 6G system shall be able to provide sensing with the following performance requirements:

NOTE: The definitions of the terms used in the following KPI tables are defined in [6].

Table 14.2.2-1: Consolidated performance requirements for Integrated Sensing and Communication.

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing area (sensing target and/or description) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| High topology mapping (NOTE 1) |  | N/A | 0.10 | 0.10 | - |  | [0.4] | - | 50 | ≤ 0.2 | ≤10 | <1 | Outdoor |
| Low-altitude UAV supervision |  | ≥90 | 1-2 | 1-2 | 3-5 | 3-5 | N/A | N/A | 100~1000 | ≤1 | ≤5 | ≤5 | Outdoor (UAV trajectory tracking) |
|  |  | ≥95 | ≤10 | ≤10 | N/A | N/A | [10] | [≥5] | [≤1000] | [≤1] | [≤5] | [≤5] | Outdoor (UAV intrusion detection) |
| Environment object reconstruction |  | 95 | [0.5-5] (NOTE 3) | [0.5-5] (NOTE 3) | N/A | N/A | [0.5] | N/A | [10,000-600,000] | [10-60] | [1-5] | [1-5] | Outdoor (Building (NOTE 2) as in Table 7.6.1-1) |
|  |  |  | 0.5 | 0.5 | 1.5 | N/A | [0.5] | [0.5] | 100 | 0.1 | 5 | 5 | Outdoor (Vehicle (NOTE 2) as in Table 7.6.1-1) |
| Road digitalization |  |  | 0.5 (vehicle) (NOTE 4) 1 (pedestrian) | N/A | 0.5 | N/A | 0.2 | 0.2 m/s | 1~5 | ≤ 0.1 | ≤ 5 | ≤ 5 | Outdoor (crossroad, as in Table 7.7.1-1) (NOTE 5) |
|  |  |  | 3 (NOTE 4) | N/A | 3 | N/A | 1 | 1 m/s | 1~5 | ≤ 0.1 | ≤ 5 | ≤ 5 | Outdoor (highway, as in Table 7.7.1-1) (NOTE 6) |
| Ship detection and tracking (NOTE 7) |  |  | ≤100, (NOTE 8) | NA | NA | NA | ≤100 | NA | 5000 | 1 | 5 | 5 | Outdoor, (Detection of ship on the near shore waters and offshore waters) |
|  |  |  | ≤10 (NOTE 8) | NA | NA | NA | ≤10 | NA | 5000 | 1 | 5 | 5 | Outdoor, (Detection of ship on the harbour entrances, harbour approaches, and port. and coastal waters) |
|  |  |  | ≤2~10 (NOTE 9) | NA | NA | NA | ≤2 | NA | 5000 | 1 | 5 | 5 | Outdoor, (Detection of ship on the inland waterway (e.g. river, lake)) |
| Structural health monitoring |  |  | 0.1, (NOTE 19) | 0.1 | N/A | N/A | N/A | N/A | 5000 | 60 | 5 | 5 | Outdoor (e.g. detection of corner reflectors on bridges, buildings in urban scenario) |
| UAV detection, classification and counting |  |  | 1 (NOTE 20) | 1 (NOTE 20) | 1 (NOTE 20) | 1 (NOTE 20) | [0.3 – 1] (NOTE 20) | 1 (NOTE 20) | ≤1000 (NOTE 20) | ≤1 (NOTE 20) | 2 (NOTE 20) | 2 (NOTE 20) | Outdoor |
| Safety Assistance for vulnerable pedestrian |  | [95] | [1] | N/A | [0.5] | N/A | [0.2] | [0.5] x N/A | ≤ [500] | ≤ [0.1] | ≤ [5] | ≤ [5] | Outdoor (Crossing) |
| Collaborative Robots (NOTE 12) (NOTE 24) |  |  | [≤ 0.1] | [≤ 0.1] |  |  |  |  | N/A |  |  |  | Indoor/outdoor |
| Infrastructure collapse monitoring |  |  | [4] | [N/A] | [N/A] | [N/A] | [4] | [N/A] | TBD | [ 1] | [1~2] | [1~2] | Outdoor (e.g. Detecting sudden collapse on infrastructure such as highway, railway, road, flyover, rural areas, farmland) |
| UAV takeoff and landing (NOTE 24) |  |  | [0.1] (NOTE 13) | [0.1] (NOTE 13) | [1] | [1] | [0.5] | [1] | [100-500] | [0.2] | [5] | [5] | Outdoor (NOTE 14) |
| Gestures Recognition (NOTE 23) (NOTE 24) |  | 99 | 0.1 (NOTE 30) | 0.1 (NOTE 30) | n/a | n/a | [0.3 to 0.5] (NOTE 30) | n/a | ≤20 | ≤0.02 | ≤ 1% (NOTE 22) | ≤ 10% (NOTE 22) | Indoor, Factory Environment, (hand gesture) (NOTE 31) |
|  |  |  | [0.1] | [0.1] | n/a | n/a | [≤0.002] | n/a | ≤20 | [≤0.02] | ≤ 1% (NOTE 22) | ≤ 10% (NOTE 22) | Indoor, Factory Environment, (facial gesture) |
|  |  |  | 0.1 (NOTE 30) | 0.1 (NOTE 30) | [0.3] (note 6) | [0.3] (note 6) | [0.1] (NOTE 30) | [0.3] (NOTE 33) | ≤20 | ≤0.02 | ≤ 1% (NOTE 22) | ≤ 10% (NOTE 22) | Indoor, Factory Environment, (head gesture) (NOTE 32) |
|  |  |  | 0.1 (NOTE 30) | 0.1 (NOTE 30) | n/a | n/a | 0.1 (NOTE 30) | n/a | ≤20 | ≤0.02 | ≤ 1% (NOTE 22) | ≤ 10% (NOTE 22) | Indoor, Factory Environment, (body and limb gesture) |
| Robots collaborating in Sensing. Sensing of Surroundings (NOTE 26) (NOTE 29) |  |  | 0.1 (NOTE 27) | n/a (NOTE 27) | 0.1 | n/a | [0.5 to 1] (NOTE 27) | 1.5 | ≤100 | 0.1 | < 10% (NOTE 25) | < 10% (NOTE 25) | Indoor, Factory Environment |
| Robots collaborating in Sensing. Detection of Pathways, Openings (NOTE 26) (NOTE 29) |  |  | 0.25 (NOTE 27) (NOTE 28) | 0.25 (NOTE 27) (NOTE 28) | 0.1 | n/a | [0.75] (NOTE 27) | 1.5 | ≤100 | 0.1 | < 10% (NOTE 25) | < 10% (NOTE 25) | Indoor, Factory Environment |
| Enhanced XR navigation |  | 99.9 (NOTE 11) | ≤0.5 (NOTE 10) | 0.5 (NOTE 10) | 0.5 | N/A | 0.1- 1 (NOTES 10, 11) | 0.5 (NOTE 10) | ≤100 | 0.1 – 1 (NOTE 11) | 1% | 1% | Indoor, Factory Environment, (10 m2) |
| Safe and economic UAV transport |  | 99.99 | ≤ 50 | 50 (NOTE 15) | 5 (NOTE 15) | 5 | 10 (NOTE 15) | 0.5 (NOTE 15) | ≤ 100 | 0.1 – 1 | 0.1% (NOTES 16, 17) | 1% (NOTES 16) | Outdoor (NOTE 18) |
| NOTE 1: The accuracy KPIs in Scenario “High Topology mapping” are extrapolated from [323] Table C2 as considered for autonomous navigation and remote driving. NOTE 2: A sensing target in Scenario “Environment object reconstruction” refers to a [segment/part] of the target object to be detected and/or tracked, whereas the size of each part is comparable to corresponding spatial resolution of object reconstruction. The percentage of missed detection/false alarm represent missed/falsely reconstruction of parts of the object statistically NOTE 3: Considering a variety of dimensions, shapes, and functionalities of urban buildings, a range of KPI values are needed to measure and provide the flexibility of reconstruction accuracy. NOTE 4: Vehicle, with the assumptions in Table 7.7.1-1. NOTE 5: Sensing target density for vehicles ≤ 750 per 1000 m x 24 m; sensing target density for pedestrians ≤100 per 1000 m x 24 m. Sensing target density is described in Clause 3.1 of the present document. NOTE 6: Sensing target density for vehicles ≤ 72 per 1000 m x 24 m. Sensing target density is described in Clause 3.1 of the present document. NOTE 7: The typical size (Length x Width x Height) of ship is 270 m x 30 m x 30 m in both sensing service category#1 and #2, and 100 m x 15 m x 15 m in sensing service category #3, according to reference [91] NOTE 8: The KPI values for detection of ship on these sensing service categories are referred to [192]. NOTE 9: The KPI values for detection of ship on this sensing service category are referred to [193]. NOTE 10: This positioning accuracy and sensing resolution are required for the detection of proximal objects within a safety area of the user NOTE 11: The KPIs are based on the Hazard Prevention in Industrial Environments for “Prediction of workers’ and machines’ actions” and “Detection of worker’s location” in [94] NOTE 12: Related usecases are captured in clause 7.2 of 3GPP TR 22.837 [9] and clause 6.2 of 3GPP TS 22.137 [6]. NOTE 13: The KPI values for accuracy of positioning are centimetre-level sourced from reference [199], [200] NOTE 14: The KPI values of this use case are effective for sensing in specific area for UAV takeoff and landing. NOTE 15: The listed positioning accuracy and sensing resolution are required for the detection of proximal objects within a safety area of the UAV and validate the reported location of the UAV itself. NOTE 16: Safety impact of missed detection is substantially higher compared to the false alarm, here fore the Missed Detection is assigned a more stringent value compared to the False Alarm. NOTE 17: To enable safe operation of BVLOS UAV flights in mixed airspace (manned & unmanned) and in high risk areas (e.g. above cities) the value for missed detection of 0.1% corresponds to the lower value of the missed detection range specified in Table 6.2-1 of TS 22.137 [6] for sensing service category 2. NOTE 18: Sensing service availability [%] is 99.999 NOTE 19: The typical length of corner reflector is 0.5m [327]. NOTE 20: Most of the requirements presented here are related to the sensing category 3 in [6] NOTE 21: Detection, classification of UAV features such as propellers may require more stringent velocity and range resolution due to the dimensions of propellers with respect to the UAVs. Some propeller diameters to UAV body ratio could be between ½ to 1/3 [330], therefore, the range resolution values are modified accordingly. NOTE 22: Depending on the actual application to be controlled by the gestures, Missed detection and False alarm may have different KPI values from < 1% to 10%. NOTE 23: References for hand gesture recognition are [331][332], for facial and head gesture recognition [333], and for body and limb gesture recognition [334][335]. NOTE 24: The relative distance measurement obtained through 3GPP Sensing is referenced to the surface of the sensed object (e.g., the closest physical boundary of a neighboring vehicle, robot, or some obstacle) from the Reference Point of Sensing (RPS) on the reference entity (e.g., regular UE, vehicle, and robot). NOTE 25: Only valid for non-time-critical applications, < 1% if integrated in time-critical processes. NOTE 26: A maximum sensing range of 10 m is required. NOTE 27: In this table, accuracy of positioning estimate is the horizontal or vertical minimum distance between distinguishable objects, range resolution is the minimum distance between distinguishable objects regarding depth sensing. NOTE 28: Operational practice of autonomous mobile devices (AMRs, AGVs) in industrial settings requires openings on the path to be at least 50 cm wider/taller than the width/height of the devices. The KPI value is half of this additional width/height. NOTE 29: The sensing KPIs are derived from discussions with application experts. Physical realization has been checked. NOTE 30: In this table, accuracy of positioning estimate is the horizontal or vertical minimum distance between distinguishable objects, range resolution is the minimum distance between distinguishable objects regarding depth sensing. NOTE 31: The sensing KPIs for Hand Gestures Recognition are derived from being able to distinguish the position of a hand. NOTE 32: The sensing KPIs for Head Gestures Recognition are derived from different Head Gestures such as shaking, nodding, or turning one’s head. NOTE 33: Detection of intentional movement of one’s head (e.g. intentional shaking or nodding) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 14.2.2-2: Consolidated performance requirements for Integrated Sensing and Communication in Network assisted transportation

|  | Communication KPIs |  |  |  | Spatial KPIs |  |
| --- | --- | --- | --- | --- | --- | --- |
| Scenario | User experienced data rate | End-to-end latency | Communication Service availability | Connection density | Location accuracy | Sensing accuracy |
| Network assisted smart transportation | [1-10 Mb/s] | [40 ms] | 99.99% (NOTE 1) | 104 devices/km2 | [1m] * [1m] * [1m] with 90% probability (NOTE 2) | category 2 or 3 (NOTES 2, 3) |
| NOTE 1: within service volume NOTE 2: within 99% of the service volume NOTE 3: Category 2 or 3 in Table 6.2-1 in TS 22.137 [6] NOTE 4: KPIs for communication, location and sensing in this table are fulfilled simultaneously. |  |  |  |  |  |  |

Table 7.2.2-3: Communication service performance requirements for robots collaborating in sensing in smart factories

| Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CS availability: target value (%) | CS reliability: mean time between failures | End-to-end latency: maximum | Service bit rate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area | Remarks |
| 99.999 | 4 h | < 20 ms | Sensing Result: 1 Mbit/s Sensing Data: 2.5 Gbit/s | 15 kByte 50 kByte | 100 ms | Not con-sidered | 1.5 m/s | ≤ 10 | 100 m x 50 m x 10 m | Sensing Information: Robot third party application |
| 99.999 | 4 h | 5 ms to 10 ms | 10 Mbit/s per video camera stream | 4 MByte | 33 ms | Not con-sidered | 1.5 m/s | ≤ 10 | 100 m x 50 m x 10 m | Video Sensing: (Robot third party application) |
| 99.999 | 4 h | 5 ms to 10 ms | 10 Mbit/s per stream | 1 MByte to 5 MByte | 100 ms | Not con-sidered | 1.5 m/s | ≤ 10 | 100 m x 50 m x 10 m | Application data derived from aggregated sensing results: (third party application Robots) |
| NOTE: References are for communication between robot and third party application [y2] and for video sensing [y1]. |  |  |  |  |  |  |  |  |  |  |

* * * End of changes * * * *