3GPP TSG SA WG 1 Meeting #112 Ad Hoc - e	S1-26xxxx

12-16 January 2026, electronic meeting

Source:	China Mobile

pCR Title:	Pseudo-CR on 6.38.6 to solve EN1 of application round trip performance

Draft Spec:	3GPP TR 22.870 v101

Agenda item:	x.x

Document for:	Approval

Contact:	Xiaonan Shi, shixiaonan@chinamobile.com,

Zixuan Shen, shenzixuan@chinamobile.com

1. Introduction

This document is developed to address the unresolved issue flagged in the Editor’s Note stating: “The value of Application round trip latency are FFS”.

- Reason for Change

- Application round trip latency is not controllable as a requirement for the 6G network since the network cannot manipulate an application’s processing time, unless the AI application is native and under full control of the network, on which we may apply such requirement. In use case 6.38, the AI application is to enhance communication and is originally native-like, but the PR does not reflect the point。

- About the 300-1000ms latency, it is studied that 300ms is not a strictly necessary limit of user experience for tasks like video or audio, thereby a lower threshold could be 600ms for native AI application tasks.

Research on human-computer interaction shows that “Although the acceptable latency for users on a single device is roughly in the range of 100–300 ms, the acceptable latency varies across contexts depending on task type, task complexity, user expectations and so on.”[1]

A study tested 300ms, 600ms, and 900ms latency and found users remain effective at 600ms, while

“the results indicate that the acceptable threshold for end-to-end delay should not exceed 900 ms”[2]

A 2024 systematic review specifically identifies 700ms as the critical point where teleoperation becomes nearly impossible, justifying a distinction between network delay and manageable application lag: “Latencies above 700 ms make it nearly impossible for the teleoperator to interact with the vehicle in a timely manner.”[3]

Therefore, a threshold of 600ms would be satisfy the critical needs in various scenarios.

The 1000ms threshold is OK according to below measurement.

Another paper lists the procedures of real-time emergence service, stating the 1000ms total time needed as:

“Achieving real-time alerts requires careful engineering of both the edge and cloud pipelines. Key latency sources include:

 Signal capture and preprocessing: Typically 10–50ms

 Network transmission: Varies, 50–500ms depending on location and bandwidth

 Inference engine processing: Optimized for sub-100ms response times

 Alert routing and user interface display: 50–150ms”

This study shows that total end-to-end latency can range from 200ms to 1 second, which is acceptable for many safety-critical applications. When even lower latency is required (e.g., autonomous driving), hybrid models may deploy lightweight inference at the edge, reserving deeper analysis for the cloud.

[1]User perceptions of operation latency in cross-device interaction, International Journal of Human-Computer Studies, 2023.

[2]Delay Threshold for Social Interaction in Volumetric eXtended Reality, Carlos Cortes et al., ACM, 2024.

[3]Network Latency in Teleoperation of Connected and Autonomous Vehicles: A Review of Trends, Challenges, and Mitigation Strategies (MDPI Sensors, 2024).

[4]Tyler, Victor. (2025). REAL-TIME HAZARD ALERTS VIA AUDIO STREAMING AND CLOUD-BASED INFERENCE ENGINES.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v101.

* * * First Change * * * *

2	References

[X] Delay Threshold for Social Interaction in Volumetric eXtended Reality, Carlos Cortes et al., ACM, 2024.

* * * Next Change * * * *

6.38	Use case on AI for disability support

6.38.1	Description

As AI evolves, it will further improve human communication, enabling proactive communication with people, devices, and surroundings. This includes hearing aids with complete noise cancellation and the ability to tune in on specific sounds, visual aids enabling recognition beyond normal vision, and communication aids using eye movements or brain interfaces. This can have a revolutionary enabling effect for people with various disabilities and people in challenging situations where typical human capabilities are insufficient, allowing them to recuperate some of the impaired capabilities through the aid of AI-powered tools.

AI enhanced communication can be about person-to-person communication, for example a phone/video call, or person-to-AI function communication, where the AI function/agent assists the person in orientation, understanding, and information processing.

AI enhanced communication can also be applicable to IMS emergency communications when a user with disabilities communicates with a PSAP.

Sustainability impact analysis

Use of AI for disability support is valuable and has benefits within health & human wellbeing area. However, challenges regarding trustworthiness and energy needs to be considered

Energy resources

From the environmental perspective, energy-efficient AI models and increased reliance on renewable energy sources could help minimize the risks. Sources of electricity for AI devices and operations is an important factor as it could impact emissions to air, water and soils.

Trustworthiness

Ethical challenges include determining responsibility - for instance, assessing the impact and entity accountable in the event of technology malfunctions or if developers or providers decide to discontinue support. Determining the appropriate level of control over current and future technology functionality, given the technology’s intimate nature. Ensuring the technology’s value alignment, particularly concerning privacy and integrity as the technology is likely to collect, analyze, and store highly sensitive user data. Other issues in terms of value alignment are mitigating bias, ensuring that the technology works for everyone, and identifying the beneficiaries of the technology (e.g. those with the greatest need versus those with the greatest ability to pay). In evaluating trustworthiness, several aspects must be considered. These include the functionality, reliability, and benevolence (friendliness) of the technology itself, as well as external factors such as the trustworthiness of developers and providers, business models, data management practices and adherence to policy and regulations

6.38.2	Pre-conditions

The following pre-conditions are considered:

- A user has impaired vision, hearing, mobility, or orientation capacity, due to either disabilities or environmental factors that affect bodily functions.

- The user is equipped with a device capable of sending video and audio streams, such as smart glasses with camera and microphone. The device is also capable of receiving audio streams and optionally video streams.

6.38.3	Service Flows

The following service flow is considered, involving an application that can be part of the network (e.g. as an IMS application) or external:

- Immersive video (RGB and depth) and audio stream is captured on user device.

- In an application, the video and audio streams are analysed for user input (cues, intents)

- User gesturing, audio cues, eye tracking, and field-of-view tracking (what the user is looking at).

- In an application, the video stream is further analyzed, and situational awareness is established.

- Based on audio, data input, or visual cues from the user, the application adds contextual information to the visual and/or audio stream or modifies the incoming audio stream. This may include:

  - Scene understanding: answering questions “What am I looking at?”, “Explain what is happening”. Adding digital tags in field-of-view.

  - Orientation: answering questions “Where am I?”, “How do I go to X?”. Adding digital arrows in field of view.

  - Visual aid: text in field-of-view read out in audio.

  - Audio aid: amplification, enhancing contrast, simplification or translation of audio stream.

  - Communication aid: assistance in communication with others, e.g. through translating user intents (gestures, eye movements, field-of-view tracking, etc.) to audio stream.

  - Reminders: assistance in remembering appointments etc., e.g. through audio or digital messages in field-of-view.

- A virtual overlay of contextual and modified information, in the form of video and/or audio streams, is transmitted to user devices.

The above service flow can also be applicable to IMS emergency communications service with a PSAP.

6.38.4	Post-conditions

Users get orientation, scene understanding, and adaptive communication support from the application through their devices and can thereby become more independent and capable.

6.38.5	Existing features partly or fully covering the use case functionality

Some forms of assistance can be implemented as an over-the-top service, on top of the existing mobile broadband connections. However, in the next subsection this contribution analyses the requirements for arrangements where the network involvement provides an enhanced service.

6.38.6	Potential New Requirements needed to support the use case

The following requirement for AI for Disability support are derived from the use case above.

[PR 6.38.6-1] Based on operator policy, local regulation and subscriber permission, the 6G system should enable the IMS services enhanced by AI capability, to enhance the audio and video stream for disability support.

NOTE:	The disability support can be e.g. describing and explaining the surroundings, audio aid (modification of the audio stream i.e. enhancements, clarifications, translations, speech to text, text to speech, etc.), visual aid (modification of the visual stream).

[PR 6.38.6-2] Based on operator policy, local regulation and subscriber permission, the 6G system may be able to enhance the IMS emergency communication service with AI capability, to enhance the audio and video stream for disability support.

[PR 6.38.6-3] The following KPIs in Table 6.38.6-1 should be supported, in Dense Urban, Urban, and Rural indoor/outdoor scenario:

Table 6.38.6-1: Performance requirements

| Profile | Characteristic parameter |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit rate downlink | Bit rate uplink | Application round trip latency | Communication service availability | Overall user density | Activity factor | UE speed |
| AI for Disability support | [1-2] Mbit/s | [50-100] Mbit/s (note 1) | [600-1000] ms for native AI application (note 2) (note 3) | Dense Urban [99.9]% Urban [99.9]% Rural [99]% | Dense Urban [2 500] / km2 Urban [1 000] / km2 Rural [10] / km2 (note 4) | [50]% | 5 km/h |
| Note 1: envisioned 4K resolution Note 2: Input stream (video, audio, data) to output stream (video, audio, data) 600-1000 ms dependent on task including the disability application[X] Note 3: Application round trip latency KPI is only applicable to tasks with native AI capability Note 4: It is assumed that 10% of all users are using AI for Disability support |  |  |  |  |  |  |  |

Editor’s Note The value of Bit rate up link are FFS