3GPP TSG SA WG 1 Meeting #112 Ad hoc-e	S1-260078

12-16 January 2026, electronic meeting	(revision of S1-26xxxx)

Source:	Siemens, EDF

pCR Title:	Pseudo-CR on Resolution of editor’s notes in 11.29.6

Draft Spec:	3GPP TR 22.870 v1.0.1

Agenda item:	1.3

Document for:	Approval

Contact:	Michael Bahr ( bahr et siemens dod com )
Daniel Hauer ( daniel dod hauer et siemens dod com)
Kim Schindhelm ( corina desh kim dod schindhelm et siemens dod com )

Abstract: Resolution of editor’s notes in 11.29.6 related to Use Case on 6G-enabled decentralized smart grid control.

1. Introduction

Time synchronization through timestamps with assured integrity are of paramount importance for the control of smart grids as critical infrastructure.

2. Reason for Change

Editors notes need to be resolved.

3. Conclusions

Integrity protection of timestamps is only needed at designated critical-infrastructure UEs with an authorized third party application at the UE. Potential new requirement is extended with this restriction and editor’s note is removed.

Requirement added that refers to Table 11.29.6-1 with time synchronization service requirements for smart grids.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v1.0.1.

* * * First Change * * * *

### 11.29.6	Potential New Requirements needed to support the use case

[PR 11.29.6-1] Subject to regulatory requirements and operator policy, the 6G system shall be able to provide secure and continuous communication with agreed QoS between a group of designated critical-infrastructure UEs in a limited service area in case of service disruptions in the mobile network.

NOTE 1:	An example for such a group of designated critical-infrastructure UEs are UEs that manage LV grid flexibility—such as load shifting or energy storage utilization—based on real-time electricity demand and LV grid conditions (controllers DS of the LV-cells, smart grid devices (e.g. renewable energy sources, energy storage) in the LV-cells).

NOTE 2:	New additional devices will not join the group of designated critical-infrastructure UEs.

NOTE 3:	Service area is up to 20 km x 30 km.

Time synchronization

[PR 11.29.6-2] The 6G system shall provide suitable means at a designated critical-infrastructure UE to provide TX (sending) timestamps with accuracy of ±4 µs to an authorized third party application at the UE.

NOTE 4: The KPI of this requirement is taken from [414].

[PR 11.29.6-3] The 6G system shall be able to provide integrity protection for TX (sending) timestamps provided at designated critical-infrastructure UEs to an authorized third party application at the UE.

[PR 11.29.6-3a] Subject to regulatory requirements and operator policy, the 6G system shall be able to support time synchronization service performance requirements for smart grids as given in Table 11.29.6-1.

Table 11.29.6-1: Time synchronization service performance requirements for smart grids

| Number of devices | Time sychronization accuracy | Service area | Scenario |
| --- | --- | --- | --- |
| ≤10,000 (note) | [< 1 µs] | ~150 km x ~150 km (note) | intelligent, renewable-powered distribution systems (medium voltage (MV) cells, low voltage (LV) cells) |
| NOTE: Time synchronization is needed per Distribution System Operator (DSO). The service area corresponds to the service area of the DSO and can vary depending on the size of the DSO’s service area and might be based on geographical conditions and regional borders. The provided numbers are for the Austrian Bundesland Lower Austria. |  |  |  |

Service performance requirements

[PR 11.29.6-4] Subject to regulatory requirements and operator policy, the 6G system shall be able to support service performance requirements as given in Table 11.29.6-2.

Table 11.29.6-2: Service performance requirements for smart grids

| Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CS availability: target value (%) | CS reliability: mean time between failures | End-to-end latency: maximum | Service bit rate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area | Remarks |
| 99.999 9 |  | 3 ms | 5.4 Mbit/s | 140 kByte | ≤ 1 ms |  | Stationary | ≤ 200 | 20 km x 30 km | Distributed Energy Resources |
| 99.999 9 | - | < 5 ms | 1.5 Mbit/s | 1,500 Byte | ≥ 1 ms | 5 ms | Stationary | 20 | 20 km x 30 km | Electrical Distribution – Distributed automated switching for isolation and service restoration. |
| 99.999 |  | 100 ms |  | ~100 Byte | 200 ms |  | stationary | 400 | 20 km x 30 km | Distributed Voltage Control |
| > 99.9 |  | < 10 ms | UL: 16 Mbit/s DL: >100 kbit/s | 800 kByte | 10 ms |  |  | >10/km2 | 20 km x 30 km | Distributed energy storage – monitoring |
| > 99.9 |  | DL: <10 ms UL: 1 s | UL: 10.4 Mbit/s DL: >100 kbit/s | UL: 1.3 MByte DL >100 kbyte | 1 s |  |  | >10/km2 | 20 km x 30 km | Distributed energy storage – data collection |
| 99.999 9 |  | 100 ms | < 1 kbit/s per distributed energy resource |  |  |  | Stationary |  | 20 km x 30 km | Ensuring uninterrupted communication service availability during emergencies. |
| NOTE: The service performance requirements are based on TS 22.104 [64] Table 5.2-1. Adaptation to the use case of 6G-enabled Decentralized Smart Grid Control is done for parameters UE speed, # of UEs, and service area. |  |  |  |  |  |  |  |  |  |  |