3GPP TSG SA WG 1 Meeting #112 Ad Hoc - e	S1-260079

12-16 January 2026, electronic meeting	(revision of S1-26xxxx)

Source:	Telefonica, Vodafone, TNO

pCR Title:	Solving EN on subscriber permission in CPRs

Draft Spec:	3GPP TR 22.870 1.0.1

Agenda item:	1.3

Document for:	Approval

Contact:	Jesus Martin, jesusmaria dot martingarcia at telefonica dot com

Abstract: This pCR aims at solving the EN on subscriber permission affecting the CPRs in clause 14 of TR 22.870 v1.0.1

1. Introduction

In SA1#112 (Dallas), there were some moderated discussions about the suitability of using the term “user consent” or different alternatives (“subscriber permission”, “subscription-based permission”, “subscriber preferences” or others). A discusion paper (S1-254338 - DP on user consent) was also provided for the discussion. Finally the CPR table in SA1-254410 was endorsed with several actions , among them:

- To substitute “user consent” with “local regulation and subscriber permission” in every CPR.

- To add Editor’s note: The relation between subscriber permission and previously used terminology is FFS

For the first bullet, 6G Rapporteurs has broght a set of pCRs for approval in SA1#112 Ad Hoc-e (S1-260011, S1-260012, S1-260013, S1-260014, S1-260015, S1-260016)

However, sometimes a simple substitution does not work well and the requirements need to be fine-tuned and adjusted. (eg. CPR#14.1.1-2-12)

There are also some CPRs where there is no need any type of permission or authorization required by the subscriber and the term needs to be deleted from the requirement.(e.g. CPR#14.1.10-1-17)

For the second bullet, this pCR, tries to trigger the necessary discussion to take a decision to use the right terminology for every CPR so that SA1 finally decides to remove the EN.

2. Reason for Change

There is a need to remove the EN in clause 14 (Editor’s Note: The relation between subscriber permission and previously used terminology is FFS.)

The simple substitution of terms may create inconsistencies and requirements need to be double-checked to make sure the new wording reads well and the meaning keeps the same.

Some CPRs where there is no need any type of permission or authorization required by the subscriber, the term needs to be deleted from the requirement.

3. Conclusions

This contribution works on top of the endorsed CPRs (S1-260012, S1-260015, S1-260016) (first, second and third changes),

on top of on top of the endorsed SA1-254410 (fourth change),

and on top of the TR 22.870 v1.0.1 (fifth change).

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 1.0.1

* * * First Change * * * *

Table 14.1.1-2: Enhancements to legacy services and capabilities

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| 14.1.1-2-1 | The 6G network shall provide a mechanism to support event triggered network sharing (e.g. disaster occurrence, network failure, overloaded situation, resource constraints). | PR 5.7.4.2-2 | Network Sharing |
| 14.1.1-2-2 | Subject to regulatory requirements or operator policy, the 6G network shall support sharing of radio access network with sensing capability among operators. | PR-5.7.10.6-1 | Network Sharing [QC/S1-254250]: move to sensing? |
| 14.1.1-2-3 | The 6G system shall support suitable access categories to manage the access attempt for the new services (such as sensing, AI application, computing) with respect to its characteristics (e.g. delay-sensitive, delay-tolerant) in congestion scenarios. | PR 5.7.6.2-2 | Unified Access Control PR modified in SA1 #112 |
| 14.1.1-2-4 | The 6G system shall provide optimized network capabilities for FWA (e.g. support stationary devices) in relevant bands taking into consideration the regulatory requirements for each specific band. | PR 5.7.1.2-1 PR 5.7.1.2-2 | Fixed Wireless Access [ZTE/S1-254096]: merge into 14.1.1-1-3 (Legacy Spt) |
| 14.1.1-2-5 | The 6G system shall enable the means to provide awareness of user service characteristics (e.g. data rate, latency) to support the RAN and CN in making real time resource allocation for FWA. | PR 5.7.1.2-3 | Fixed Wireless Access |
| Alt 14.1.1-2-5 | Based on operator policy, the 6G system shall support means to be aware of user service characteristics (e.g. data rate, latency, predicted changes to each traffic flow component of its service/application to the 6G network) to dynamically adjust and optimse network resources.. | PR 5.7.1.2-3 PR-5.9.8.2-1 PR-5.9.8.2-2 | Service awareness including Fixed Wireless Access [ZTE/S1-254096] |
| NEW # | The 6G system shall provide mechanisms to support efficient bandwidth utilization by the FWA CPE. NOTE: An FWA Customer Premises Equipment (CPE) is used to connect to the network, like any other UE, using a 3GPP access. | PR 5.7.1.2-5 | Fixed Wireless Access EN cleared in SA1 #112 |
| 14.1.1-2-6 | The 6G system shall enhance the Short Message Service to enable a network operator to verify the identity of the SMS sender and information concerning operator verified SMS sender information to the recipient of a SMS. NOTE 1: Operator-verified SMS sender information is used to inform the recipient of a SMS that the identity of the SMS sender is operator-verified and support displaying additional information (e.g. brand name, logo, etc.) of the SMS sender. Human interface aspects are out of scope of this requirement. NOTE 2: Indication that the identity of the SMS sender is operator-verified, any additional information about the SMS sender and the message itself is assumed to be integrity protected. NOTE 3: Based on interworking agreements and trust relationships, the requirements above apply also when the SMS recipient is roaming or receives a SMS from a sender served by other operators. NOTE 4: The requirements above apply to A2P SMS and may apply to Person-to-Person SMS. | PR 5.7.3.2-1 PR 5.7.3.2-2 | SMS |
| Alt 14.1.1-2-6 | The 6G system shall enhance the Short Message Service to enable a network operator to verify the identity of the SMS sender and provide operator verified SMS sender information to the recipient of a SMS. NOTE 1: Operator-verified SMS sender information is used to inform the recipient of a SMS that the identity of the SMS sender is operator-verified and support displaying additional information (e.g. brand name, logo, etc.) of the SMS sender. Human interface aspects are out of scope of this requirement. NOTE 2: Indication that the identity of the SMS sender is operator-verified, any additional information about the SMS sender and the message itself is assumed to be integrity protected. NOTE 3: Based on interworking agreements and trust relationships, the requirements above apply also when the SMS recipient is roaming or receives a SMS from a sender served by other operators. NOTE 4: The requirements above apply to A2P SMS and may apply to Person-to-Person SMS. | PR 5.7.3.2-1 PR 5.7.3.2-2 | SMS [QC/S1-254250] |
| 14.1.1-2-7 | Subject to operator’s policy and agreement with 3rd party, the 6G network shall support a mechanism to start and stop offering certain network service(s) in a local area network adapting to the demand of e.g. the users, 3rd party or the network operator. | PR 5.9.6.6-1 | 6G LAN [ZTE/S1-254096]: proposed to be moved to new table below) [QC/S1-254250]: merge w/other Local NW PRs in Verticals [Huawei/S1-254300]: rename to 6G local area network within a PLMN |
| 14.1.1-2-8 | Subject to operator policies, and agreement between the PLMN operator and authorized 3rd party, the 6G network shall support a mechanism to - authorize PLMN’s users to access a subscribed service provided by an authorized 3rd party via a local area network (deployed by the PLMN operator), and - minimize service interruption when the serving network changes between the local area network and the PLMN network. | PR 5.9.6.6-2 | 6G LAN [ZTE/S1-254096]: proposed to be moved to new table below) [Huawei/S1-254300r1]: rename to 6G local area network within a PLMN |
| 14.1.1-2-9 | Subject to operator’s policy and regulation, for an operator with multiple 6G core networks, when there is UE mobility from one 6G core network to another 6G core network of the same PLMN, the 6G network shall support efficient traffic routing for the traffic from UE to data network. NOTE 1: The above term does not imply any architectural assumption, e.g. whether 6G CN is a new or evolved CN (compared to 5G). NOTE 2: This requirement only impacts core network. | PR 5.9.7.6-1 | Multiple Core Networks |
| 14.1.1-2-10 | Based on operator policy, the 6G network shall support the ability to allow an authorized 3rd party service provider to provide information of the service characteristics for each traffic flow component of its service/application to the 6G network. | PR 5.9.8.2-1 | Service Awareness [ZTE/S1-254096] merged into Service awareness CPR (Alt 14.1.1-2-5) |
| 14.1.1-2-11 | Based on operator policy, the 6G network shall support mechanisms to dynamically adjust and optimize network resources based on the service characteristics, including their predicted changes, provided by the service or application. | PR 5.9.8.2-2 | Service Awareness [ZTE/S1-254096] merged into Service awareness CPR (Alt 14.1.1-2-5) |
| 14.1.1-2-12 | Subject to operator policy and local regulation and subscriber permission , the 6G system shall support means to provide subscribers with differentiation of QoS and charging based on subscribers’ digital identity information issued by a third party and suscriber information. | PR 5.5.9.6-1 | Digital Identity [QC/S1-244250] this PR is also in the Charging section…maybe split the QoS and Charging parts? PR modified in SA1 #112 |
| 14.1.1-2-13 | The multimedia telephony service [144] provided by IMS shall be able to minimise user perception of the transition during codec modification of an ongoing voice call, e.g. a codec change during communication link fluctuation. | PR 5.7.8.2-1 | IMS Codec change QoE |
| 14.1.1-2-13 | The 6G and IMS systems shall provide improved system capabilities for the Multimedia Telephony Service to support an IP-CAN in the 6GS. | PR 5.7.2.2-1 | IMS EN cleared in SA1 #112 |
| 14.1.1-2-14 | Subject to operator policy, the IMS shall support means to minimize impact on the user experience (e.g. call failure) when a UE is engaged in one IMS session where more than one IMS service is triggered. NOTE: Typical example of such situation can be a IMS media related service (e.g. play tone, play announcement) in conjunction with an IMS data channel based service. | PR.5.7.7.6-1 | IMS EN cleared in SA1 #112 |
| 14.1.1-2-15 | Subject to operator’s policy and regulatory requirements, the 6G system shall support a mechanism to enable home operator to authorize UE to access 6G services from home operator’s partner operators when UE only has subscription data of home operator. | PR 5.9.11-1 (was PR 5.5.11.6-1) | Partner PLMNs Agreed in SA1 #112 Moved from 5.5.11 to 5.9.11 |
| 14.1.1-2-16 | Subject to operator’s policy and regulatory requirements, the 6G system shall support a UE to be aware of 6G services provided by home operator’s partner operators. | PR 5.9.11-2 (was PR 5.5.11.6-2) | Partner PLMNs Agreed in SA1 #112 Moved from 5.5.11 to 5.9.11 |
| 14.1.1-2-17 | Subject to operator’s policy and regulatory requirements, the 6G system shall allow home operator to use, monitor, and update the set of 6G services exposed by its partner operators to the home operator. | PR 5.9.11-3 (was PR 5.5.11.6-3) | Partner PLMNs Agreed in SA1 #112 Moved from 5.5.11 to 5.9.11 |
| 14.1.1-2-18 | The 6G system should support potential enhancement of network slicing, e.g.: - Create and delete a network slice in an optimized manner by leveraging automated operations, - Modify (e.g. reconfigure or change resources) a network slice efficiently, and - Improve the mechanism to select, reselect and access network slice(s). | PR 5.7.5.2-1 | Network Slicing EN cleared in SA1 #112 |

* * * Second Change * * * *

Table 14.1.8-3 – 3rd party AI Agent

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 14.1.8-3-1 | Based on operators’ policy and local regulation and subscriber permission, 6G network shall support trusted network access for 3rd party AI Agent and support a mechanism to expose 3rd party AI Agent’s attributes (e.g. related users, sensing capabilities, AI capabilities, service features) to other 3rd party AI Agents. | PR 6.7.6-1 PR 6.23.6-3 | Proposed merged CPR on 3rd party AI Agent, exposure between AI Agents Modified in SA1 #112 |
| - | Based on regulatory requirements, operators’ policy and user consent, 6G network shall support trusted network access for 3rd party AI Agent and support a mechanism to expose 3rd party AI Agent’s attributes (e.g. related users, sensing capabilities, AI capabilities, service features) to other 3rd party AI Agents. | PR 6.7.6-1 | 3rd party AI Agent, access, exposure between AI Agent |
| - | Based on user consent and operator's policy, the 6G network shall support a secure mechanism to expose information of AI application (e.g. AI Agent application) on UE (e.g. related user) to AI application (e.g. AI Agent application) on other UE. | PR 6.23.6-3 | 3rd party AI Agent, exposure between AI Agent |
| CPR 14.1.8-3-2 | Based on local regulation and subscriber permission, operators’ policy and agreement with authorized 3rd party, the 6G network shall be able to support mechanisms to enable secure identification of 3rd party AI Agents associated with a user (e.g. AI Agents belonging to a customer). | PR 6.7.6-2 PR 6.55.6-1 | Proposed merged CPR on 3rd party AI Agent, identification Modified in SA1 #112 |
| - | Based on regulatory requirements, user consent, operators’ policy and agreement with authorized 3rd party, the 6G network shall be able to support security identification for 3rd party AI Agents provided by authorized 3rd party associated with a user (e.g. AI Agents belonging to a customer). | PR 6.7.6-2 | 3rd party AI Agent, identification |
| - | Based on regulatory requirements and operators’ policy, 6G network shall support dynamic identification of 3rd party AI Agents. NOTE: Dynamic means the identification of this AI Agent can be temporary assigned based on task, and this identification can be assigned by different operators based on different tasks. | PR 6.55.6-1 | 3rd party AI Agent, identification |
| CPR 14.1.8-3-3 | Based on regulatory requirements, operators’ policy and user preference, 6G network shall support mechanisms for authorized 3rd party AI Agents to provide their attributes to 6G network, and discover other authorized 3rd party AI Agents. NOTE: Attributes can include e.g. capabilities, associated authorized users | PR 6.7.6-3 PR 6.46.6-1 PR 6.62.6-1 | Proposed merged CPR on 3rd party AI Agent, register and discover |
| - | Based on regulatory requirements, operators’ policy and user consent, 6G network shall support mechanisms for 3rd party AI Agents to provide/register their attributes (e.g. sensing capabilities, AI capabilities, service features, associated authorized users) to 6G network, and discover other authorized 3rd party AI Agents to achieve collaborative task. | PR 6.7.6-3 | 3rd party AI Agent, register and discover, collaboration |
| - | Based on the user consent and operator’s policy, the 6G network shall be able to support the discovery of 3rd party AI Agent (application) on the UE. | PR 6.46.6-1 | 3rd party AI Agent, discover |
| - | Based on regulatory requirements, operator's policy and agreement with 3rd party, the 6G network shall support a mechanism for an authorized 3rd party to provide the information about the AI Agent application (e.g. authorized users suing the 3rd party AI Agent application, capabilities related to the 3rd party AI Agent application) to the 6G network. | PR 6.62.6-1 | 3rd party AI Agent, provide information of AI Agent to network NEW: Agreed in SA1 #112 |
| CPR 14.1.8-3-4 | Based on operator's policy and user preference, the 6G network shall support a mechanism to manage the attributes (e.g. sensing capabilities, AI capabilities, service features, associated authorized users) of AI application (e.g. AI Agent application) on UE (e.g. related user). | PR 6.23.6-1 | 3rd party AI Agent, management |
| CPR 14.1.8-3-5 | Based on local regulation and subscriber permission and operator's policy, the 6G system shall support secure interoperability and efficient mechanism between AI applications (e.g. AI Agents) on multiple UEs. NOTE 1: Interoperability between AI Agents refers to the ability to discover, authenticate and authorize AI Agents to communicate, exchange data, and work together seamlessly. NOTE 2: Collaborative task refers to an activity, action requiring the involvement of two or more AI Agents. | PR 6.9.6-2 PR 6.23.6-4 PR 6.30.6-2 PR 6.40.6-1 | Proposed merged CPR on 3rd party AI Agent, collaboration |
| - | The 6G system shall support secure interoperability between AI Agents and between AI Agents and applications to achieve a collaborative task. NOTE 1: Interoperability between AI Agents refers to the ability to discover, authenticate and authorize AI Agents to communicate, exchange data, and work together seamlessly. NOTE 2: Collaborative task refers to an activity, action requiring the involvement of two or more AI Agents. | PR 6.9.6-2 | 3rd party AI Agent, collaboration |
| - | Based on user consent and operator's policy, the 6G network shall support a secure mechanism to provide communication service between AI applications (e.g., AI Agent applications) on multiple UEs for a collaborative task. | PR 6.23.6-4 | 3rd party AI Agent, collaboration |
| - | Subject to operator’s policy, the 6G network shall be able to provide AI service to enable collaborative task for AI applications running on multiple UEs. | PR 6.30.6-2 | 3rd party AI Agent, collaboration |
| - | Subject to user consent and operator’s policy, the 6G network shall provide efficient mechanisms to support the collaboration of UEs. | PR 6.40.6-1 | 3rd party AI Agent, collaboration |
| CPR 14.1.8-3-6 | The 6G network shall provide means to support coordination within and across groups of 3rd party AI Agents to achieve a collaborative task associated with a 3GPP service. NOTE 2: Coordination can facilitate the discovery of attributes & capabilities across the 3rd party AI Agents, the attribution of roles such as a task coordinator/supervisor of the group, the decision to setup another group for a sub task etc. | PR 6.7.6-6 | 3rd party AI Agent, collaboration NEW: Agreed in SA1 #112 |
| CPR 14.1.8-3-7 | Based on regulatory requirements and operators’ policy and user preference, the 6G network shall provide means to support efficient and secure communication (including multi-modality exchange) between multiple e.g. AI applications/3rd party AI Agents on UEs over a target area considering data characteristics. NOTE: This requirement can apply to 3rd party AI Agents of same users or different users. It is expected that the required communication service would be provisioned in the range of minutes to days, depending on use case. Lower for temporary task and higher for long term task. | PR 6.7.6-4 PR 6.8.6-3 PR 6.23.6-4 | Proposed merged CPR on 3rd party AI Agent, communication |
| CPR 14.1.8-3-8 | Based on operators’ policy and local regulation and subscriber permission, the 6G system shall support hosting of large amount of 3rd party AI Agents managed and controlled by the 6G network and/or multiple AI Agent applications on a UE. | PR 6.9.6-1 | 3rd party AI Agent, general |
| - | The 6G system shall support hosting large amounts of AI applications (e.g. AI Agent applications) managed and controlled by the 6G network and/or multiple AI Agent applications on a UE. | PR 6.9.6-1 | PR 6.9.6-1 was modified during SA1 #112 |
| CPR 14.1.8-3-9 | Based on operator policy, the 6G network shall be able to support secure means to expose 3GPP services (e.g. 6G computing service in Service Hosting Environment) to the authorised third-party AI Agent based on its intent. | PR 6.6.6-1 PR 6.7.6-5 | Proposed merged CPR on 3rd party AI Agent, service exposure |
| - | Based on operator policy, the 6G network shall be able to support secure means to expose its services to the authorised third-party AI Agent based on its intent. | PR 6.6.6-1 | 3rd party AI Agent, service exposure |
| - | Based on operator policy, the 6G network shall be able to support secure means to expose different services, e.g. computing offloading service in Service Hosting Environment, to the authorized third-party AI Agent. | PR 6.7.6-5 | 3rd party AI Agent, service exposure |
| CPR 14.1.8-3-10 | Based on local regulation and subscriber permission and operator policy, the 6G system shall provide a suitable means for an AI Agent application on UE to invoke some 3GPP services (e.g. IMS service, AI service) upon request. | PR 6.8.6-1 PR 6.14.6-2 PR 6.30.6-1 | Proposed merged CPR on 3rd party AI Agent, service invoke Modified in SA1 #112 |
| - | Based on user consent and operator policy, the 6G system shall provide a suitable means for an AI Agent application on UE to invoke some 3GPP services (e.g. IMS service). | PR 6.8.6-1 | 3rd party AI Agent, service invoke |
| - | Subject to operator’s policy, the 6G network shall support mechanisms for a 3rd party AI-based application on UE (e.g. UAV) to invoke an AI service upon request. | PR 6.14.6-2 | 3rd party AI Agent, service invoke |
| - | Subject to operator’s policy and user consent, the 6G network shall be able to support mechanism for AI application on UE to invoke AI services provided by 6G network. | PR 6.30.6-1 | 3rd party AI Agent, service invoke |
| CPR 14.1.8-3-11 | The 6G network shall be able to provide a suitable means to allocate network resources (e.g., network slice) to a group of trusted third parties (e.g., applications running on multiple UEs/robots or third party AI Agents), considering dynamic changes of traffic demand and QoS characteristics. | PR 6.52.6-2 | 3rd party AI Agent, resource |
| CPR 14.1.8-3-12 | Based on the operator's policy and local regulation and subscriber permission, the 6G network shall support a mechanism to authenticate and authorize 3rd party AI Agent on the UE. | PR 6.23.6-2 PR 6.41.6-1 PR 6.41.6-2 | Proposed merged CPR on 3rd party AI Agent, authentication and authorization |
| - | The 6G network shall support a mechanism to authorize AI application (e.g. AI Agent application) on UE to invoke 3GPP services. | PR 6.23.6-2 | 3rd party AI Agent, service invoke, authorization |
| - | The 6G network shall support a mechanism to authenticate and authorize 3rd party AI Agent. | PR 6.41.6-1 | 3rd party AI Agent, authentication and authorization |
| - | Based on the operator's policy, the 6G network shall support a secure mechanism for authenticated and authorized 3rd party AI Agent to invoke 3GPP services. | PR 6.41.6-2 | 3rd party AI Agent, service invoke, authentication and authorization |
| CPR 14.1.8-3-13 | Based on local regulation and subscriber permission, operator policy and regulatory requirements, the 6G system shall provide an efficient way to expose information (e.g. change of QoS) to authorized 3rd party AI Agents (e.g. by means suitable for prompt augmentation). | PR 6.8.6-2 PR 6.13.6-1 PR 6.9.6-3 | Proposed merged CPR on 3rd party AI Agent, exposure to AI Agent |
| - | Based on user consent, operator policy and regulatory requirements, the 6G system shall provide an efficient way to expose information (e.g. change of QoS) to the application on the UE. | PR 6.8.6-2 | 3rd party AI Agent, exposure to AI Agent |
|  | Subject to operator policies, the 6G network shall be able to provide authorized AI applications (e.g. AI Agents applications) in the UE and in the 6G network) with communication service performance information (e.g. throughput, latency) relevant for their operation, including to achieve a collaborative task.Agent | PR 6.9.6-3 | 3rd party AI Agent, exposure to AI Agent PR 6.9.6-3 was Modified in SA1 #112 |
| - | Subject to operator’s policy, the 6G network shall be able to expose information from the network to authorized 3rd party AI Agents by means suitable for prompt augmentation. NOTE: Prompt augmentation is an approach of adding further information, or instructions, to a prompt in order to enhance the AI-generated response, e.g. in terms of quality or relevance. | PR 6.13.6-1 | 3rd party AI Agent, exposure to AI Agent |
| CPR 14.1.8-3-14 | Based on the local regulation and subscriber permission and operator’s policy, the 6G system shall be able to support means for the network to invoke the 3rd party AI Agent (application) on the UE. | PR 6.46.6-2 | 3rd party AI Agent, network invoke AI Agent |
| CPR 14.1.8-3-15 | Subject to operator’s policy, regulatory requirements and local regulation and subscriber permission, the 6G system shall support a mechanism for a subscriber-authorized AI application (e.g. AI Agent) in the Service Hosting Environment to autonomously initiate communication with emergency services on behalf of the subscriber, i.e., the communication session is associated with the subscriber's identity and location. | PR 6.47.6-1 | 3rd party AI Agent, emergency service |
| CPR 14.1.8-3-16 | Subject to operator’s policy, regulatory requirements and local regulation and subscriber permission , the 6G network shall support a mechanism to log the data (e.g., sensor data, context information) used by an AI application (e.g., AI Agent) in the Service Hosting Environment, which autonomously decided to initiate communication with emergency services on behalf of the subscriber. | PR 6.47.6-2 | 3rd party AI Agent, emergency service |
| CPR 14.1.8-3-17 | Subject to operator’s policy, regulatory requirements and local regulation and subscriber configuration, the 6G system shall support a mechanism for a subscriber to provide policies which define the autonomous actions that can be taken by their authorized AI applications (e.g. AI Agents) in the Service Hosting Environment. NOTE: For example, a subscriber could provide a policy authorizing their AI Agent to share location data with a pre-defined family member. A separate policy could authorize their AI Agent to initiate communication with emergency services on the suscriber's behalf when the Agent identifies a critical safety event | PR 6.47.6-3 | 3rd party AI Agent, emergency service |

* * * Third Change * * * *

Table 14.1.10-1: ISAC

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| 14.1.10-1-1 | Subject to regulatory requirements, operator’s policy, the 6G network should support suitable means to collect non-3GPP sensing data from third party if available. | PR 7.16.6-1 | [ZTE] Non-3GPP sensing data collection Considering the sensing data collection is specific, it is suggested to remain it in 14.1.10 |
| 14.1.10-1-2 | Subject to operator policies, the 6G Network shall provide sensing service to detect, classify and count one or more sensing targets (e.g. detect UAVs, distinguish UAVs from birds, identify specific UAV characteristics.). NOTE: Classification refers to the identification of specific characteristics of the detected target objects and grouping together of the detected target objects with similar characteristics. | PR 7.23.6-1 | Sensing modes Classification of objects |
| 14.1.10-1-3 | Subject to operator policies, the 6G Network shall provide mechanisms for configuring a sensing operation with a single or multiple sensing modes from all sensing modes supported (e.g. bistatic, monostatic, multistatic). | PR 7.14.6-1 | Sensing modes configuration |
| 14.1.10-1-4 | Subject to regulation and operator’s policy, the 6G system should provide mechanisms to ensure sensing service is able to be provided with a given sensing system capacity or/and a latency upper-bound to nearby UEs (e.g. AMRs), requested by the trusted third party. NOTE: The term 'sensing system capacity' is the maximum number of targets that can be detected per unit area given sensing QoS requirements per target, which include localization accuracy and sensing service latency [11]. NOTE 2: The latency depends on different types of applications in various verticals, such as factory, mining and on how fast the AMR is moving in the zone of interest. | PR 7.5.6-3 PR 7.7.6-1 | Sensing Service Capacity Multiple targets [ZTE]: Sensing QoS: capacity/density/latency upper-bound |
|  | Subject to operator policy, local regulation and subscriber permission, the 6G system shall mechanisms to protect data privacy during the processing of sensing data. | PR 7.12.6-2 | PR modified in SA1 #112 |
| 14.1.10-1-5 | Subject to regulation and operator's policy, 6G system shall support sensing target density requested by the third party for a given sensing service. | PR 7.7.6-1 | Target Density |
| 14.1.10-1-6 | Based on operator policy, regional and/or national regulations, the 6G network in the area of the disaster shall provide secure mechanisms to collect sensing results with a requested level of accuracy that can be used to generate real time maps. | PR 7.2.6-1 | Accuracy Target Density Latency 3rd Party Support |
| 14.1.10-1-7 | Subject to operator’s policy and regulation, the 6G network shall be able to provide a sensing service to derive predicted location and/or velocity of sensing target(s). | PR 7.10.6-1 | Prediction |
| 14.1.10-1-8 | Subject to regulation and operator policy, the 6G network shall provide a target prediction capability to derive predicted target characteristics (e.g. size, shape, location, velocity), while maintaining the privacy of the sensing target(s) and means to expose the prediction of location and/or velocity of sensing target(s) to a trusted third-party. | PR 7.10.6-1 PR 7.10.6-2 | Prediction Exposure Privacy Third Party support |
| 14.1.10-1-9 | Subject to regulatory requirements, operator’s policy, local regulation and subscriber permission, 6G network shall support the use of stored sensing data to provide a sensing service and ensure that only authorised entities are able to access the stored sensing data and results. | PR 7.11.6-1 PR 7.12.6-1 | Sensing Data Storage Usage & Security PR modified in SA1 #112 |
| 14.1.10-1-10 | The 6G system shall be able to prioritize communication, sensing and positioning together used in Network Assisted Smart Transportation. | PR 7.19.6-2 | Prioritization |
| 14.1.10-1-11 | Subject to operator’s policy, the 6G system shall provide exposure mechanism(s) to activate and deactivate exposing sensing results to a UE (AMR) that are used for prediction in a given sensing area of interest at a particular time of interest to nearby UEs at the request of a trusted third party. | PR 7.8.6-1 | Exposure Third party support Delivery synchronization |
| 14.1.10-1-12 | The 6G Network shall provide suitable mechanisms for the exposure of sensing results in a synchronised manner with other types of traffic (e.g. audio, video, haptics) to the sensing service consumer. | PR 7.14.6-2 | Sensing result exposure sync.with other traffic, |
| 14.1.10-1-13 | Subject to operator’s policy, local regulation and subscriber permission, the 6G network shall be able to authorise UE and expose sensing results to an application on the UE for a specific service. NOTE: As an example, UE could use the provided sensing results (e.g. environment characteristics around UE) to optimize communication service. | PR 7.20.6-1 PR 7.21.6-1 PR 7.5.6-4 | Exposure to UE PR modified in SA1 #112 |
| 14.1.10-1-14 | Subject to operator’s policy, local regulation and subscriber permission, the 6G network shall be able to provide sensing results to a UE for a specific service, where the UE is authorized by mobile network operator providing sensing service. | PR 7.5.6-4 | PR modified in SA1 #112 |
| 14.1.10-1-15 | Subject to operator’s policy and regulation, the 6G system shall be able to link sensing results with communication service area for communication service. | PR 7.20.6-2 | ISAC Linking |
| 14.1.10-1-16 | Subject to regulatory requirements and user permission, the 6G network shall be able to use the 6G sensing service to monitor and recognize human gestures. | PR 7.24.6-1 | Gesture Recognition |
| 14.1.10-1-17 | Subject to operator’s policy, and local regulation, the 6G network shall be able to provide secure means to expose the prediction of location and/or velocity of sensing target(s) to a trusted third-party, while maintaining the privacy of the sensing target(s). | PR 7.10.6-1 PR 7.10.6-2 | Sensing Data Security and Privacy Protection |
| 14.1.10-1-18 | The 6G system shall be able to support energy-efficient sensing operations. | PR 7.5.6-2 (To be consolidated with PR 7.8.6-2 | Energy-efficient Sensing Operations [ZTE} PR 7.5.6-2 move to EE part 14.1.4 |
| 14.1.10-1-19 | Subject to operators’ policies, regulations, when offering sensing service, if the assistance information (e.g. the actual sensing target characteristics) from a trusted 3rd party is available, the 6G network shall provide means for a mobile network operator to monitor and validate the sensing result (e.g. by comparing the sensing results with the actual sensing target characteristics etc.). | PR.7.26.6-1 | Assistance Info from trusted 3rd party NEW: Agreed in SA1 #112 |
| 14.1.10-1-20 | Subject to network operator policy, the 6G network shall be able to provide secure means to an authorized 3rd party for providing dynamic re-configuration requests of communication services in order to ensure continuous reliable connectivity to mobile UEs with the required Quality of Service as provided in Table 7.27.6-2 in a changing environment. | PR 7.27.6-1 | Dynamic reconfiguration requests for reliability NEW: Agreed in SA1 #112 PR-2 and -3 are KPIs |

* * * Fourth Change * * * *

## Y.1.11	Ubiquitous Connectivity

Editor’s Note: This clause contains consolidated potential requirements for 6G that include (but not limited to):s upport for non-terrestrial networks (NTN) and support for positioning services and capabilities. This includes (but not limited to) PRs from clause 8.

Editor’s Note: PR 8.2.6-1 and PR 8.17.6-1 have Editor’s Notes associated with them, so they are not included for consolidation into Table Y.1.11-1 at this time.

Editor’s Note: PRs that only state a requirement to meet KPIs in a “Table” will be revised to point to the appropriate consolidated tables in Y.2.

Editor’s Note: additional consideration is needed to determine which CPRs are moved from Table Y.1.11-1 (Communications) to Table Y.1.11-3 (Others). Some requirements may currently be shown in both tables, or in different potential consolidation proposal.

Table Y.1.11-1 –Satellite-based communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.11-1-1 | The 6G system with satellite access shall be able to support flexible configurations (e.g. parameters for satellite access) for both sparse satellite constellation and dense satellite constellation and optimize the coverage per LEO satellite for sparse satellite constellation. | PR 8.3.6-1 PR 8.3.6-3 | [Satellite access] LEO Coverage Flex configs |
| Y.1.11-1-2 | The 6G system with satellite access shall support service continuity when UE is within the coverage of the LEO constellation, e.g. in the sparse LEO scenarios. | PR 8.3.6-2 | [Satellite access] Service continuity LEO |
| Y.1.11-1-3 | Subject to regulatory requirements, the 6G system with satellite access, shall support text messaging services and PWS for UEs in adverse propagation conditions e.g. light indoor conditions, dense forest. | PR 8.4.6-3 PR 8.3.6-3 | [Satellite access] PWS Text Messaging |
| Y.1.11-1-4 | The 6G system shall support communication between UEs under the coverage of base stations onboard HAPS platforms, without the user traffic going through the ground network. | PR 8.13.6-1 | Other non-terrestrial access] UE-HAPS-UE direct communications |
| Y.1.11-1-5 | The 6G system with satellite backhaul links between a 6G base station on board of UAV and the 6G CN, shall support the selection and switching between satellite links, each having different characteristics, based on e.g. traffic load, quality of the link, satellite availability. NOTE: The following terms 6G base station/CN do not imply any architectural assumption, e.g. whether 6G CN/base station is a new or evolved CN/base station (compared to 5G). | PR 8.12.6-1 | Satellite backhaul links selection/switching Should be moved out of this table (about SAT access), e.g. can go in “others” |
| Y.1.11-1-6 | The 6G system with satellite access shall be able to provide time synchronization to UEs and applications using 3GPP technologies, independently of non-3GPP technologies (e.g. GNSS). | PR 8.14.6-1 | Time synchronization reference |
| Y.1.11-1-7 | Subject to operator’s policy, the 6G network with satellite access shall support onboard Service Hosting Environment(s) and mechanisms to m Minimise the necessary bandwidth of the inter-satellite and feeder links, and Modify the path for routing data traffic between a UE and the Service Hosting Environment to minimize service interruption considering the movement of UE and/or satellite, provide a computing service via a to UEs (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities, and subject to subscriber permission, support data sharing among multiple UEs (e.g. UAV). NOTE: The shared data is the processed results based on non-3GPP sensing data from the UEs (e.g. UAV) provided by Service Hosting Environment on aboard satellite. | PR 8.15.6-1 PR 8.9.6-1 | Onboard Service Hosting Environment Modification of data routing Computing service UE Data Sharing Non-3GPP sensing It is suggested to seperate which are targeting different aspects. |
| Y.1.11-1-8 | Subject to operator’s policy, local regulation and subscriber permission, the 6G system with satellite access shall be able to support data sharing among multiple UEs (e.g. UAV). NOTE: The shared data is the processed results based on non-3GPP sensing data from the UEs (e.g. UAV) , provided by Service Hosting Environment on aboard satellite. | PR 8.9.6-3 |  |
| Y.1.11-1-9 | Subject to operator’s policy, the 6G network with satellite access shall be able to support modifying the path for routing data traffic between a UE and Service Hosting Environment on board satellites to minimize service interruption considering the movement of UE and/or satellite. | PR 8.9.6-4 |  |
| Y.1.11-1-10 | The 6G system using satellite access shall support mechanisms to improve communication QoS and capacity under constrained satellite link conditions, | PR 8.18.6-1 | QoS Improvement |

Table Y.1.11-2 – Satellite-based positioning

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.11-2-1 | The 6G system with satellite access shall be able to provide a mechanism for energy constrained UEs to determine their position using 3GPP positioning technology. | PR 8.7.6-1 | Energy constrained devices |
| Y.1.11-2-2 | The 6G system with satellite access shall be able to provide a positioning service supported by the combination of 3GPP satellite and GNSS positioning technologies, including Network-based Assisted GNSS. | PR 8.11.6-1 | Support of NTN & GNSS positioning technologies |
| Y.1.11-2-3 | The 6G system with satellite access shall be able to provide different positioning service to different types of UE (e.g. energy constrained device, UAV, airplane) using satellite access, supported by single satellite based positioning method or hybrid 3GPP positioning methods. | PR 8.5.6-1 PR 8.7.6-1 PR 8.10.6-1 | Positioning Service based on satellite-based positioning technology |
| Y.1.11-2-4 | Subject to regulatory requirements, operator policy, and subscriber permission, the 6G system with satellite access shall be able to determine UE location based on 3GPP positioning technologies for UE only using satellite access. | PR 8.5.6-2 | Determine UE location using satellite-based positioning |

Table Y.1.11-3 – Others

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.11-3-1 | The 6G system with satellite access shall be able to provide time synchronization to UEs and applications using 3GPP technologies, independently of non-3GPP technologies (e.g. GNSS). | PR 8.14.6-1 | Time synchronization |
| Y.1.11-3-2 | The 6G system using with satellite access based on regenerative satellites shall be able to support the transfer of computing information (e.g. pre-processed data) of a computing task between S within the Service Hosting Environments on board ) between satellites over a given area. | PR 8.15.6-2 | Satellite-Satellite Data transfer between onboardof computing inforesources |
| Y.1.11-3-3 | Subject to operator’s policy, local regulation and subscriber permission, the 6G system with satellite access shall be able to share the data (e.g. the processed result based on the non-3GPP sensing data from multiple UEs) provided by the Service Hosting Environment on board satellite among multiple UEs (e.g. UAV). | PR 8.9.6-3 | Computing data sharing |
|  | Subject to operator’s policy, the 6G network with satellite access shall be able to support modifying the path for routing data traffic between a UE and Service Hosting Environment on board satellites to minimize service interruption considering the movement of UE and/or satellite. |  | Routing modification |
|  | Subject to operator’s policy and agreement with 3rd party, the 6G network with satellite access shall be able to provide a computing service via a suitable support onboard Service Hosting Environment on board (s) satellite to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities, or |  | Onboard Computing Marked in yellow because of the controversial definition of SHE |
|  | Subject to operator’s policy, the 6G network with satellite access may support onboard Service Hosting Environment(s) and mechanisms to - Minimise the necessary bandwidth of the inter-satellite and feeder links, - Modify the path for routing data traffic between a UE and the Service Hosting Environment to minimize service interruption considering the movement of UE and/or satellite, - provide a computing service via a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities, and - subject to subscriber permission, support data sharing among multiple UEs (e.g. UAV). NOTE: The shared data is the processed results based on non-3GPP sensing data from the UEs (e.g. UAV) provided by Service Hosting Environment on aboard satellite. This implies some need to transfer computing information in/related to Service Hosting Environment (e.g. pre-processed data within the Service Hosting Environment) between satellites over a given area. | PR 8.15.6-1 PR 8.9.6-3 PR 8.9.6-4 PR 8.15.6-2 | Onboard Service Hosting Environment Modification of data routing Computing service UE Data Sharing Non-3GPP sensing and transfer of computing service related info related to SHE |
|  | Subject to operator’s policy and agreement with 3rd party, the 6G system with satellite access may be able to provide to logistic companies computing service in a given area via a suitable Service Hosting Environment on board satellite to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities. | PR 8.9.6-1 |  |
|  | Subject to operator’s policy and agreement with 3rd party, the 6G system with satellite access shall be able to provide a computing service via a suitable Service Hosting Environment on board satellite to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities | [PR 8.9.6-1] |  |

## Y.1.12	Immersive Communication

Editor’s Note: This includes (but not limited to) PRs from clause 9.

Editor’s Note: PR 9.5.6-4, PR 9.8.6-1, PR 9.8.6-2, and PR 9.18.6-1 have Editor’s Notes associated with them, so they will not be consolidated into Table(s) Y.1.12-x at this time.

Editor’s Note: PRs that only state a requirement to meet KPIs in a “Table” will be revised to point to the appropriate consolidated tables in Y.2.

Table Y.1.12-1 – Immersive Communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.12-1-1 |  |  |  |
| Y.1.12-1-2 |  |  |  |
| Y.1.12-1-3 |  |  |  |
| Y.1.12-1-4 |  |  |  |
| Y.1.12-1-5 |  |  |  |
| Y.1.12-1-6 |  |  |  |
| Y.1.12-1-7 |  |  |  |
| Y.1.12-1-8 |  |  |  |
| Y.1.12-1-9 |  |  |  |
| Y.1.12-1-10 |  |  |  |
| Y.1.12-1-11 |  |  |  |
| Y.1.12-1-12 |  |  |  |
| Y.1.12-1-13 |  |  |  |
| Y.1.12-1-14 |  |  |  |
| Y.1.12-1-15 |  |  |  |

Table Y.1.12-1 – Immersive Communication related to IMS

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.12-1-1 |  |  |  |
| Y.1.12-1-2 |  |  |  |
| Y.1.12-1-3 |  |  |  |
| Y.1.12-1-4 |  |  |  |

## Y.1.13	Massive Communication

Editor’s Note: This includes (but not limited to) PRs from clause 10.

Table Y.1.13-1 – Massive Communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

## Y.1.14	Industry and Verticals

Editor’s Note: This includes (but not limited to) PRs from clause 11.

Editor’s note: The local network related PRs are also presented in “Resilience”, “Enhancements to legacy services and capabilities” groups. It is FFS whether to set up a new group for localized network.

Editor’s note: It is FFS how to consolidate the computing related PRs.

Table Y.1.14-1 – UAV, UAM and aircraft

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.14-1-1 | 6G system with satellite access shall provide a means to obtain and expose communication service interruption information (e.g. expected interruptions to the communication service) to authorized third party. | PR 11.5.6-2 | QC:[should go under Satellite CPRs] exposure |
| CPR Y.1.14-1-2 | Subject to regulatory requirements and subscriber permission, the 6G network shall provide secure means to expose to an authorized 3rd party application the information related to a UAV or a UAM aircraft from the stored service data. | PR 11.7.6-2 | Exposure |
| Alternative CPR Y.1.14-1-2 | Subject to regulatory requirements and subscriber permission, the 6G network shall provide secure means to store the service data for a UAV or a UAM aircraft based on the request information (e.g. service type, storage duration, time expiry) and expose the information to an authorized 3rd party application. . NOTE: The service data may be the processing data for a specific network service such as sensing data, positioning measurement data, or the exposed data such as sensing results, positioning information. | PR 11.7.6-2 PR 11.7.6-1 | Store and Exposure |
| CPR Y.1.14-1-3 | Subject to regulatory requirements and subscriber permission, the 6G network shall be able to securely store the service data for a UAV or a UAM aircraft based on the request information (e.g. service type, storage duration, time expiry). NOTE: The service data may be the processing data for a specific network service such as sensing data, positioning measurement data, or the exposed data such as sensing results, positioning information. | PR 11.7.6-1 | Data storage |

* * * Fifth Change * * * *

# 14	Consolidated potential functional and performance requirements

Editor’s Note: The allocation/consolidation of computing CPRs is FFS.

* * * End of changes* * * *