3GPP TSG-SA WG1 Meeting #112	S1-254112

17-21 November 2025, Dallas, USA	(revision of S1-25xxxx)

Source:	vivo

pCR Title:	Update on UC#6.50

Draft Spec:	3GPP TR 22.870 v0.4.1

Agenda item:	8.1.3	Artificial Intelligence

Document for:	Approval

Contact:	Hank YU, yuhang.txyjy@vivo.com

Yanchao Kang, kangyanchao@vivo.com

Abstract: This paper proposes a new dimensionality to evaluate the service performance of the computing service: ‘computing accuracy’, which is related to the quality of the computing. If the latency is the only target index of the computing process, then how to compare and evaluate the quality of two computation tasks within the same latency? A new performance requirement is needed to evaluate the computing quality apart from computing speed.

* * * First Change * * * *

6.50.3	Service Flows

Figure 6.50.3-1: Real-Time Video Super-Resolution Service provided by 6G Network

The service flow for the real time video super-resolution service offered by 6G network is shown in Figure 6.50.3-1.

1.	The third-party video service providers or 6G operator can pre-configure specific AI model/algorithm selection rules and related AI models/algorithms tailored for different types of content, such as scenery, sports, or games. The same AI model/algorithm applied to different types of video content can produce varying effects. Besides, the 6G network can pre-train AI models for real time video super-resolution and store them within the network.

2.	Alice is streaming video outdoors via a 6G network, but due to limitations with her user equipment and network uplink bandwidth, the video is captured in standard or suboptimal resolution (e.g. 540p).

3.	Alice's UE transmits the standard or suboptimal resolution live streams to the 6G network.

4.	When the video stream is received, the 6G network considers the user preferences provided by Bob, indicating a higher resolution than the current one, and therefore, the 6G network decides to apply video super-resolution. 6G network coordinates the AI nodes, computing nodes, and communication nodes to offer real time video super-resolution services for this video stream.

The 6G network can determine network-based or mobile-based for the real time video super-resolution process, and further select an appropriate AI model/algorithm considering following aspects:

a) Network aspect: considering different network downlink communication performance and network computing load, either network-based video super-resolution or mobile-based video super-resolution can be performed. For example, when communication links performance between the 6G network and Bob is optimal, the 6G network can handle real time video super-resolution and deliver high-definition content directly. If communication links performance between the 6G network and Bob is suboptimal, low-resolution videos will be received from the network and maybe enhanced on the UE/client side utilizing the local computing resources;

b) Video processing aspect: a suitable AI model/algorithm should be selected for each specific video type, as the same AI model/algorithm has different effects on video content (e.g. scenery, sports, or games) regarding the colour, block size, and details of the video;

c) User aspect: for different user preferences (e.g. Full HD, Ultra HD) on video resolution, 6G network can accordingly choose different AI models or traditional algorithms to satisfy the user experience.

5.	The 6G network selects and applies an appropriate AI model/algorithm for the real time video super-resolution service.

6.	The 6G network selects the most suitable computing resources considering factors like computing load, node location and other potential information. Utilizing available computing resources, the network applies the AI model/algorithm to enhance the video graphics, in order to guarantee the computing accuracy requirements received from users.

NOTE:	Computing accuracy is a general dimensionality to evaluate the quality of the computing processing apart from the computing latency. In case of computing for video super-resolution service, computing accuracy can be interpreted as some performance requirements of video graphics, e.g., video resolution, SSIM index (Structural SIMilarity index), PSNR (Peak Signal-to-Noise Ratio).

7.	The enhanced high-resolution live streams, such as 1080p, are transmitted to viewers, including Bob.

8	The video is decoded and presented to the user, Bob.

9	6G network monitors the performance of real time video super-resolution and periodically sends reports of real time video super-resolution outcomes to the manager. Such reports may encompass various metrics, including AI model inference accuracy, measured by calculating pixel differences between the upsampled video and the original high-definition video. They may also include Motion Estimation, which assesses whether the model effectively restores details in fast-moving scenes, as well as QoE metrics. Such reports can be used by an operator or 3rd party video service provider to further update or retrain AI models, or update rules or algorithms in order to enhance the user experience for video services.

* * * Next Change * * * *

6.50.6	Potential New Requirements needed to support the use case

[PR 6.50.6-1] Subject to operator policy, the 6G network shall be able to manage and coordinate various network operations (e.g. AI model training/selection, computing resource selection, communication performance monitoring) upon receiving a request (e.g. combined 3GPP service that combines services such as 6G AI service and communication service) with the requested service requirement.

[PR 6.50.6-2] Subject to operator policy, the 6G network shall be able to support a mechanism to fulfil the performance requirements (e.g. latency, computing accuracy, AI model inference accuracy) when providing combined 3GPP service (e.g. combines 6G AI service and communication service).

NOTE:	Computing accuracy is one metric to describe the quality of the computing process apart from the computing latency. An example of computing accuracy for video super-resolution: video resolution.

[PR 6.50.6-3] Subject to operator policy, the 6G network shall be able to monitor the performance (e.g. AI model inference accuracy) and report them to the 3rd party.

* * * End of Changes * * * *