3GPP TSG SA WG 1 Meeting #112 	S1-254121

17-21 November 2025, Dallas, Texas, USA	(revision of S1-25xxxx)

Source:	CATT

pCR Title:	Pseudo-CR on CPRs of Ubiquitous Connectivity(Clause Y.1.11)

Draft Spec:	3GPP TR 22.870 v0.4.1

Agenda item:	8.1.10

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: The contribution proposes the consolidated functional requirements for ubiquitous connectivity (Clause Y.1.11)

1. Introduction

Based on the discussion in S1-254120, this contribution introduces the consolidated functional requirements for ubiquitous connectivity.

2. Reason for Change

Introduce consolidated functional potential requirements of ubiquitous connectivity to Clause Y.1.11

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.4.1.

* * * First Change * * * *

Y	Consolidated potential functional and performance requirements

Y.1	Consolidated potential Functional Requirements

Y.1.11	Ubiquitous Connectivity

Editor’s Note: This clause contains consolidated potential requirements for 6G that include (but not limited to): support for non-terrestrial networks (NTN) and support for positioning services and capabilities. This includes (but not limited to) PRs from clause 8.

Table Y.1.11-1 – NTN Communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.11-1-1 | The 6G system with satellite access shall be able to support flexible configurations (e.g. parameters for satellite access) for both sparse satellite constellation and dense satellite constellation and optimize the coverage per LEO satellite for sparse satellite constellation. | PR 8.3.6-1 PR 8.3.6-3 | [Satellite access ] LEO Coverage Flex configs |
| Y.1.11-1-2 | The 6G system with satellite access shall support service continuity when UE is within the optimized coverage of the LEO constellation, e.g. in the sparse LEO scenarios | PR 8.3.6-2 | [Satellite access ] Service continuity LEO |
| Y.1.11-1-3 | The 6G system with satellite access shall support service continuity (with minimum service interruption) for UEs moves between different non-terrestrial access networks (e.g. satellite, HAPS), or between terrestrial access network and non-terrestrial access network. | PR 8.4.6-1 PR 8.8.6-1 | [Satellite access ] Service continuity TN-NTN |
| Y.1.11-1-4 | Subject to regulatory requirements, the 6G system with satellite access shall support text messaging services and PWS for UEs in adverse propagation conditions e.g. light indoor conditions, dense forest. | PR 8.4.6-3 PR 8.3.6-3 | [Satellite access ] PWS Text Messaging |
| Y.1.11-1-5 | The 6G system with satellite access shall be able to provide broadband communication service to UEs mounted on Size, Weight and Power constrained moving platforms (e.g. flying drone, small vessels mounted). | PR 8.6.6-1 | [Satellite access ] eMBB |
| Y.1.11-1-6 | The 6G system with satellite access shall support mechanisms to improve communication QoS and capacity under constrained satellite link conditions, e.g. using signalling optimization, codec rate adaptation. | PR 8.18.6-1 | [Satellite access] QoS Improvement |
| Y.1.11-1-7 | The 6G system with satellite access shall provide means to obtain and expose service interruption information (e.g. estimated interruption time) to authorized third party to minimize interruption for immersive media services during UE moves between terrestrial access and satellite access network. | PR 11.5.6-2 | [Satellite access] Information exposure for Service continuity |
| Y.1.11-1-8 | The 6G system shall support communication between UEs under the coverage of base stations onboard HAPS platforms, without the user traffic going through the ground network. | PR 8.13.6-1 | [Other non-terrestrial access] UE-HAPS-UE communications |
| Y.1.11-1-9 | The 6G system with satellite backhaul links between a 6G base station on board of UAV and the 6G CN, shall support the selection and switching between satellite links, each having different characteristics, based on e.g. traffic load, quality of the link, satellite availability. NOTE: The following terms 6G base station/CN do not imply any architectural assumption, e.g. whether 6G CN/base station is a new or evolved CN/base station (compared to 5G). | PR 8.12.6-1 | [Satellite backhaul] links selection/switching |
| Y.1.11-1-10 | Subject to operator’s policy, the 6G network shall support mechanisms to connect the base stations (onboard satellite) and the ground core networks using 3GPP technology. | PR 8.17.6-1 | [Satellite backhaul] New connection method |

Table Y.1.11-2 – Positioning for UE with satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.11-2-1 | The 6G system with satellite access shall be able to provide the positioning service in complaint with regulatory requirements to UEs under the coverage of sparse or dense satellite constellations. | PR 8.3.6-4 | [General] Regulatory Support |
| Y.1.11-2-2 | The 6G system with satellite access shall be able to determine the integrity of a positioning service and provide the assistance information related integrity to a UE. | PR 8.16.6-1 | [General] Positioning integrity |
| Y.1.11-2-3 | The 6G system with satellite access shall be able to provide different positioning services to different types of UE (e.g. energy constrained device, UAV, airplane) using satellite access, supported by single satellite-based positioning method or hybrid 3GPP positioning methods. NOTE: hybrid 3GPP positioning methods include the combination of 3GPP satellite based positioning technology and various legacy RAT dependent positioning methods such as U- TDOA, E-OTD, etc. | PR 8.5.6-1 PR 8.7.6-1 PR 8.10.6-1 | Positioning Service based on satellite-based positioning technology |
| Y.1.11-2-4 | The 6G system with satellite access shall be able to provide a positioning service to a UE using satellite access, supported by the combination of 3GPP satellite-based positioning method and GNSS-based positioning technologies (e.g. pure GNSS, Network-based Assisted GNSS). | PR 8.11.6-1 | Support of NTN & GNSS positioning technologies |
| Y.1.11-2-5 | Subject to regulatory requirements, operator policy, and user’s consent, the 6G system with satellite access shall be able to determine UE location based on 3GPP positioning technologies for UE only using satellite access. | PR 8.5.6-2 | Determine UE location using satellite-based positioning |

Table Y.1.11-3 – Other NTN capabilities

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.11-3-1 | The 6G system with satellite access shall be able to provide time synchronization to UEs and applications using 3GPP technologies, independently of non-3GPP technologies (e.g. GNSS). | PR 8.14.6-1 | Time synchronization |
| Y.1.11-3-2 | Subject to operator’s policy and agreement with 3rd party, the 6G system with satellite access shall be able to provide a computing service via a suitable Service Hosting Environment on board satellite to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities, or minimizing the necessary bandwidth of the inter satellite links and feeder links. . | PR 8.9.6-1 PR 8.15.6-1 | Onboard computing |
| Y.1.11-3-3 | Subject to operator’s policy and user’s consent, the 6G system with satellite access shall be able to share the data (e.g. the processed result based on the non-3GPP sensing data from multiple UEs) provided by the Service Hosting Environment on board satellite among multiple UEs (e.g. UAV). | PR 8.9.6-3 | Computing data sharing |
| Y.1.11-3-4 | Subject to operator’s policy, the 6G network with satellite access shall be able to support modifying the path for routing data traffic between a UE and Service Hosting Environment on board satellites to minimize service interruption considering the movement of UE and/or satellite. | PR 8.9.6-4 | Routing modification |
| Y.1.11-3-5 | The 6G system with satellite access shall be able to support the transfer of computing information (e.g. pre-processed data) of a computing task between Service Hosting Environments on board satellite over a given area. | PR 8.15.6-2 | Data transfer between onboard computing resources |

* * * End Change * * * *