3GPP TSG SA WG 1 Meeting #112 	S1-254232

Dallas, USA, November 17 – 21, 2025	(revision of S1-253231)

Source:	Huawei, China Mobile, CATT

pCR Title:	Update of clause 5.9.2 use case on efficient data collection and control for 6G system

Draft Spec:	3GPP TR 22.870 v0.4.0

Agenda item:	8.1.2.2

Document for:	Approval

Contact:	Fangyuan ZHU, zhufangyuan@huawei.com

Abstract: This contribution proposes to update the existing clause 5.9.2 use case on efficient data collection and control for 6G system.

1. Introduction

This contribution proposes to update the existing clause 5.9.2 use case on efficient data collection and control for 6G system.

2. Reason for Change

- Addressing the following Editor’s Note about user consent:

Editor’s Note: user consent in this section is FFS.

- Adding new requirements about data quality, which was submitted in S1-253231 for SA1#111.

Data quality is a key component for data collection and consumption, which is critical for the success of business model and specific use cases that consume precise data such as autonomous vehicles and industrial automation. High-quality data, characterized by accuracy, completeness, timeliness, and consistency, enables reliable analysis and real-time decision-making.

The data quality assurance is the foundation for SLAs of data service, which requires end-to-end coordination across the entire data lifecycle. For example, the metrics for evaluating the quality of data are listed in Table1. The 6G system needs to monitor, and enforce metrics of data quality on data itself and data handling (e.g., data processing, data transmission), such as accuracy, and timeliness.

Table 1: Data Quality Metrics[1]

| Data Quality Metrics | Definition |
| --- | --- |
| Completeness | refers to the degree to which all required or expected data values are present and available for a given purpose. It quantifies the absence of missing information, ensuring that a dataset contains all the necessary attributes and records to adequately represent the phenomena it describes or to fulfill the requirements of a specific task (e.g., model training, validation, or inference). In the context of a feature set for an AI/ML model, completeness ensures that all relevant features for each observation are populated. For a time-series dataset, it pertains to the presence of data points at expected intervals |
| Reliability | refers to the degree to which data can be trusted to be consistent, stable, and free from unexpected variations over time and across different sources. It measures the dependability of the data generation and collection processes, ensuring that the data produced is accurate, reproducible, and fit for its intended use under stated conditions. In the context of AI/ML systems, reliable data ensures that models are trained and evaluated on a consistent foundation, reducing the risk of performance degradation due to data drift or source inconsistencies |
| Accuracy | refers to the degree to which data matches the true, real-world values or states it is intended to represent. It quantifies the absence of errors and deviations, ensuring that data values are correct and faithful to the actual entities or events they model. |
| Timeliness | refers to the degree to which data is available for use within the required timeframe or by a specified deadline. It measures the delay between the occurrence of a real-world event and the point when the corresponding data becomes accessible and usable in the system. Timely data is critical for operational efficiency, real-time decision-making, and maintaining the relevance of information. Data that is accurate and complete but delivered too late may lose its value and utility, potentially leading to missed opportunities or ineffective responses |
| Availability | refers to the degree to which data and the data systems are accessible for users and applications when needed. It is a critical operational metric that focuses on the accessibility, discoverability, and recoverability of the data. High availability means the data can be reliably retrieved and used on demand, minimizing downtime or delays caused by system failures, maintenance, or complex access protocols. It is often measured in the context of system uptime and data freshness, especially in real-time or mission-critical applications |

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.4.1.

4. Reference

[1]	https://portal.etsi.org/webapp/WorkProgram/Report_WorkItem.asp?WKI_ID=74875

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[r1]	https://portal.etsi.org/webapp/WorkProgram/Report_WorkItem.asp?WKI_ID=74875

* * * Second Change * * * *

### 5.9.2	Efficient data collection and consumption for 6G system

#### 5.9.2.1	Description

To better serve the users and manage the network, as well as provide non-connectivity service (e.g. LCS), 3GPP system need to conduct collection of data associated with the network and services provided by the 3GPP system. The data collection in the 5GA network for network/service management and non-connectivity service is defined for specific use cases, resulting in varying solutions across different use cases, different layers and different domains. Here is a brief summary of different data collection work done in RAN/SA2/SA4 and corresponding security mechanism in SA3:

-	In RAN domain, data collection from UE is based on MDT (Minimization of drive testing) method as defined in TS 37.320 [237] and TS 38.331 [238]. The user consent of UE location data collection for MDT is defined in TS 32.422 [239].

-	For data collection for AI/ML in core network, the data collection feature permits NWDAF to retrieve data from various sources (e.g. NF such as AMF, SMF, PCF, NSACF, GMLC, and AF, OAM, etc.), as a basis of the computation of network analytics as defined in TS 23.288 [114]. The operator can leverage these network analytics to optimize the management of network operations. For instance, as outlined in clause 6.22 of TS 23.288 [114], the operator can collect network data (such as OAM data and CP signalling) to detect or predict a signalling storm and use the analytics to mitigate or prevent its impact. The data collection is performed via SBI between CN NFs, DCCF (Data Collection Coordination Function) is introduced as logical functionality to coordinate data collection in CN, and ADRF (Analytics Data Repository Function) is defined to store the collected data. The user consent of UE data collection for AI/ML is defined in TS 33.501 [250].

-	For UE positioning, the input data is collected from UE/RAN to CN via CP/UP path as the basis of the computation of UE location as defined in TS 23.273 [240]. The privacy check of UE location collection is also defined in TS 23.273 [240].

-	To collect data and to expose data between CN domain and AF domain, the network exposure mechanism in core network has been defined in clause 5.20, TS 23.501 [140] and clause 4.15, TS 23.502 [30], where the NEF is used as the termination node to isolate both sides. Especially for edge, exposure of UE data is defined in TS 33.558 [241].

-	If the data collection is triggered by OTT server in application layer from UE, the UP path may be used with the assistance of DCAF, as defined in TS 26.531 [242].

The aforementioned mechanisms are defined for different network functions to support data collection in different scenarios, leading to high standardization overhead. There is lack of coordination between these network functions, leading to isolated data collection and duplicated data collection. Meanwhile, the data transmission based on the control plane are not suitable for transmitting large amounts of data, leading to inefficient data transmission. Therefore, the data collection and control mechanism need to be improved in 6G.

Table 5.9.2.1-1 lists several potential use cases for data collection in 6G, and illustrates the corresponding data type, data provider, data consumer and data volumes for each use case. Take AI/ML data as example, there is a large amount of data types defined in 3GPP apart from those non-standardized ones, and the scale of data volume is usually large (from 10k to 100M).

Table 5.9.2.1-1: Heterogeneous 6G System Data

| Use case category | Data types | Data provider | Data Consumer | Data volume |
| --- | --- | --- | --- | --- |
| Sensing data collection/sensing result exposure (e.g. measurement) | Sensing measurement data requires further study and discussion. The potential sensing measurement can be channel estimation results, delay, Doppler, angle, signal strength, etc. | UE, RAN | Sensing Function | Related to use cases: e.g. data rate of measurement is about 10Mbps for objects creating hazards on roads |
| AI/ML data collection / analytics exposure (e.g. AI/ML for air interface, AI/ML for core network) | - For RAN data collection for Air interface, see TR 38.843 [243] - For core network data collection, see the data collection framework in clause 6.2, TS 23.288 [114], and the detailed data type are defined from clause 6.3 to clause 6.21, TS 23.288 [114]. | CN, RAN, UE, AF | CN, RAN, UE | Typically, the internal data collection for AI model training yields tens of thousands of samples. Related to AI model: e.g. the data volume is about 10k bytes to 100M bytes for AI model of beam management |
| Energy Efficiency (EE information exposure to AF) | still under the discussion in SA2 R19 EnergySys WID, see SP-250069. | CN, OAM | EIF (Energy Information Function) | Related to use cases: e.g. could be similar to data volume for charging |
| Positioning, | See clause 6, TS 23.273 [240] | UE, RAN | LMF, UE | Related to use cases |
| Network Exposure in core network | See clause 5. 20, TS 23.501 [140], and clause 4.15, TS 23.502 [30] | 5GC NF, AF | 5GC NF, AF | Related to use cases |
| … |  | … | … | … |

All the above use cases for data collection and control share the following commonality:

- The data collection and control mechanisms need to support procedures such as data collection, data storage, data exposure and data deletion.

- The collected data could be requested or consumed by multiple data consumers within 3GPP system to support multiple services; for example, a single spatiotemporal environmental dataset can be concurrently utilized by multiple services, including IoT-enabled pervasive sensing service, latency-sensitive extended reality (XR) applications requiring <10ms motion-to-photon delays, and intelligent transportation systems (ITS) with centimetre-level geolocation precision.

- The upper bond of the collected data volume is very high, e.g. on the order of 100 M byte;

- Latency requirements for data collection and transmission exhibit variations across diverse use cases.

- The collected data transmission path between different entities within 3GPP system can vary widely, encompassing scenarios such as UE to RAN node，UE to CN node, RAN node to CN node, and CN node to CN node.

- All these use cases that involve collecting UE related data need to support security, privacy protection and user consent in data lifecycles of collecting, transmitting, processing and storing. In addition, security requirements needs to consider different data types.

- All these use cases that involve data exposure need to be subject to operator policy and user consent.

Figure 5.9.2.1-1: Use case specific data mechanisms in 5G  common data mechanism in 6G

As shown in Figure 5.9.2.1-1, use case specific mechanisms are defined for each service scenario in 5G, leading to system complexity and inefficient data collection. As 6G networks integrate advanced capabilities progressively, such as AI, enhanced computing power, multi-dimensional sensing and digital twin network, they will need to process massive amounts of multi-source, heterogeneous data (from diverse formats and data resource) generated by 6G system. Consequently, 6G system needs to support efficient data collection and control mechanism capable of addressing both universal requirements and scenario-specific demands across heterogeneous use cases such as AI/ML, sensing, XR service and digital twin network. The efficient data collection and control mechanism needs to support various data providers and consumers, which maintains service-agnostic characteristics while embedding extensibility for emerging service.

In addition, in order to fulfil the service performance requested by data consumer, quality assurance shall be considered in terms of both data content[r1] and data handling. By considering quality assurance on both data content and data handling, the E2E service performance will be guaranteed by 6G system. For example:

-	Quality assurance in terms of data itself [r1]: e.g., data freshness, data accuracy (e.g., distance resolution 1 m), data reliability (e.g. error ratio 0.001%), and data volume (e.g. 500M byte).

-	Quality assurance in terms of data processing: e.g., data processing latency (e.g., 200ms).

-	Quality assurance in terms of data transmission: e.g., transmission latency (e.g., 100ms).

-	Quality assurance in terms of data retrieval: e.g., latency of data retrieval from massive amount of data stored in distributed data storage.

In conclusion, the 6G system needs to support efficient data collection and control mechanism, which includes the following characteristics:

-	Common data framework that can serve the need of multiple use cases, to improve data collection efficiency and avoid duplication data collection

-	Support data collection from various data providers and data exposure to various data consumers;

-	Support data quality assurance;

-	Support user consent for data collection and privacy protection; and

-	Support data exposure subject to operator policy and user consent.

#### 5.9.2.2	Potential New Requirements

[PR 5.9.2.2-1] The 6G system shall support mechanisms for 6G System Data collection and consumption minimizing the impact to 6G services.

[PR 5.9.2.2-2] Subject to operator’s policy, regulation and user consent, the 6G system shall support processing of 6G System Data.

NOTE:	Examples of data processing are use case dependant, e.g. data fusion, data anonymization and data analysis.

[PR 5.9.2.2-3] Subject to user consent, regulation and operator's policy, the 6G system shall support secure means to expose 6G System Data to authorized trusted third-party, authorized network function or authorized UE.

[PR 5.9.2.2-4] Subject to user consent, the 6G system shall support security and privacy protection of the 6G System Data lifecycle including collection, transmission, processing and storage.

[PR 5.9.2.2-5] The 6G system shall be able to provide charging and accounting mechanisms for 6G System Data.

[PR 5.9.2.2-6] Subject to regulation, user consent and operator policy, the 6G system shall support storage and retrieval of 6G System Data.

[PR 5.9.2.2-7] Subject to regulation and operator policy, the 6G system shall support transfer 6G System Data between different data providers and data consumers within the 6G system.

[PR 5.9.2.2-8] Subject to operator’s policy, the 6G system shall support mechanisms to support quality assurance for 6G System Data and the associated end-to-end handling (e.g., data processing, transmission and retrieval) based on request from data consumer.

* * * End of Change * * * *