3GPP TSG-SA WG1 Meeting #112 	S1-254xxx

17-21 November 2025, Dallas, USA	(revision of S1-254015)

Title:	Rapporteur proposed initial consolidated PRs (Clause Y.1.1)

Agenda Item:	TBD

Source:	6G Study Rapporteurs

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: Initial allocation of PRs into Clause Y.1.1 tables.

# Y	Consolidated potential functional and performance requirements

Editor’s Note:  PRs with associated Editor’s Note(s) will not be considered for consolidation until Editor’s Note(s) associated with the PR are resolved.

Editor’s Note: CPR Table(s) can be modified (e.g. split, merged, renamed, moved) during the consolidation process.

## Y.1	Consolidated potential Functional Requirements

### Y.1.1	Basic Services and Capabilities

Editor’s Note: This clause contains consolidated potential requirements for 6G that include (but not limited to):	Support for Legacy (i.e., 5G and earlier) services and capabilities, Enhancements to Legacy services and capabilities, Legacy services and capabilities not supported in 6G, and device support. This includes (but not limited to) PRs from clauses 5.2, 5.3, 5.4, 5.7, 5.9, and 5.10.

Table Y.1.1-1 – Support for legacy services and capabilities

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.1-1-1 | The 6G system shall support new 6G services and capabilities and existing 5G services and system requirements, unless explicitly identified. This includes requirements in [14] clause 5.1.2.2, related to inter-RAT capabilities not to be supported by 5GS, apply similarly to 6GS with the following modification: - voice service continuity from the radio access network of the 6G system to UTRAN CS shall not be supported. | Clause 5.1 Clause 5.2 PR 5.5.10.2-1 PR 5.7.4.2-1 PR 5.7.6.2-1 PR 5.9.1.3-1 | The following four PRs “should be” implicitly included in Y.1.1-1 and not needed to be identified separately. |
|  | The 6G system shall have a means to ensure that the visited network provides services to an inbound roamer only once the subscriber identity (e.g. SUPI, IMSI) is obtained from the home network. | PR 5.5.10.2-1 | Roaming |
|  | The 6G system shall support network sharing, based on 5GS network sharing requirements. | PR 5.7.4.2-1 | Network Sharing |
|  | The 6G system shall support Unified Access Control requirements as specified in [14]. | PR 5.7.6.2-1 | Unified Access Control |
|  | Subject to operators' policy, the 6G system shall provide a mechanism for the support of Femtocell deployment. | PR 5.9.1.3-1 | Femtocells |
| Y.1.1-1-2 | The 6G system shall be able to support the services listed and those identified as not needing to support (i.e., CS Fallback, CS based voice call) in Clause 5.4.2. | Clause 5.4.2 |  |
| Y.1.1-1-3 | The 6G system shall be able to support 5G system requirements (functional and performance requirements) defined by the specifications listed in Clause 5.4.3 for 3GPP and non-3GPP accesses. These requirements do not apply to requirements involving the legacy systems/RATs (e.g. E-UTRAN, UTRAN, GERAN). | Clause 5.4.3 Clause 5.3 |  |
| Y.1.1-1-4 | Subject to operator’s policy, the 6G system shall support mobility procedures - between the core network (CN) of the 6G system and a 5G core network, and - between the core network of the 6G System and the evolved Packet Core Network (EPC), - between the 6G 3GPP access and non-3GPP access with minimum impact to the user experience (e.g. Quality of Service (QoS), Quality of Experience (QoE)). NOTE 1: Complexity on introducing the above interworking requirement needs to be minimized. | Clause 5.2 Clause 5.3 |  |
| Y.1.1-1-5 | The 6G system shall be able to provide 3GPP services to a UE using various 3GPP and non-3GPP access networks (e.g. satellite, WLAN or Wireline). | Clause 5.3 Clause 5.4.1 |  |
| Y.1.1-1-6 | The 6G system shall support multiple access technologies to be used simultaneously for one or more services active on the UE. | Clause 5.3 |  |

Table Y.1.1-2 – Enhancements to legacy services and capabilities

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.1-2-1 | The 6G network shall provide a mechanism to support event triggered network sharing (e.g. disaster occurrence, network failure, overloaded situation, resource constraints). | PR 5.7.4.2-2 | Network Sharing |
| Y.1.1-2-2 | Subject to regulatory requirements or operator policy, the 6G network shall support sharing of radio access network with sensing capability among operators. | PR-5.7.10.6-1 | Network Sharing |
| Y.1.1-2-3 | The 6G system shall support suitable access categories to manage the access attempt for the new services (such as sensing, AI application, computing) in congestion scenarios. | PR 5.7.6.2-2 | Unified Access Control |
| Y.1.1-2-4 | The 6G system shall provide optimized network capabilities for FWA (e.g. support stationary devices) in relevant bands taking into consideration the regulatory requirements for each specific band. | PR 5.7.1.2-1 PR 5.7.1.2-2 | Fixed Wireless Access |
| Y.1.1-2-5 | The 6G system shall enable the means to provide awareness of user service characteristics (e.g. data rate, latency) to support the RAN and CN in making real time resource allocation for FWA. | PR 5.7.1.2-3 | Fixed Wireless Access |
| Y.1.1-2-6 | The 6G system shall enhance the Short Message Service to enable a network operator to verify the identity of the SMS sender and information concerning operator verified SMS sender information to the recipient of a SMS. NOTE 1: Operator-verified SMS sender information is used to inform the recipient of a SMS that the identity of the SMS sender is operator-verified and support displaying additional information (e.g. brand name, logo, etc.) of the SMS sender. Human interface aspects are out of scope of this requirement. NOTE 2: Indication that the identity of the SMS sender is operator-verified, any additional information about the SMS sender and the message itself is assumed to be integrity protected. NOTE 3: Based on interworking agreements and trust relationships, the requirements above apply also when the SMS recipient is roaming or receives a SMS from a sender served by other operators. NOTE 4: The requirements above apply to A2P SMS and may apply to Person-to-Person SMS. | PR 5.7.3.2-1 PR 5.7.3.2-2 | SMS |
| Y.1.1-2-7 | Subject to operator’s policy and agreement with 3rd party, the 6G network shall support a mechanism to start and stop offering certain network service(s) in a local area network adapting to the demand of e.g. the users, 3rd party or the network operator. | PR 5.9.6.6-1 | 6G local area network within a PLMN |
| Y.1.1-2-8 | Subject to operator policies, and agreement between the PLMN operator and authorized 3rd party, the 6G network shall support a mechanism to - authorize PLMN’s users to access a subscribed service provided by an authorized 3rd party via a local area network (deployed by the PLMN operator), and - minimize service interruption when the serving network changes between the local area network and the PLMN network. | PR 5.9.6.6-2 | 6G local area network within a PLMN |
| Y.1.1-2-9 | Subject to operator’s policy and regulation, for an operator with multiple 6G core networks, when there is UE mobility from one 6G core network to another 6G core network of the same PLMN, the 6G network shall support efficient traffic routing for the traffic from UE to data network. NOTE 1: The above term does not imply any architectural assumption, e.g. whether 6G CN is a new or evolved CN (compared to 5G). NOTE 2: This requirement only impacts core network. | PR 5.9.7.6-1 | Multiple Core Networks |
| Y.1.1-2-10 | Based on operator policy, the 6G network shall support the ability to allow an authorized 3rd party service provider to provide information of the service characteristics for each traffic flow component of its service/application to the 6G network. | PR-5.9.8.2-1 | Service Awareness |
| Y.1.1-2-11 | Based on operator policy, the 6G network shall support mechanisms to dynamically adjust and optimize network resources based on the service characteristics, including their predicted changes, provided by the service or application. | PR-5.9.8.2-2 | Service Awareness |
| Y.1.1-2-11 | Subject to operator policy and user consent, the 6G system shall support means to provide users with differentiation of QoS and charging based on users’ digital identity information issued by a third party and users’ subscription information. | PR 5.5.9.6-1 | Digital Identity |
| Y.1.1-2-12 | The multimedia telephony service [144] provided by IMS shall be able to minimise user perception of the transition during codec modification of an ongoing voice call, e.g. a codec change during communication link fluctuation. | PR 5.7.8.2-1 | IMS Codec change QoE |

NEW Editor’s Note:  PRs 5.7.1.2-5, PR 5.7.2.2-1, PR 5.7.5.2-1, and PR 5.7.7.6-1 have editor’s notes associated with them, so they are not included in consolation into Table Y.1.1-2 at this time.

Table Y.1.1-3 – Device Support

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.1-3-1 | The 6G system shall support UEs with different characteristics such as data rate, latency, etc. | PR 5.10.1.3-1 | Editor's Note: additional considerations on the above requirement is FFS. |
| Y.1.1-3-2 | The 6G system with satellite access shall support UEs with the performance requirements in Table 5.10.2.2-1 | PR 5.10.2.2-1 | Table values are FFS |

NEW Editor’s Note:  Both PRs for Table Y.1.1-3 have editor’s notes associated with them, but neither of the ENs are against the actual text of the PR. PR 5.10.1.3-1’s EN is about “additional considerations” and PR 5.10.2.2-1’s EN is against the associated Table values.  They are provided for discussion/consideration.

### Y.1.2	Security and Privacy

Editor’s Note: This clause contains consolidated potential requirements for 6G that include security, privacy and lawful interception. This includes (but not limited to) PRs from clauses 5.5 and X.1.

NEW Editor’s Note:  PR 5.5.1.2-5, PR 5.5.4.2-10, and PR 5.5.6.3-1 have editor’s notes associated with them, so they are not included in consolation into Table Y.1.2-1 at this time.

Table Y.1.2-1 – Security and Privacy

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.2-1-1 | The 6G system shall provide FWA a comparable level of security and privacy protection to other 6G services. | PR 5.7.1.2-4 | Fixed Wireless Access |
| Y.1.2-1-2 | The 6G system shall be able to protect UE’s subscriber identities from attacks, and potential attacks posed by quantum computing. | PR 5.5.7.3-3 PR 5.5.2.6-1 | UE Identity protection Quantum Resistant Security |
| Y.1.2-1-3 | The 6G network shall provide security mechanisms for secure access to elements of the core network of the 6G system and enable the network operator to ensure there are no unintended changes of the elements of the 6G network | PR 5.5.1.2-1 PR 5.5.1.2-3 | Network Security |
| Y.1.2-1-4 | The 6G network shall support establishment of secure communication between elements of the network and secure communication on all 3GPP defined interfaces of the core network of the 6G system while protecting network related information (e.g. network element identities, topology) from disclosure to unauthorized parties. | PR 5.5.1.2-1 PR 5.5.1.2-2 | Network Security |
| Y.1.2-1-5 | The 6G system shall provide security protection for communication (e.g. authentication methods) against the potential attacks posed by quantum computing. | PR 5.5.2.6-1 | Quantum Resistant Security |
| Y.1.2-1-6 | The 6G system shall ensure the cryptography agility (i.e. post-quantum cryptography algorithms-related smooth migration, switching, update) for the 6G system and its services to remain secure against new threats. | PR 5.5.2.6-3 | Quantum Resistant Security |
| Y.1.2-1-7 | Subject to operator policy and regulatory requirements, the 6G system shall support a means for a UE to be able to distinguish a False Base Station from an authentic Base Station. | PR 5.5.3.2-1 | System Security False Base Station |
| Y.1.2-1-8 | The 6G system shall provide efficient mechanisms to support authentication and secure communication in inter-PLMN and intra-PLMN networks. | PR 5.5.4.2-1 | System Security Network Security |
| Y.1.2-1-9 | The 6G system shall support fine-grained security isolation based on the security needs of different businesses requirements for intra-PLMN networks. | PR 5.5.4.2-2 | System Security Network Security |
| Y.1.2-1-10 | The 6G system shall be able to provide flexible orchestration of security capabilities to ensure customized security requirements from customers. | PR 5.5.4.2-3 | System Security Network Security Security Management |
| Y.1.2-1-11 | The 6G network shall support security management capabilities that enable - the identification of potential security threats to communication and network elements, - security analysis and security enhancement policy generation to mitigate attacks and/or security issues in operator management system, - security response to implement enhanced security policy from management system to network elements, in order to recover network from disturbance | PR 5.5.4.2-4 PR 5.5.4.2-5 PR 5.5.4.2-6 | Security Management |
| Y.1.2-1-12 | The 6G system shall enable operators using SIM as trust anchor to provide identity and authentication service to the 3rd parties. | PR 5.5.4.2-7 | Identity Authorization Service |
| Y.1.2-1-13 | The 6G system shall support digital identities by - verifying the association between the SIM identity and the digital identity of the different digital representatives, - authorising required services of the digital identity of the digital representatives of users and revoking the digital identity if needed. | PR 5.5.4.2-8 PR 5.5.4.2-9 | Digital Identity |
| Y.1.2-1-14 | The 6G system shall support security and privacy protection of the 6G System Data. | PR 5.9.2.2-4 | 6G System Data System Security |
| Y.1.2-1-15 | Subject to operator policy and regulatory requirements, the 6G system shall support privacy protection, location privacy, identity protection for UEs accessing 6G network for services (e.g. communication, sensing, AI inferencing), and for the corresponding information for any information exposure to an authorized 3rd party. | PR 5.5.7.3-1 PR 5.5.7.3-2 | Security & Privacy for services, data & exposure |
| Y.1.2-1-16 | Subject to regulation, operator policy and user consent, the 6G system shall support mechanisms to protect data privacy during the processing of sensing data. NOTE: The processing of sensing data is compliance with applicable regulations, and other pre-defined service criteria. | PR 7.12.6-2 | Data Privacy Sensing Data Processing |

Table Y.1.2-2– Lawful Interception

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.2-2-1 | The 6G system shall be able to support the following services: Lawful Interception, ref TS 22.261 [14] and TS 33.126 [312] and Other regulatory services, based on regional/national regulatory requirements. | Clause 5.4.2 | Could be considered as part of Table Y.1.1-1 |
| Y.1.2-2-2 | The 6G network shall provide security mechanisms that offer protection of communication and data while at the same time not hindering Lawful Interception (ref TS 33.126 [312], clause 6.4). | PR 5.5.1.2-4 | Could be considered as part of Table 1.2.-1 |
| Y.1.2-2-3 | The 6G system shall support the mapping of permanent and temporary identifiers of all subscribers, e.g. for Lawful Interception purposes. | PR 5.5.2.6-2 | Identifiers |
| Y.1.2-2-4 | Subject to national or regional regulatory requirements, the 6G system shall provide a privacy override mechanism to enable access to retained data for law enforcement purposes and Lawful Interception purposes which may otherwise be subject to the privacy protection requirements [313]. | PR 5.5.7.3-4 | Privacy override |
| Y.1.2-2-5 | Subject to operator policy and regulatory requirement, the intelligent immersive calling service shall comply to the existing regulatory requirements (e.g. lawful interception). | PR 9.10.6-2 | Intelligent immersive calling service |
| Y.1.2-2-6 | In addition to the LI requirements specified in TS 22.261 [14] the 6G system shall support Lawful Interception - Where the 6G system is operated or where the services are provided, - When (some) network functions are 3rd party provided or operated by different operators, - In all supported interworking and interoperating scenarios, including roaming (in visited networks) NOTE: These requirements apply to all services which must comply with the 3GPP SA3-LI specifications, subject to national or regional regulations. | Clause X.1 | How much of this is covered as part of Table Y.1.1-1? |
| Y.1.2-2-7 | The 6G system shall support the retention of data to meet national or regional regulatory requirements, [314], [315], [313], and [316]. | Clause X.1 | Data retention Could be considered as part of Table Y.1.5-1 – Data Collection and Consumption |

### Y.1.3	Resilience

Editor’s Note: This clause contains consolidated potential requirements for 6G that address resiliency. This includes (but not limited to) PRs from clauses 5.6 and 5.9.

Table Y.1.3-1 – Resilience

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.3-1-1 | Subject to regulatory requirements and operator’s policy, 6G network should support suitable means to ensure the stability of services provided by the edge network in case of abnormal transmission occurring between the public network and the edge network. | PR 5.6.1.2-2 | Edge resilience |
| Y.1.3-1-2 | Subject to operator policies and service level agreements, the 6G system shall support on-demand rollout (e.g. within hours) of new or updated services/capabilities - with minimal disruption to existing services, including the ability to efficiently rollback those services/capabilities, as needed (e.g. in case of failures or demand from other services), while minimizing the impact to the user experience | PR 5.9.5.6-1 PR 5.9.5.6-2 | On-demand/quick rollout for new/updated services |
| Y.1.3-1-x1 | Subject to operator policies and service level agreements, the 6G system shall enable operators to provision network services as part of the operator’s PLMN network on-demand, e.g. in response to an urgent event (e.g. disaster, emergency and DDoS events), with certain level of local control and specific functionalities in a given area during a specific time period. NOTE 1: The level of local control can be based on operator policies and agreements with 3rd party. For example, the authorization and policy control of users to access the provisioned services are not affected by the failure of the operator’s PLMN network. NOTE 2: The enabled functionalities can be based on operator policies and agreements with 3rd party. For example, data connectivity service and voice service are prioritized when an urgent event happens in a residential community; small data transfer service is prioritized when an urgent event happens in an IoT based farmland. NOTE 3: Some situations can target the required network services to be provisioned within hours to serve certain users whose QoE is impacted by an urgent event. NOTE 4: Local control refers to the capability of part of the operator’s PLMN network to operate autonomously and independently, e.g. management of local subscription, local traffic, without interaction with the operator’s PLMN. | PR 5.6.2.6-1 | Fast network provisioning to improve resilience |
| Y.1.3-1-x2 | The 6G system shall be able to support means to simplify network operation and service delivery compared to 5G system, e.g. in order to reduce signalling, minimise connection setup time. NOTE: This simplification should not mean reducing service quality provided. | PR 5.9.4.2-1 | simplify network operation and service delivery |
| Y.1.3-1-3 | Subject to operator policies and service level agreements, the 6G system shall enable a network operator to authorize a UE, that is subscribed to local network services, to access services from the PLMN of the same operator. NOTE 5: This applies to scenarios where a service is not available in the local network services that have been provisioned on-demand but is available from the PLMN of the same operator. | PR 5.6.2.6-2 | Same operator/cross PLMN service |
| Y.1.3-1-4 | Subject to operator policy and regulatory requirements, the 6G system shall be able to minimize service interruption or degradation during disasters taking into account the affected/not affected parts of a single or multiple network(s) serving an area impacted by a specific type of disaster. NOTE: In case of multiple networks serving the disaster area, it is assumed that under disaster conditions, the remaining available parts of the networks in that area can serve the UEs of other networks. | PR 5.6.4.6-1 |  |
| Y.1.3-1-5 | The 6G network shall provide a mechanism to minimize the signalling arising from recovery of network failure. NOTE: Massive signalling can happen e.g. when a network element fails and registrations or data sessions are re-routed to a backup network element. | PR 5.6.5.6-1 | Recovery |

### Y.1.4	Energy-related aspects

Editor’s Note: This clause contains consolidated potential requirements for 6G that address energy efficiency and sustainability. This includes (but not limited to) PRs from clause 5.8.

NEW Editor’s Note:  PR 5.8.2.6-2 has an editor’s note associated with it (concerning energy consumption information), so it is not included in consolation into Table Y.1.4-1 at this time.

Table Y.1.4-1 – Energy-related aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.4-1-1 | The 6G system shall improve energy efficiency of 6G system compared to 5G system. | PR 5.8.1.6-1 | System Energy Efficiency |
| Y.1.4-1-2 | The 6G system shall support mechanisms to improve UE energy efficiency when providing services to the subscriber, optimizing the UE energy saving considering the current traffic conditions of the UE (subject to local regulations and user consent), enabling task offloading from the UE to the Service Hosting Environment, considering battery life and thermal issue of UE, and provide means for a user to provide a preference for UE energy efficiency. NOTE: The 6G network uses this preference when improving UE energy efficiency. Examples of preference include: - user preference on prioritizing service performance, - prioritizing UE energy efficiency, or - prioritizing essential service (e.g. voice call) while improving UE energy efficiency. | PR 5.8.1.6-2 PR 5.8.1.6-3 PR 5.8.4.6-2 PR 5.8.5.6-1 | UE Energy Efficiency |
| Y.1.4-1-3 | The 6G network with multiple access networks (e.g. terrestrial, satellite) shall provide means to determine (e.g. through measurement and/or calculation) the energy consumed by the access networks for the provision of a one-to-many service (e.g. PWS [62], broadcast, multicast) characterised by at least the QoS, targeted service area and/or targeted set of users. | PR 5.8.2.6-1 | Access Energy Consumption |
| Y.1.4-1-4 | Subject to operator’s policy, the 6G system shall support network slice-specific energy management policies that - define a maximum slice energy credit limit for services without QoS criteria, - support a mechanism to perform slice energy credit limit control for services without QoS criteria. NOTE: The result of the credit control is not specified by this requirement. | PR 5.8.3.6-1 PR 5.8.3.6-2 | Slice Energy Management |
| Y.1.4-1-5 | Subject to local regulation and user consent, the 6G system shall be able to optimize network and UE energy saving jointly, while meeting the service performance requirements. | PR 5.8.4.6-3 | System Energy Saving |
| Y.1.4-1-6 | Subject to operator’s policy, regulation and user’s consent, the 6G network shall provide suitable energy saving methods targeting different scenarios (e.g. different working time) and optimize the network energy saving considering the current traffic conditions of the network (subject to local regulation). | PR 5.8.6.6-1 PR 5.8.4.6-1 | Network Energy Saving |
| Y.1.4-1-7 | Subject to regulation and operator’s policy, the 6G network shall be able to expose to a trusted third-party the network energy consumption information including the energy consumption related to sensing, AI, and computing services, over a specific time period (e.g. month etc.). | PR 5.8.6.6-2 | Network Energy Consumption |
| Y.1.4-1-8 | Subject to user consent and operator policy, the 6G system shall support a means for a group of cooperating UEs to reduce the energy consumption for communication of the group of UEs whilst meeting requested service performance. | PR 5.8.8.6-1 | Cooperating UEs Energy Consumption |
| Y.1.4-1-9 | The 6G system should support energy-efficient sensing operations. | PR 7.5.6-2 | Energy Efficient Sensing |

### Y.1.5	Data Collection and Consumption

Editor’s Note: This clause contains consolidated potential requirements for 6G that address data collection, management, and consumption. This includes (but not limited to) PRs from clauses 5.9.2 and W.

NEW Editor’s Note:  PR 5.9.2.2-2, -3, and -6 have editor’s notes associated with them (concerning user consent), so they are not included in consolation into Table Y.1.5-1 at this time.

Table Y.1.5-1 – Data Collection and Consumption

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.5-1-1 | The 6G system shall support mechanisms for 6G System Data collection and consumption minimizing the impact to 6G services. | PR 5.9.2.2-1 |  |
| Y.1.5-1-2 | Subject to regulation and operator policy, the 6G system shall support transfer 6G System Data between different data providers and data consumers within the 6G system. | PR 5.9.2.2-7 |  |
| Y.1.5-1-3 | Subject to regulation, operator(s) policy and user consent, the 6G network shall support access from authorized third parties to processed data related to UEs served by the network (for example but not limited to number of UEs in a geographical location, their mobility pattern, application usage trends) without exposing UE identities and individual user data including Personally Identifiable Information or sensitive data. NOTE 1: Processed data refers to the analysis of data to produce new data including among other statistics on the data, correlating data, aggregating data. NOTE 2: The existing security, privacy policies and procedures for collecting and processing user data (e.g. via anonymisation) will be preserved. | PR 5.5.5.3-1 | Processed Data |

### Y.1.6	Charging

Editor’s Note: This includes (but not limited to) PRs on charging throughout the document.

NEW Editor’s Note: PR 7.17.6-4 is for FFS, so it is not included in consolation into Table Y.1.6-1 at this time.

Table Y.1.6-1 – Charging

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.6-1-1 | Subject to operator policy and user consent, the 6G system shall support means to provide users with differentiation of QoS and charging based on users’ digital identity information issued by a third party and users’ subscription information. | PR 5.5.9.6-1 | Digital Identity |
| Y.1.6-1-2 | The 6G system shall be able to provide charging and accounting mechanisms for 6G System Data. | PR 5.9.2.2-5 | System Data |
| Y.1.6-1-3 | The 6G system shall provide appropriate charging support for differentiated services per media component (e.g. to meet SLA requirements). | PR-5.9.8.2-3 | Service Awareness Differentiated Svcs SLA support |
| Y.1.6-1-4 | The 6G network shall support charging for the usage of network resources by third parties. | PR 6.2.6-2 | 3rd party charging |
| Y.1.6-1-5 | The 6G network shall support charging information collection for the intelligent communication assistant service. | PR 6.11.6-2 | Intelligent Communication Assistant |
| Y.1.6-1-6 | The 6G system shall be able to collect charging information for the computing resource(s) coordinated by the 6G network e.g. per request, per UE. | PR 6.14.6-1 | Computing |
| Y.1.6-1-7 | The 6G network shall support charging information collection per UE and/or for the usage of network resources related to multiple 3GPP services (e.g. communication service, sensing service, AI service) to satisfy the intents received from the UE. | PR 6.21.6-5 | UE charging collection Charging for multiple services |
| Y.1.6-1-8 | The 6G network (e.g. in conjunction to IMS) shall support charging information collection for the intelligent calling service. | PR 6.22.6-2 | IMS Intelligent Calling Service |
| Y.1.6-1-9 | Subject to operator’s policy, the 6G network shall support the collection of charging information for the AI/ML model training, AI model inference and management which are within the service hosting environment for authorized third parties. | PR.6.25.6-3 | AI/ML model training AI model inference & Mgt |
| Y.1.6-1-10 | The 6G network shall be able to collect charging information based on unique traffic characteristics pertinent to e.g. specific GenAI applications, or enhanced/guaranteed user experience. | PR 6.26.6-4 | Charging for unique traffic characteristics or enhanced/guaranteed user experience |
| Y.1.6-1-11 | The 6G network shall be able to collect charging information for AI service per user. | PR 6.29.6-2 | AI service |
| Y.1.6-1-12 | The 6G network shall be able to collect charging information for the usage of AI/ML models that are stored or generated within the Service Hosting Environment. | PR 6.36.6-3 | AL/ML model usage |
| Y.1.6-1-13 | Based on operator policy and user consent, the 6G network shall support mechanism to collect charging information for customized service based on received intent(s) from user. | PR 6.44.6-3 | Customized Service based on received intent(s) |
| Y.1.6-1-14 | Based on regulatory requirements, operators’ policy and agreement with 3rd party, 6G network shall support charging for services provided to 3rd party AI agents (e.g. combination of communication, computing service, sensing service, etc.). | PR 6.55.6-2 | 3rd party AI agents |
| Y.1.6-1-15 | The 6G system shall support collection of charging information associated with initiating and terminating an intelligent immersive calling service. | PR 9.10.6-3 | Intelligent Immersive Calling Service |

### Y.1.7	Other operational aspects

Editor’s Note: This includes (but not limited to) PRs from clause 5.5.8, 5.8.7, 5.9.3, throughout the document.

Table Y.1.7-1 – NDT

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.7-1-1 | Subject to operator’s policy, the 6G network should be able to support a Network Digital Twin for network and service management, enabling autonomous networks (including network and OAM) for better service offering, e.g. through simulating in the network characteristics and behaviours provide services to users (e.g. user traffic running on the virtual replica of mobile network or part of it). NOTE 2: The network characteristics can be NFs configuration, UE throughput in NFs, the number of current subscribers, UE QoS, user experience data, fault prediction and non 3GPP data such as user complaint data etc. | PR 5.5.8.6-1 PR 5.9.3.6-1 | Basie NDT functionality |
| Y.1.7-1-2 | The 6G network shall provide suitable means for communicating with the NDT of operator controlled compute resources that are in the Service Hosting Environment or the cloud, for the purpose of energy consumption analysis and optimisation. | PR 5.8.7.6-1 | Network Energy Consumption (note) Could also go into Table Y.1.4-1 – Energy-related aspects |
| Y.1.7-1-3 | The 6G system shall expose an API for the 6G network to transfer network function related information (e.g. NF configuration, UE context related data such as UE QoS etc.) to NDT to replicate 6G network in NDT. | PR 5.9.3.6-2 | (note) |
| Y.1.7-1-4 | The 6G system shall expose an API for the 6G network to receive feedback (e.g. optimal QoS parameter values, NF configuration etc.) from NDT. | PR 5.9.3.6-3 | NDT Exposure (note) |
| Y.1.7-1-5 | Subject to operator’s policy, the 6G system shall be able to transfer the data (e.g. related to service outage) from 6G Network to NDT. | PR 5.9.3.6-4 | (note) |
| NOTE: These requirements only apply if the 6G network supports a Network Digital Twin. |  |  |  |

Table Y.1.7-2 – OAM

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.7-2-1 | Subject to operators’ policies, regulations and user consent, the 6G system shall provide means for a network operator to monitor network coverage and/or traffic usage, including collection of information from UEs (with subscription to the network operator). NOTE 1: Monitoring and collection of information from a UE is assumed to be authorized and configured by the UE’s home operator, for certain geographical area(s) and/or time(s), NOTE 2: The traffic usage information collected from a UE is for traffic associated with that UE. | PR 5.7.9.3-1 | Network Metrics Could also go into Table Y.1.5-1 – Data Collection and Consumption |
| Y.1.7-2-2 | Subject to operator’s policy, the 6G network (OAM) should support autonomous detection, isolation, and recovery of failed service with minimum human intervention, to ensure continuous operation. | PR 5.6.1.2-1 PR 5.6.3.6-1 | Resilience |