3GPP TSG-SA WG1 Meeting #112 	S1-25xxxx

17-21 November 2025, Dallas, USA	(revision of S1-25xxxx)

Title:	Rapporteur proposed potential requirements allocation for CPRs Y.1.14

Agenda Item:	TBD

Source:	6G Study Rapporteurs

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: Initial proposed potential requirements allocation for CPRs in Y.1.14

Editor’s Note:  PRs with associated Editor’s Note(s) will not be considered for consolidation until Editor’s Note(s) associated with the PR are resolved.

Editor’s Note: CPR Table(s) may be modified (e.g. split, merged, renamed, moved) during the consolidation process.

# Y	Consolidated Potential Requirements

### Y.1.14	Industry and Verticals

Editor’s Note: This includes (but not limited to) PRs from clause 11.

Table Y.1.14-1 – UAV, UAM and aircraft

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.14-1-1 | 6G system with satellite access shall provide a means to obtain and expose application service interruption information (e.g. expected interruptions to the service) to authorized third party to minimize interruption for immersive media services. | PR 11.5.6-2 | Exposure |
| CPR Y.1.14-1-2 | Subject to regulatory requirements and user consent, the 6G network shall provide secure means to expose to an authorized 3rd party application the information related to a UAV or a UAM aircraft from the stored service data. | PR 11.7.6-2 | Exposure |
| CPR Y.1.14-1-3 | Subject to regulatory requirements and user consent, the 6G network shall be able to securely store the service data for a UAV or a UAM aircraft based on the request information (e.g. service type, storage duration, time expiry). NOTE: The service data may be the processing data for a specific network service such as sensing data, positioning measurement data, or the exposed data such as sensing results, positioning information. | PR 11.7.6-1 | Data storage |

Table Y.1.14-2 – Localized network

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.14-2-1 | Subject to an agreement between a PLMN operator and authorized 3rd party, operator policies and regulatory requirements, the 6G network shall support a mechanism to authorize UEs owned by a trusted authorized 3rd party (e.g. vertical industry) to access services provided by a localized network (deployed by the PLMN operator for the authorized 3rd party). | PR 11.10.6-1 | Authorization |
| CPR Y.1.14-2-2 | Subject to an agreement between a PLMN operator and authorized 3rd party, operator policies and regulatory requirements, the 6G network shall support a mechanism to authorize and authenticate UEs owned by a trusted authorized 3rd party to access services from the PLMN operator. NOTE: The realization or deployment of localized network may be an enhancement of NPN, PALS or other network topology options. | PR 11.10.6-2 | Authorization |
| CPR Y.1.14-2-3 | Subject to regulatory requirements and the operators’ policy, the 6G system shall provide the possibility of local authorization for a new device or UE joining the local network. | PR 11.11.6-3 | Authorization |
| CPR Y.1.14-2-4 | The 6G network shall support a mechanism to allow the operator or authorized 3rd party to perform local control of the localized network when the localized network loses connection with the PLMN network. | PR 11.10.6-3 | Local control |
| CPR Y.1.14-2-5 | Subject to regulatory requirements and the operators’ policy, 6G system shall be able to support (partial-)autonomous local networks for the communication between UEs and devices. NOTE: (Partial-)autonomous means that the management of local network can be done locally even in case of loss of connectivity to the mobile network. | PR 11.11.6-1 | Local control |
| CPR Y.1.14-2-6 | Subject to regulatory requirements and operator policy, the 6G network shall allow the operator to provide the required control information including configurations to the local network that enables it to provide necessary configurations for the local communication and to operate also out of coverage of the mobile network. | PR 11.11.6-2 | Local control |
| CPR Y.1.14-2-7 | Subject to operator’s policy, the 6G network shall support means to enable communication between UEs and a service hosting environment in a local 6G network, managed by an authorized 3rd party, when the connection with the remote (non-local) network fails. NOTE 1: The local 6G network is composed of at least 6G RAN and core network of 6G which are deployed in the local area e.g. factory. NOTE 2: The above assumes agreement with the 3rd party and control by the core network of 6G system | PR 11.23.6-1 | Local control |
| CPR Y.1.14-2-8 | The 6G system shall support a mechanism to allow the UE to discover and select the services provided by a localized network. | PR 11.10.6-4 | Service discover and select |

Table Y.1.14-3 – Mission critical services

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.14-3-1 | The 6G system shall support MCX service continuity during UE mobility across inter-PLMN, as well as between PLMN and PSBN, subject to regulatory requirements, inter-operator agreements and capabilities (e.g. agreed Quality, Priority and Pre-emption parameter settings). NOTE 1: (PSBN is a network generally owned and operated by a public safety network operator and specially designed for public safety users (e.g. police, ambulance, fire service), having their own PLMN identity. | PR 11.14.6-1 |  |
| CPR Y.1.14-3-2 | The 6G system shall minimize interruption to ongoing MCX service during UE mobility across different access (e.g. terrestrial access and satellite access), and/or when MCX service switch between on-network and off-network operations with or without UE relay through UAVs, HAPS, and Satellites access. NOTE 2: The requirement above may include also ProSe-enabled access, where relay UEs are equipped with co-located routing functionality, capable of maintaining connectivity in dynamic and infrastructure-less environments. | PR 11.14.6-2 |  |
| CPR Y.1.14-3-3 | The 6G system shall enable an MCX UE to determine its position using hybrid positioning methods, including network-assisted GNSS via satellite access, to improve availability and resiliency, and to convey its position information to the MCX server, triggered by an event, or periodically. | PR 11.14.6-3 |  |
| CPR Y.1.14-3-4 | The 6G system shall support the MCPTT Service to enable interworking with non-3GPP PTT Systems that are compliant with the PDT standards, in alignment with P25 and TETRA. | PR 11.14.6-4 |  |
| CPR Y.1.14-3-5 | The 6G system shall support interworking between the MCData SDS and PDT Short Data Service, in alignment with TETRA. | PR 11.14.6-5 |  |

Table Y.1.14-4 – Robotics

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.14-4-1 | Subject to operator policy and regulatory requirements, the 6G system shall be able to provide a means to support efficient data sharing among multiple UEs (e.g. AMRs) initiated by a UE (e.g. AMR) considering the dynamics of the communication, e.g. frequent change of the involved UEs and the varied duration of the group communication to match the lifecycle of the task, etc. NOTE 3: The shared data is collaborative awareness data that a UE (AMR) shares with other UEs (AMRs) and can include the real time control/manoeuvre with its immediate plans. | PR 11.9.6-2 | Data sharing |
| CPR Y.1.14-4-2 | The 6G system shall be able to provide a means to expose its ability to meet the accuracy level of clock synchronization for a group of UEs (service robots) requested by trusted third party. | PR 11.12.6-1 | exposure |
| CPR Y.1.14-4-3 | The 6G system shall be able to provide a means to check its ability to meet the recovery time from direct network connection interruption, requested by trusted third party. | PR 11.12.6-2 | exposure |
| CPR Y.1.14-4-4 | The 6G system shall be able to provide a means to expose its ability to meet the recovery time from direct network connection interruption, requested by trusted third party | PR 11.12.6-3 | exposure |
| CPR Y.1.14-4-5 | Subject to the operator’s policy and service agreement, the 6G system shall provide a suitable means that enables remote UE to operate under extreme conditions via indirect network connection. NOTE: UEs operating under extreme conditions include UEs under difficult networking conditions where more advanced mechanisms are needed, such as mining robots, carts and collaborative robots in an underground mining site. | PR 11.13.6-1 | indirect network connection |
| CPR Y.1.14-4-6 | While the remote UE is using multiple indirect network connection paths for a single traffic flow, the 6G system shall be able to maintain service continuity when the communication path(s) to the network changes, e.g. when existing path is released and a new path is added. | PR 11.15.6-1 | indirect network connection |

Table Y.1.14-5 – Other

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.14-5-1 | The 6G network with AI capabilities should be able to collaborate with the AI capabilities in the authorized third-party (e.g.: split inference) to assist Real Time Digital Twin in meeting the required service performance. | PR 11.4.6-2 | Digital Twin |
| CPR Y.1.14-5-2 | The 6G network should be able to provide multiple types of data, such as network status data, sensing data and etc., to enable Real Time Digital Twin. | PR 11.4.6-3 | Digital Twin |
| CPR Y.1.14-5-3 | Subject to operator policy, agreement with the 3rd party and user’s consent, the 6G network shall support mechanisms to process the data collected from 3GPP UEs (e.g. AR split-rendering), in the Service Hosting Environment. | PR 11.8.6-1 | Data process |
| CPR Y.1.14-5-4 | Subject to operator’s policy, regulatory requirements, agreement with the 3rd party, and user consent, the 6G system shall support to collect data from multiple 3GPP UEs (e.g. XR device, High-resolution camera, 3D scanner, depth sensors etc.) which belong to one 3rd party, within an given timeframe, and associate the collected data with one application of the 3rd party. | PR 11.8.6-2 | Data collection |
| CPR Y.1.14-5-5 | Subject to operator policy, regulatory requirements and user consent, the 6G network shall provide suitable APIs to allow authorized third parties to collect data from UEs (e.g. connected vehicles) that are located in a specific area and are capable of collecting data upon network request. | PR 11.19.6-1 | Data collection |
| CPR Y.1.14-5-6 | Subject to operator policy, regulatory requirements and user consent, the 6G network shall enable UEs (e.g. connected vehicles) to indicate whether they can collect data upon network request. | PR 11.19.6-2 | Data collection |
| CPR Y.1.14-5-7 | Subject to operator’s policy, agreement with the 3rd party, the 6G network shall be able to expose and update data processing result to the 3rd party. | PR 11.8.6-3 | Data exposure |
| CPR Y.1.14-5-8 | Subject to operator’s policy, the 6G network shall be able to expose information related to a network slice (e.g. current or predicted latency and reliability) to the authorized third party. | PR 11.21.6-1 | Exposure |
| CPR Y.1.14-5-9 | Subject to regulatory requirements and operator policy, the 6G system shall support a means for an MNO to improve availability for regulated services (e.g. PWS [62], emergency call [58]) despite a lack of energy supply, through exchange of information with a DSO. | PR 11.18.6-1 |  |
| CPR Y.1.14-5-x1 | Subject to operator’s policy and agreement with the 3rd party, the 6G system shall provide a mechanism for operators to configure, adjust and manage the computing resource in Service Hosting Environment to support factory digital twin based on e.g. UE location, 6G Computing Service requirements from 3rd party. | PR 11.22.6-1 | Computing support factory digital twin |
| CPR Y.1.14-5-x2 | Subject to operator’s policy and agreement with the 3rd party, the 6G system shall support to select one or more Service Hosting Environment to fulfil the 6G Computing Service as required by factory digital twin application from 3rd party, considering the UE location, computing capabilities and traffic load, etc. | PR 11.22.6-2 | Computing support by multiple SHEs |
| CPR Y.1.14-5-10 | The 6G System shall ensure that IoT communication services shall minimize impact on other services (e.g. regular voice, video and data). | PR 11.25.6-1 | IoT |
| CPR Y.1.14-5-11 | Subject to the Operator’s policies and control, the 6G system shall provide means to support extended coverage. | PR 11.25.6-2 | IoT |
| CPR Y.1.14-5-12 | The 6G System shall support diverse device types with long lifetime (e.g. 10 to 20 years). NOTE: 3GPP should consider support of device lifetimes longer than the lifetime of the supporting 6G core network. | PR 11.25.6-3 | IoT |

Table Y.x-6 – KPI

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
|  | The 6G System shall support the transmission of sensing result information to a UAM aircraft about its surroundings with the following KPI requirements. | PR 11.2.6-1 |  |
|  | The 6G System shall support services provided to the UAM applications (for the passengers and devices onboard the aircraft) with the following KPI requirements. | PR 11.2.6-2 |  |
|  | The 6G System shall be able to support the following performance requirements to support Cooperating Mobile Robots. | PR 11.3.6-1 |  |
|  | The 6G System shall be able to support the following positioning performance requirements to support Cooperating Mobile Robots. | PR 11.3.6-2 |  |
|  | 6G system with satellite access shall provide a means to support immersive media services with the following KPIs: | PR 11.5.6-1 |  |
|  | The 6G System shall support high-data rate aircraft communications using satellite access with the following KPI requirements. | PR 11.6.6-1 |  |
|  | The 6G System shall support high data rate aircraft communications for on-board UEs with the following KPI requirements. | PR 11.6.6-2 |  |
|  | The 6G System shall support high data-rate aircraft communications with massive data transfer when the aircraft is on the ground or near the ground with the following KPI requirements. | PR 11.6.6-3 |  |
|  | Subject to operator’s policy and agreement with the 3rd party, the 6G system shall support positioning performance requirements in Table 11.8.6-1. | PR 11.8.6-4 |  |
|  | The 6G system shall support communication performance requirements in Table 11.8.6-2. | PR 11.8.6-5 |  |
|  | Subject to operator policy and regulatory requirements, the 6G system shall be able to provide a means to support data sharing among multiple UEs (AMRs) served by one or more 6G networks moving in a coverage area according to the following KPIs (Table 11.9.6-1). NOTE 1: The shared data is collaborative awareness data that a UE (AMR) shares with other UEs (AMRs) and can include the real time control/manoeuvre with its immediate plans, and its allowed range on ground mobility, types of role with which other UEs (AMRs) can get aware of what would be necessary for that UE (AMR) in mutual decision-making in the warehouse. NOTE 2: The data sharing is via direct network connection or via direct device connection. | PR 11.9.6-1 |  |
|  | The 6G system shall be able to support the following KPIs. | PR 11.16.6-1 |  |
|  | The 6G system shall be able to support the following KPIs utilising the 3GPP and non-3GPP positioning technologies. | PR 11.17.6-1 |  |
|  | Subject to operator policy and user consent, the 6G system shall support the communication between two UEs via direct network connection in a local network with fulfilling the following KPIs: | PR 11.20.6-1 |  |

Table Y.x-7 – FFS Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
|  | The 6G system should be able to enable a Real Time Digital Twin service with following KPIs. | PR 11.4.6-1 |  |
|  | Subject to Operator’s policy, the 6G network shall support end-to-end, secure and reliable IoT device communication with the network following KPI’s in Table 11.24.6-1 | PR 11.24.6-1 |  |
|  | Subject to Utility Company and Mobile Operator’s policy, the 6G network should provide alternate means to communicate between devices at both ends in the event when a UE loses its connectivity to the 6G system. | PR 11.24.6-2 |  |
|  | Subject to the Utility Operator’s and MNO policies, the 6G system shall support the 6G IoT communication services with the following KPI values: | PR 11.25.6-4 |  |
|  | The 6G system shall be able to support the following KPIs associated: | PR 11.26.6-1 |  |