3GPP TSG SA WG 1 Meeting #112 	S1-254291r1

17-21 November 2025, Dallas, Texas, USA	(revision of S1-25xxxx)

Source:	6G Study Rapporteurs

pCR Title:	Merged Proposed changes on Clauses Y.1.1 to Y.1.7 (Basic Services & Capabilities)

Draft Spec:	3GPP TR 22.870 v0.4.1

Agenda item:	8.1.10

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: Rapporteur’s collection of proposed changes to CPRs in Clauses Y.1.1- Y.1.7

1. Introduction

The following TDOCs submitted into SA1 # 112 have been collected into this document to faciliitate discussion and approval of CPRs.

S1-254001, 2st Draft Agenda for SA1#112 was used as the basis of this contribution.

- S1-54020 (Clause Y.1.1 from zip file)

- S1-254200 (Deutsche Telekom, Charter): Consolidation of clauses 5.1 to 5.4

- S1-254191 (ZTE Corporation):  New tables Y.1.X-1 Localized network and -2 Localized network in vertical

- S1-254096 (ZTE Corporation):  Tables Y.1.1-1, and Y.1.1 -2

- S1-254085 (Charter Communications, Inc): Table Y.1.1-1

- S1-254166 (CEWiT): Y.1.2 (Table Y.1.2-1) moves CPR to Y.1.5 (Table Y.1.5-1)

- S1-254092 (China Telecom): Table Y.1.3-1

- S1-254159 (NTT DOCOMO): Table Y.1.3-1

- S1-254160 (ZTE Corporation): Table Y.1.4-1

- S1-254131 (NEC Corporation (ARIB)): Table Y.1.5-1

- S1-254130 (NEC Corporation (ARIB)): Table Y.1.7-2

- S1-254250 and S1-254250r1 (Qualcomm)

- S1-254300r1 (Huawei)

- Email reflector discussions (see individual comments)

Editor’s Note: If some of these are approved, changes to CPR Tables from other clauses may need to be aligned.

This TDOC is the output of the initial consolidation discussions for Basic Services & Capabilities.

2. Reason for Change

There are several proposals to consolidate functional requirements. This combines the proposals to facilitate discussions in the meeting.

3. Proposal

It is proposed to disscuss and agree the following changes for inclusion into Clause Y in 3GPP TR 22.870 v0.4.1.

* * * First Change: Y, Y.1, Y.1.1 and

Table Y.1.1-1 – Support for legacy services and capabilities * * *

# Y	Consolidated potential functional and performance requirements

Editor’s Note:  PRs with associated Editor’s Note(s) will not be considered for consolidation until Editor’s Note(s) associated with the PR are resolved.

Editor’s Note: CPR Table(s) can be modified (e.g. split, merged, renamed, moved) during the consolidation process.

## Y.1	Consolidated potential Functional Requirements

### Y.1.1	Basic Services and Capabilities

Editor’s Note: This clause contains consolidated potential requirements for 6G that include (but not limited to):	Support for Legacy (i.e., 5G and earlier) services and capabilities, Enhancements to Legacy services and capabilities, Legacy services and capabilities not supported in 6G, and device support. This includes (but not limited to) PRs from clauses 5.2, 5.3, 5.4, 5.7, 5.9, and 5.10.

Proposed Comments/Changes:

S1-254200 (DT/Charter):  Add the following text:

All assumptions and requirements from clauses 5.1 to 5.4 in this TR apply without modification and should be progressed to a TS in the normative phase.

- Rapporteur comment: Would result in deleting Y.1.1-1-1 through Y.1.1-1-6. What is the resolution for the following PRs (in red in initial proposed table)?

  - PR 5.5.10.2-1

  - PR 5.7.4.2-1

  - PR 5.7.6.2-1

  - PR 5.9.1.3-1

  - PR 5.5.10.2-1

  - PR 5.7.4.2-1

  - PR 5.7.6.2-1

  - PR 5.9.1.3-1

- Rapporteur comment: S1-254085 (Charter et al) conflicts with this proposal.

S1-254096 (ZTE): adds the following NOTE and EN:

NOTE:	Terms referring to 5G (e.g. “the 5G system”, “NG RAN”) should be implicitly replaced by the corresponding terms for 6G (e.g. “6G system”, “radio access network of the 6G system”) in requirements in Table Y.1.1-1.

Editor’s Note: Clauses 5.1 and 5.4.1 are introduction or description parts. It is suggested not to include in the CPR table.

S1-254250 (Qualcomm) comment to rename Table Y.1.1-1 – Support for Legacy services, requirements  and inter-system.

[QC]: for this set of PRs, we suggest to import & consolidate the requirements only (not the intro/surrounding text), from TR clauses 5.2-4, with the following order: 5.4.2, 5.4.3, 5.2, 5.3. Intro/additional text can be discussed later. We could also split/separate the inter-system requirements (from the rest).

Table Y.1.1-1 – Support for legacy services and capabilities

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.1-1-1 | The 6G system shall support new 6G services and capabilities and existing 5G services and system requirements, unless explicitly identified. This includes requirements in [14] clause 5.1.2.2, related to inter-RAT capabilities not to be supported by 5GS, apply similarly to 6GS with the following modification: - voice service continuity from the radio access network of the 6G system to UTRAN CS shall not be supported. NOTE: Terms referring to 5G (e.g. “the 5G system”, “NG RAN”) should be implicitly replaced by the corresponding terms for 6G (e.g. “6G system”, “radio access network of the 6G system”) in requirements in Table Y.1.1-1 | Clause 5.1 Clause 5.2 PR 5.5.10.2-1 PR 5.7.4.2-1 PR 5.7.6.2-1 PR 5.9.1.3-1 | The following four PRs “should be” implicitly included in Y.1.1-1 and not needed to be identified separately. Proposed to be changed by S1-254096 |
| Y.1.1-1-1 | The 6G system shall support new 6G services and capabilities and existing 5G services and system requirements, unless explicitly identified. This includes requirements in [14] clause 5.1.2.2, related to inter-RAT capabilities not to be supported by 5GS, apply similarly to 6GS with the following modification: - voice service continuity from the radio access network of the 6G system to UTRAN CS shall not be supported. | Clause 5.1 Clause 5.2 PR 5.5.10.2-1 PR 5.7.4.2-1 PR 5.7.6.2-1 PR 5.9.1.3-1 | The following four PRs “should be” implicitly included in Y.1.1-1 and not needed to be identified separately. Proposed to be changed by S1-254096 |
|  | The 6G system shall have a means to ensure that the visited network provides services to an inbound roamer only once the subscriber identity (e.g. SUPI, IMSI) is obtained from the home network. | PR 5.5.10.2-1 | Roaming Proposed to be changed by S1-254096 |
|  | The 6G system shall support network sharing, based on 5GS network sharing requirements. | PR 5.7.4.2-1 | Network Sharing Proposed to be changed by S1-254096 |
|  | The 6G system shall support Unified Access Control requirements as specified in [14]. | PR 5.7.6.2-1 | Unified Access Control |
|  | Subject to operators' policy, the 6G system shall provide a mechanism for the support of Femtocell deployment. | PR 5.9.1.3-1 | Femtocells Proposed to be changed by S1-254096 |
| Y.1.1-1-2 | The 6G system shall be able to support the services listed and those identified as not needing to support (i.e., CS Fallback, CS based voice call) in Clause 5.4.2. | Clause 5.4.2 | Proposed to be changed by S1-254096 |
| Y.1.1-1-3 | The 6G system shall be able to support 5G system requirements (functional and performance requirements) defined by the specifications listed in Clause 5.4.3 for 3GPP and non-3GPP accesses. These requirements do not apply to requirements involving the legacy systems/RATs (e.g. E-UTRAN, UTRAN, GERAN). | Clause 5.4.3 Clause 5.3 | Proposed to be changed by S1-254096 |
| Y.1.1-1-4 | Subject to operator’s policy, the 6G system shall support mobility procedures - between the core network (CN) of the 6G system and a 5G core network, and - between the core network of the 6G System and the evolved Packet Core Network (EPC), - between the 6G 3GPP access and non-3GPP access with minimum impact to the user experience (e.g. Quality of Service (QoS), Quality of Experience (QoE)). NOTE 1: Complexity on introducing the above interworking requirement needs to be minimized. | Clause 5.2 Clause 5.3 | Proposed to be changed by S1-254085 |
| Y.1.1-1-5 | The 6G system shall be able to provide 3GPP services to a UE using various 3GPP and non-3GPP access networks (e.g. satellite, WLAN or Wireline). | Clause 5.3 Clause 5.4.1 | Proposed to be changed by S1-254096 |
| Y.1.1-1-6 | The 6G system shall support multiple access technologies to be used simultaneously for one or more services active on the UE. | Clause 5.3 | Proposed to be changed by S1-254085 Proposed to be changed by S1-254096 |

Company Proposals:

- Y1.1.1-1-1:

  - S1-254096: (ZTE)

| Y.1.1-1-1 | The 6G system shall support new 6G services and capabilities and existing 5G services and system requirements, unless explicitly identified. requirements in [14] clause 5.1.2.2, related to inter-RAT capabilities not to be supported by 5GS, apply similarly to 6GS with the following modification: - voice service continuity from the radio access network of the 6G system to UTRAN CS shall not be supported. NOTE: Terms referring to 5G (e.g. “the 5G system”, “NG RAN”) should be implicitly replaced by the corresponding terms for 6G (e.g. “6G system”, “radio access network of the 6G system”) in these requirements | Clause 5.2 not supported IWK | The following four PRs “should be” implicitly included in Y.1.1-1 and not needed to be identified separately. [ZTE] only include 5.2 not supported PR |
| --- | --- | --- | --- |
|  | The 6G system shall have a means to ensure that the visited network provides services to an inbound roamer only once the subscriber identity (e.g. SUPI, IMSI) is obtained from the home network. | PR 5.5.10.2-1 | Roaming security [ZTE]Move to table Y.1.2 should be merged with legacy requirements |
|  | The 6G system shall support network sharing, based on 5GS network sharing requirements. | PR 5.7.4.2-1 | Network Sharing [ZTE] Merged into Y.1.1-1-3 (clause 5.4.3 legacy requirements) |
|  | The 6G system shall support Unified Access Control requirements as specified in [14]. | PR 5.7.6.2-1 | Unified Access Control [ZTE] Merged into Y.1.1-1-3(clause 5.4.3 legacy requirements) |
|  | Subject to operators' policy, the 6G system shall provide a mechanism for the support of Femtocell deployment. | PR 5.9.1.3-1 | Femtocells [ZTE] Merged into Y.1.1-1-3 (clause 5.4.3 legacy requirements) |

- Y.1.1-1-2

  - S1-254096 (ZTE)

| Y.1.1-1-2 | The 6G system does not need to support CS related telephony services (i.e., CS Fallback, CS based voice call). | Clause 5.4.2 not supported services |  |
| --- | --- | --- | --- |

- Y.1.1-1-3

  - S1-254096 (ZTE)

| Y.1.1-1-3 | The 6G system shall be able to support 5G system requirements (functional and performance requirements) defined e.g. in TS 22.261 [14], TS 22.104 [64], TS 22.011 [60], TS 22.173 [59], TS 22.071 [61], TS 22.262 [57], TS 22.185 [65], TS 22.186 [66], TS 22.125 [35], TS 22.263 [67], TS 22.115 [68], TS 22.101 [58], TS 22.153 [63], TS 22.289 [69], TS 22.468 [70], TS 22.368 [71], TS 22.156 [28], TS 22.137 [6] for 3GPP and non-3GPP accesses. NOTE: These requirements do not apply to requirements involving the legacy systems/RATs (e.g. E-UTRAN, UTRAN, GERAN). | Clause 5.4.3 Clause 5.3 PR 5.7.1.2-1 PR 5.7.1.2-2 PR 5.7.4.2-1 PR 5.7.6.2-1 PR 5.9.1.3-1 | [ZTE] non-3GPP access PRs in clause 5.3 and FWA general PRs in clause 5.7.1 could be merged into Y.1.1-1-3 |
| --- | --- | --- | --- |
|  | The 6G system shall be able to support a user to access network services via 3GPP and/or non-3GPP access (e.g. WLAN or Wireline). | Clause 5.3 | [ZTE] It could be merged into Y.1.1-1-3 |

- Y.1.1-1-4:

  - S1-254085: Charter communications, Lenovo, CableLabs

| Y.1.1-1-4 | Subject to operator’s policy, the 6G system shall support mobility procedures - between the core network (CN) of the 6G system and a 5G core network, and - between the core network of the 6G System and the evolved Packet Core Network (EPC), and - between the 6G 3GPP access and non-3GPP access with minimum impact to the user experience (e.g. Quality of Service (QoS), Quality of Experience (QoE)). NOTE 1: Complexity on introducing the above interworking requirement needs to be minimized. |
| --- | --- |

- Y.1.1-1-5

  - S1-254096 (ZTE)

|  | The 6G system shall be able to provide 3GPP services to a UE using various 3GPP and non-3GPP access networks (e.g. satellite, WLAN or Wireline). | Clause 5.3 Clause 5.4.1 | [ZTE] non-3GPP access PRs and FWA general PRs could be merged into Y.1.1-1-3 |
| --- | --- | --- | --- |
| Y.1.1-1-5 | The 6G system shall be able to support the following services: - Mission Critical Services, i.e. MCPTT, MCData, MCVideo, ref TS 22.179 [53], TS 22.280 [54], TS 22.281 [55]. and TS 22.282 [56], - Message service, ref TS 22.262 [57], - Short Message Service (SMS), ref TS 22.101 [58], - Multimedia communication services, ref TS 22.101 [58], TS 22.261 [14], TS 22.173 [59], - IMS Multimedia Telephony Service, ref TS 22.261 [14], TS 22.173 [59], - Roaming services, ref TS 22.011 [60], TS 22.101 [58], TS 22.261 [14], - Location and positioning services, ref TS 22.261 [14], TS 22.071 [61], - Broadcast and Multicast Services, ref TS 22.261 [14], - Emergency Services, ref TS 22.101 [58], - Public Warning System (PWS), ref TS 22.268 [62], - Multimedia Priority Service (MPS), ref TS 22.153 [63], - Lawful Interception, ref TS 22.261 [14] and TS 33.126 [312], and - - Other regulatory services, based on regional/national regulatory requirements. | 5.4.2 supported legacy services | [ZTE] The supported services in Clause5.4.2 need to be included. |

Editor’s Note: ZTE to clarify if the intention was to delete the original Y.1.1-1-5 and replace with (new) Y.1.1-1-5.

- Y.1.1-1-6

  - S1-254085: Charter communications, Lenovo, CableLabs

| Y.1.1-1-6 | The 6G system shall be able to select the most appropriate 3GPP or non-3GPP access technology for a service and support multiple access technologies (i.e. 3GPP and non-3GPP access) to be used simultaneously for one or more services active on a UE. | Clause 5.3 |  |
| --- | --- | --- | --- |

  - S1-254096: ZTE

| Y.1.1-1-6 | The 6G system shall support multiple access technologies to be used simultaneously for one or more services active on the UE. | Clause 5.3 | [ZTE] The original sentence is not PR |
| --- | --- | --- | --- |

* Next  Change: Table Y.1.1-2 – Enhancements to legacy services & capabilities *

Table Y.1.1-2 – Enhancements to legacy services and capabilities

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.1-2-1 | The 6G network shall provide a mechanism to support event triggered network sharing (e.g. disaster occurrence, network failure, overloaded situation, resource constraints). | PR 5.7.4.2-2 | Network Sharing |
| Y.1.1-2-2 | Subject to regulatory requirements or operator policy, the 6G network shall support sharing of radio access network with sensing capability among operators. | PR-5.7.10.6-1 | Network Sharing [QC] move to Sensing? From S1-254250 |
| Y.1.1-2-3 | The 6G system shall support suitable access categories to manage the access attempt for the new services (such as sensing, AI application, computing) in congestion scenarios. | PR 5.7.6.2-2 | Unified Access Control |
| Y.1.1-2-4 | The 6G system shall provide optimized network capabilities for FWA (e.g. support stationary devices) in relevant bands taking into consideration the regulatory requirements for each specific band. | PR 5.7.1.2-1 PR 5.7.1.2-2 | Fixed Wireless Access Proposed to be changed by S1-254096 |
| Y.1.1-2-5 | The 6G system shall enable the means to provide awareness of user service characteristics (e.g. data rate, latency) to support the RAN and CN in making real time resource allocation for FWA. | PR 5.7.1.2-3 | Fixed Wireless Access Proposed to be changed by S1-254096 |
| Y.1.1-2-6 | The 6G system shall enhance the Short Message Service to enable a network operator to verify the identity of the SMS sender and information concerning operator verified SMS sender information to the recipient of a SMS. NOTE 1: Operator-verified SMS sender information is used to inform the recipient of a SMS that the identity of the SMS sender is operator-verified and support displaying additional information (e.g. brand name, logo, etc.) of the SMS sender. Human interface aspects are out of scope of this requirement. NOTE 2: Indication that the identity of the SMS sender is operator-verified, any additional information about the SMS sender and the message itself is assumed to be integrity protected. NOTE 3: Based on interworking agreements and trust relationships, the requirements above apply also when the SMS recipient is roaming or receives a SMS from a sender served by other operators. NOTE 4: The requirements above apply to A2P SMS and may apply to Person-to-Person SMS. | PR 5.7.3.2-1 PR 5.7.3.2-2 | SMS Proposed to be changed by S1-254096 Proposed to be changed by S1-254250 |
| Y.1.1-2-7 | Subject to operator’s policy and agreement with 3rd party, the 6G network shall support a mechanism to start and stop offering certain network service(s) in a local area network adapting to the demand of e.g. the users, 3rd party or the network operator. | PR 5.9.6.6-1 | 6G LAN proposed to be moved to new Table Y.1.x-1 by S1-254191 Proposed to be changed by S1-254096 Proposed to be changed by S1-254300r1 |
| Y.1.1-2-8 | Subject to operator policies, and agreement between the PLMN operator and authorized 3rd party, the 6G network shall support a mechanism to - authorize PLMN’s users to access a subscribed service provided by an authorized 3rd party via a local area network (deployed by the PLMN operator), and - minimize service interruption when the serving network changes between the local area network and the PLMN network. | PR 5.9.6.6-2 | 6G LAN proposed to be moved to new Table Y.1.x-1 by S1-254191 Proposed to be changed by S1-254096 Proposed to be changed by S1-254300r1 |
| Y.1.1-2-9 | Subject to operator’s policy and regulation, for an operator with multiple 6G core networks, when there is UE mobility from one 6G core network to another 6G core network of the same PLMN, the 6G network shall support efficient traffic routing for the traffic from UE to data network. NOTE 1: The above term does not imply any architectural assumption, e.g. whether 6G CN is a new or evolved CN (compared to 5G). NOTE 2: This requirement only impacts core network. | PR 5.9.7.6-1 | Multiple Core Networks Proposed to be changed by S1-254096 |
| Y.1.1-2-10 | Based on operator policy, the 6G network shall support the ability to allow an authorized 3rd party service provider to provide information of the service characteristics for each traffic flow component of its service/application to the 6G network. | PR-5.9.8.2-1 | Service Awareness Proposed to be changed by S1-254096 |
| Y.1.1-2-11 | Based on operator policy, the 6G network shall support mechanisms to dynamically adjust and optimize network resources based on the service characteristics, including their predicted changes, provided by the service or application. | PR-5.9.8.2-2 | Service Awareness Proposed to be changed by S1-254096 |
| Y.1.1-2-11 | Subject to operator policy and user consent, the 6G system shall support means to provide users with differentiation of QoS and charging based on users’ digital identity information issued by a third party and users’ subscription information. | PR 5.5.9.6-1 | Digital Identity Proposed to be changed by S1-254096 |
| Y.1.1-2-12 | The multimedia telephony service [144] provided by IMS shall be able to minimise user perception of the transition during codec modification of an ongoing voice call, e.g. a codec change during communication link fluctuation. | PR 5.7.8.2-1 | IMS Codec change QoE Proposed to be changed by S1-254096 |

NEW Editor’s Note:  PRs 5.7.1.2-5, PR 5.7.2.2-1, PR 5.7.5.2-1, and PR 5.7.7.6-1 have editor’s notes associated with them, so they are not included in consolation into Table Y.1.1-2 at this time.

Company Proposals:

- Y.1.1-2-4

  - S1-254096 (ZTE)

|  | The 6G system shall provide optimized network capabilities for FWA (e.g. support stationary devices) in relevant bands taking into consideration the regulatory requirements for each specific band. | PR 5.7.1.2-1 PR 5.7.1.2-2 | Fixed Wireless Access [ZTE] merged into Y.1.1-1-3 |
| --- | --- | --- | --- |

- Y.1.1-2-5

  - S1-254096 (ZTE)

| Y.1.1-2-4 | Based on operator policy, the 6G system shall support means to aware of user service characteristics (e.g. data rate, latency, predicted changes to each traffic flow component of its service/application to the 6G network) and dynimically adjust and optimize network resource. | PR 5.7.1.2-3 PR-5.9.8.2-1 PR-5.9.8.2-2 | Service awareness including Fixed Wireless Access |
| --- | --- | --- | --- |

- Y.1.1.2-6

  - S1-254096 (ZTE)

| Y.1.1-2-5 | The 6G system shall enhance the Short Message Service to enable a network operator to verify the identity of the SMS sender and information concerning operator verified SMS sender information to the recipient of a SMS. NOTE 1: Operator-verified SMS sender information is used to inform the recipient of a SMS that the identity of the SMS sender is operator-verified and support displaying additional information (e.g. brand name, logo, etc.) of the SMS sender. Human interface aspects are out of scope of this requirement. NOTE 2: Indication that the identity of the SMS sender is operator-verified, any additional information about the SMS sender and the message itself is assumed to be integrity protected. NOTE 3: Based on interworking agreements and trust relationships, the requirements above apply also when the SMS recipient is roaming or receives a SMS from a sender served by other operators. NOTE 4: The requirements above apply to A2P SMS and may apply to Person-to-Person SMS. | PR 5.7.3.2-1 PR 5.7.3.2-2 | SMS |
| --- | --- | --- | --- |

  - S1-254250 (Qualcomm):

| Y.1.1-2-6 | The 6G system shall enhance the Short Message Service to enable a network operator to verify the identity of the SMS sender and provide operator verified SMS sender information to the recipient of a SMS. NOTE 1: Operator-verified SMS sender information is used to inform the recipient of a SMS that the identity of the SMS sender is operator-verified and support displaying additional information (e.g. brand name, logo, etc.) of the SMS sender. Human interface aspects are out of scope of this requirement. NOTE 2: Indication that the identity of the SMS sender is operator-verified, any additional information about the SMS sender and the message itself is assumed to be integrity protected. NOTE 3: Based on interworking agreements and trust relationships, the requirements above apply also when the SMS recipient is roaming or receives a SMS from a sender served by other operators. NOTE 4: The requirements above apply to A2P SMS and may apply to Person-to-Person SMS. | PR 5.7.3.2-1 PR 5.7.3.2-2 | SMS |
| --- | --- | --- | --- |

- Y.1.1.2-7

  - S1-264096 (ZTE)

| Y.1.1-2-6 | Subject to operator’s policy and agreement with 3rd party, the 6G network shall support a mechanism to start and stop offering certain network service(s) in a local area network adapting to the demand of e.g. the users, 3rd party or the network operator. | PR 5.9.6.6-1 | 6G LAN [ZTE] It is suggested to set up a new group for 6G LAN, refer S1-25XXXX |
| --- | --- | --- | --- |

  - S1-254191 (ZTE Corporation): PRs from 5.6.2, 5.9.5 and 5.9.6 are grouped in Table Y.1.x-1. (see #11 at the end of this clause)

  - S1-254250 (Qualcomm): [QC] merge with other Local NW PRs (in Verticals)

  - S1-254300 (Huawei)

| Y.1.1-2-7 | Subject to operator’s policy and agreement with 3rd party, the 6G network shall support a mechanism to start and stop offering certain network service(s) in a local area network adapting to the demand of e.g. the users, 3rd party or the network operator. | PR 5.9.6.6-1 | 6G local area network within a PLMN |
| --- | --- | --- | --- |

- Y.1.1.2-8

  - S1-254096 (ZTE)

| Y.1.1-2-7 | Subject to operator policies, and agreement between the PLMN operator and authorized 3rd party, the 6G network shall support a mechanism to - authorize PLMN’s users to access a subscribed service provided by an authorized 3rd party via a local area network (deployed by the PLMN operator), and - minimize service interruption when the serving network changes between the local area network and the PLMN network. | PR 5.9.6.6-2 | 6G LAN [ZTE] It is suggested to set up a new group for 6G LAN, refer S1-25XXXX |
| --- | --- | --- | --- |

  - S1-254191 (ZTE Corporation): PRs from 5.6.2, 5.9.5 and 5.9.6 are grouped in Table Y.1.x-1. (see #11 at the end of this clause)

  - S1-254250 (Qualcomm): [QC] merge with other Local NW PRs (in Verticals)

  - S1-254300r1 (Huawei)

| Y.1.1-2-8 | Subject to operator policies, and agreement between the PLMN operator and authorized 3rd party, the 6G network shall support a mechanism to - authorize PLMN’s users to access a subscribed service provided by an authorized 3rd party via a local area network (deployed by the PLMN operator), and - minimize service interruption when the serving network changes between the local area network and the PLMN network. | PR 5.9.6.6-2 | 6G local area network within a PLMN |
| --- | --- | --- | --- |

- Y.1.1.2-9

  - S1-254096 (ZTE)

| Y.1.1-2-8 | Subject to operator’s policy and regulation, for an operator with multiple 6G core networks, when there is UE mobility from one 6G core network to another 6G core network of the same PLMN, the 6G network shall support efficient traffic routing for the traffic from UE to data network. NOTE 1: The above term does not imply any architectural assumption, e.g. whether 6G CN is a new or evolved CN (compared to 5G). NOTE 2: This requirement only impacts core network. | PR 5.9.7.6-1 | Multiple Core Networks |
| --- | --- | --- | --- |

- Y.1.1.2.10

  - S1-254096 (ZTE)

|  | Based on operator policy, the 6G network shall support the ability to allow an authorized 3rd party service provider to provide information of the service characteristics for each traffic flow component of its service/application to the 6G network. | PR-5.9.8.2-1 | Service Awareness [ZTE] merged into Service awareness CPR |
| --- | --- | --- | --- |

- Y.1.1.2.11

  - S1-254096 (ZTE)

|  | Based on operator policy, the 6G network shall support mechanisms to dynamically adjust and optimize network resources based on the service characteristics, including their predicted changes, provided by the service or application. | PR-5.9.8.2-2 | Service Awareness [ZTE] merged into Service awareness CPR |
| --- | --- | --- | --- |

- Y.1.1.2.11 (duplicate numbering in original document)

  - S1-254096 (ZTE)

| Y.1.1-2-9 | Subject to operator policy and user consent, the 6G system shall support means to provide users with differentiation of QoS and charging based on users’ digital identity information issued by a third party and users’ subscription information. | PR 5.5.9.6-1 | Digital Identity |
| --- | --- | --- | --- |

  - S1-254250 (Qualcomm): [QC] this PR is also in the Charging section…maybe split the QoS and Charging parts?

- Y.1.1.2.12

  - S1-254096 (ZTE)

| Y.1.1-2-10 | The multimedia telephony service [144] provided by IMS shall be able to minimise user perception of the transition during codec modification of an ongoing voice call, e.g. a codec change during communication link fluctuation. | PR 5.7.8.2-1 | IMS Codec change QoE |
| --- | --- | --- | --- |

- From S1-254191 (ZTE Corporation): PRs from 5.6.2, 5.9.5 and 5.9.6 are grouped in Table Y.1.x-1.

### Y.1.x	Localized network

Editor’s Note: This clause contains consolidated potential requirements for 6G that address localized network. This includes (but not limited to) PRs from clauses 5.6, 5.9 and 11.

Table Y.1.x-1 – Localized network

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.x-1-1 | Subject to operator policies and service level agreements, the 6G system shall enable operators to provision network services as part of the operator’s PLMN network on-demand, e.g. in response to an urgent event (e.g. disaster, emergency and DDoS events), with certain level of local control and specific functionalities in a given area during a specific time period. NOTE 1: The level of local control can be based on operator policies and agreements with 3rd party. For example, the authorization and policy control of users to access the provisioned services are not affected by the failure of the operator’s PLMN network. NOTE 2: The enabled functionalities can be based on operator policies and agreements with 3rd party. For example, data connectivity service and voice service are prioritized when an urgent event happens in a residential community; small data transfer service is prioritized when an urgent event happens in an IoT based farmland. NOTE 3: Some situations can target the required network services to be provisioned within hours to serve certain users whose QoE is impacted by an urgent event. NOTE 4: Local control refers to the capability of part of the operator’s PLMN network to operate autonomously and independently, e.g. management of local subscription, local traffic, without interaction with the operator’s PLMN. | PR 5.6.2.6-1 | provision network services on-demand |
| Y.1.x-1-2 | Subject to operator policies and service level agreements, the 6G system shall enable a network operator to authorize a UE, that is subscribed to local network services, to access services from the PLMN of the same operator. NOTE 5: This applies to scenarios where a service is not available in the local network services that have been provisioned on-demand, but is available from the PLMN of the same operator. | PR 5.6.2.6-2 | authorize a local UE accessing service in PLMN |
| Y.1.x-1-3 | The 6G system shall support on-demand rollout (e.g. within hours) of new or updated services/capabilities with minimal disruption to existing services, including the ability to efficiently rollback those services/capabilities, as needed (e.g. in case of failures or demand from other services). | PR 5.9.5.6-1 | on-demand rollout service/capability |
| Y.1.x-1-4 | The 6G network shall provide means to minimise the impact to the user experience during the rollout and rollback (if needed) of new and updated services/capabilities. | PR 5.9.5.6-2 | minimise impact to user experience |
| Y.1.x-1-5 | Subject to operator’s policy and agreement with 3rd party, the 6G network shall support a mechanism to start and stop offering certain network service(s) in a local area network adapting to the demand of e.g. the users, 3rd party or the network operator. | PR 5.9.6.6-1 | start and stop service in local network |
| Y.1.x-1-6 | Subject to operator policies, and agreement between the PLMN operator and authorized 3rd party, the 6G network shall support a mechanism to - authorize PLMN’s users to access a subscribed service provided by an authorized 3rd party via a local area network (deployed by the PLMN operator) - minimize service interruption when the serving network changes between the local area network and the PLMN network | PR 5.9.6.6-2 | authorize a PLMN UE accessing service in local network， service continuity between PLMN and local network |

Table Y.1.x-2 – Localized network in vertical

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.14-2-1 | Subject to an agreement between a PLMN operator and authorized 3rd party, operator policies and regulatory requirements, the 6G network shall support a mechanism to authorize UEs owned by a trusted authorized 3rd party (e.g. vertical industry) to access services provided by a localized network (deployed by the PLMN operator for the authorized 3rd party). | PR 11.10.6-1 | Authorization |
| CPR Y.1.14-2-2 | Subject to an agreement between a PLMN operator and authorized 3rd party, operator policies and regulatory requirements, the 6G network shall support a mechanism to authorize and authenticate UEs owned by a trusted authorized 3rd party to access services from the PLMN operator. NOTE: The realization or deployment of localized network may be an enhancement of NPN, PALS or other network topology options. | PR 11.10.6-2 | Authorization Comment: access services from the PLMN operator is not clear. The service is from PLMN or PLMN operator |
| CPR Y.1.14-2-3 | Subject to regulatory requirements and the operators’ policy, the 6G system shall provide the possibility of local authorization for a new device or UE joining the local network. | PR 11.11.6-3 | Authorization |
| CPR Y.1.14-2-4 | The 6G network shall support a mechanism to allow the operator or authorized 3rd party to perform local control of the localized network when the localized network loses connection with the PLMN network. | PR 11.10.6-3 | Local control |
| CPR Y.1.14-2-5 | Subject to regulatory requirements and the operators’ policy, 6G network shall allow the operator to provide the required control and configuration information to support (partial-)autonomous local networks for the communication between UEs and devices out of coverage of the 6G network. NOTE: (Partial-)autonomous means that the management of local network can be done locally even in case of loss of connectivity to the mobile network. | PR 11.11.6-1 PR 11.11.6-2 | Local control |
| CPR Y.1.14-2-5 | Subject to regulatory requirements and the operators’ policy, 6G system shall be able to support (partial-)autonomous local networks for the communication between UEs and devices. NOTE: (Partial-)autonomous means that the management of local network can be done locally even in case of loss of connectivity to the mobile network. | PR 11.11.6-1 | Local control |
| CPR Y.1.14-2-6 | Subject to regulatory requirements and operator policy, the 6G network shall allow the operator to provide the required control information including configurations to the local network that enables it to provide necessary configurations for the local communication and to operate also out of coverage of the mobile network. | PR 11.11.6-2 | Local control |
| CPR Y.1.14-2-7 | Subject to operator’s policy, the 6G network shall support means to enable communication between UEs and a service hosting environment in a local 6G network, managed by an authorized 3rd party, when the connection with the remote (non-local) network fails. NOTE 1: The local 6G network is composed of at least 6G RAN and core network of 6G which are deployed in the local area e.g. factory. NOTE 2: The above assumes agreement with the 3rd party and control by the core network of 6G system | PR 11.23.6-1 | Local control |
| CPR Y.1.14-2-8 | The 6G system shall support a mechanism to allow the UE to discover and select the services provided by a localized network. | PR 11.10.6-4 | Service discover and select |

* * * Next Change: Table Y.1.1-3 – Device Support* * *

Table Y.1.1-3 – Device Support

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.1-3-1 | The 6G system shall support UEs with different characteristics such as data rate, latency, etc. | PR 5.10.1.3-1 | Editor's Note: additional considerations on the above requirement is FFS. |
| Y.1.1-3-2 | The 6G system with satellite access shall support UEs with the performance requirements in Table 5.10.2.2-1 | PR 5.10.2.2-1 | Table values are FFS [QC] should be part of SAT KPIs (from S1-254250) |

NEW Editor’s Note:  Both PRs for Table Y.1.1-3 have editor’s notes associated with them, but neither of the ENs are against the actual text of the PR. PR 5.10.1.3-1’s EN is about “additional considerations” and PR 5.10.2.2-1’s EN is against the associated Table values.  They are provided for discussion/consideration.

* * * Next Change: Table Y.1.2-1 – Security and Privacy * * *

### Y.1.2	Security and Privacy

Editor’s Note: This clause contains consolidated potential requirements for 6G that include security, privacy and lawful interception. This includes (but not limited to) PRs from clauses 5.5 and X.1.

NEW Editor’s Note:  PR 5.5.1.2-5, PR 5.5.4.2-10, and PR 5.5.6.3-1 have editor’s notes associated with them, so they are not included in consolation into Table Y.1.2-1 at this time.

Table Y.1.2-1 – Security and Privacy

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.2-1-1 | The 6G system shall provide FWA security and privacy protection comparable to other 6G services. | PR 5.7.1.2-4 | Fixed Wireless Access Proposed to be changed by S1-254250 |
| Y.1.2-1-2 | The 6G system shall be able to ensure subscriber identity privacy. | PR 5.5.7.3-3 PR 5.5.2.6-1 | UE Identity protection Quantum Resistant Security [QC] the Quantum part can be covered by Y.1.2-1-5 Proposed to be changed by S1-254250 Blue highlighted changes from S1-254250r1 |
| Y.1.2-1-3 | The 6G network shall provide security mechanisms for secure access to elements of the core network of the 6G system and enable the network operator to ensure there are no unintended changes of the elements of the 6G network | PR 5.5.1.2-1 PR 5.5.1.2-3 | Network Security |
| Y.1.2-1-4 | The 6G network shall support establishment of secure communication between elements of the network and secure communication on all 3GPP defined interfaces of the core network of the 6G system while protecting network related information (e.g. network element identities, topology) from disclosure to unauthorized parties. | PR 5.5.1.2-1 PR 5.5.1.2-2 | Network Security |
| Y.1.2-1-5 | The 6G system shall provide security protection for communication (e.g. authentication methods) against the potential attacks posed by quantum computing. | PR 5.5.2.6-1 | Quantum Resistant Security |
| Y.1.2-1-6 | The 6G system shall ensure cryptography agility (e.g. post-quantum cryptography algorithms smooth migration, switching, update) in order to provide security against new threats. | PR 5.5.2.6-3 | Quantum Resistant Security Proposed to be changed by S1-254250 Blue highlighted changes from S1-254250r1 |
| Y.1.2-1-7 | Subject to operator policy and regulatory requirements, the 6G system shall support a means for a UE to be able to distinguish a False Base Station from an authentic Base Station. | PR 5.5.3.2-1 | System Security False Base Station |
| Y.1.2-1-8 | The 6G system shall provide mechanisms to support authentication and secure communication in inter-PLMN and intra-PLMN networks. | PR 5.5.4.2-1 | System Security Network Security Blue highlighted changes from S1-254250r1 |
| Y.1.2-1-9 | The 6G system shall support flexible security capabilities to meet security needs of different intra-PLMN networks. | PR 5.5.4.2-2 | System Security Network Security Blue highlighted changes from S1-254250r1 |
| Y.1.2-1-10 | The 6G system shall be able to provide flexible security capabilities to meet customized security requirements from different PLMN users. | PR 5.5.4.2-3 | System Security Network Security Security Management Blue highlighted changes from S1-254250r1 |
| Y.1.2-1-11 | The 6G network shall support security management capabilities that enable - the identification of potential security threats to communication and network elements, - security analysis and security enhancement policy generation to mitigate attacks and/or security issues in operator management system, - security response to implement enhanced security policy from management system to network elements, in order to recover network from disturbance | PR 5.5.4.2-4 PR 5.5.4.2-5 PR 5.5.4.2-6 | Security Management |
| Y.1.2-1-12 | The 6G system shall enable operators using SIM as subscription credential to provide digital identity and authentication service to authorized 3rd parties or users. | PR 5.5.4.2-7 | Identity Authorization Service Proposed to be changed by S1-254250 [QC] any better term than “trust anchor”? Blue highlighted changes from S1-254250r1 |
|  |  |  | Blue highlighted changes from S1-254250r1 [QC[ We think #12 is good enough. Rest is very solution specific |
| Y.1.2-1-14 | The 6G system shall support security and privacy protection of the 6G System Data. | PR 5.9.2.2-4 | 6G System Data System Security |
| Y.1.2-1-15 | Subject to operator policy and regulatory requirements, the 6G system shall support privacy protection, location privacy, identity protection for UEs accessing 6G network for services (e.g. communication, sensing, AI inferencing), and UE information exposure to an authorized 3rd party. | PR 5.5.7.3-1 PR 5.5.7.3-2 | Security & Privacy for services, data & exposure Proposed to be changed by S1-254250 Blue highlighted changes from S1-254250r1 |
| Y.1.2-1-16 | Subject to regulation, operator policy and user consent, the 6G system shall support mechanisms to protect data privacy during the processing of sensing data. | PR 7.12.6-2 | Data Privacy Sensing Data Processing [QC] move to Sensing? (from by S1-254250) by S1-254250 Blue highlighted changes from S1-254250r1 |
| Y.1.2-1-17 | Subject to regulation, operator(s) policy and user consent, the 6G network shall support access from authorized third parties to processed data related to UEs served by the network (for example but not limited to number of UEs in a geographical location, their mobility pattern, application usage trends) without exposing UE identities and individual user data including Personally Identifiable Information or sensitive data. NOTE 1: Processed data refers to the analysis of data to produce new data including among other statistics on the data, correlating data, aggregating data. NOTE 2: The existing security, privacy policies and procedures for collecting and processing user data (e.g. via anonymisation) will be preserved. | PR 5.5.5.3-1 | Security & Privacy for information exposure Proposed to be moved here from Y.1.5.1-3 by S1-254166 |

* * * Next Change: Table Y.1.2-2– Lawful Interception* * *

Table Y.1.2-2– Lawful Interception

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.2-2-1 | The 6G system shall be able to support Lawful Interception requirements ([14], [312]) and other regulatory services, based on regional/national regulatory requirements. | Clause 5.4.2 | Could be considered as part of Table Y.1.1-1 [QC] ok to keep all LI CPR together Proposed to be changed by S1-254250 |
| Y.1.2-2-2 | The 6G network shall provide security mechanisms that offer protection of communication and data while at the same time not hindering Lawful Interception ( [312], clause 6.4). | PR 5.5.1.2-4 | Could be considered as part of Table 1.2.-1 [QC] ok to keep all LI CPR together Proposed to be changed by S1-254250 |
| Y.1.2-2-3 | The 6G system shall support the mapping of permanent and temporary identifiers of all subscribers, e.g. for Lawful Interception purposes. | PR 5.5.2.6-2 | Identifiers |
| Y.1.2-2-4 | Subject to national or regional regulatory requirements, the 6G system shall provide a privacy override mechanism to enable access to retained data for law enforcement purposes and Lawful Interception purposes which may otherwise be subject to the privacy protection requirements [313]. | PR 5.5.7.3-4 | Privacy override |
|  |  |  | Proposed to be deleted by S1-254250 [QC] Other PRs refer to subject to “regulatory”, so it’s implicit that requlatory reqs are met |
|  |  |  | Proposed to be deleted by S2-254250 [QC] no need to make this a requirement (can be part of some intro text, to be discussed later) |
| Y.1.2-2-7 | The 6G system shall support the retention of data to meet national or regional regulatory requirements, [314], [315], [313], and [316]. | Clause X.1 | Data retention Could be considered as part of Table Y.1.5-1 – Data Collection and Consumption From S1-254250: [QC] agree with the suggestion |

* * * Next Change: Table Y.1.3-1 – Resilience * * *

### Y.1.3	Resilience

Editor’s Note: This clause contains consolidated potential requirements for 6G that address resiliency. This includes (but not limited to) PRs from clauses 5.6 and 5.9.

Table Y.1.3-1 – Resilience

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.3-1-1 | Subject to regulatory requirements and operator’s policy, 6G network should support suitable means to ensure the stability of services provided by the edge network in case of abnormal transmission occurring between the public network and the edge network. | PR 5.6.1.2-2 | Edge resilience From S1-254250 [QC] clarify “stablity”…change to resilience? |
| Y.1.3-1-2 | Subject to operator policies and service level agreements, the 6G system shall support on-demand rollout (e.g. within hours) of new or updated services/capabilities - with certain level of local control and specific functionalities in a given area during a specific time period, - with minimal disruption to existing services, including the ability to efficiently rollback those services/capabilities, as needed (e.g. in case of failures or demand from other services), while minimizing the impact to the user experience - without reducing service quality NOTE 1: The level of local control can be based on operator policies and agreements with 3rd party. For example, the authorization and policy control of users to access the provisioned services are not affected by the failure of the operator’s PLMN network. NOTE 2: The enabled functionalities can be based on operator policies and agreements with 3rd party. For example, data connectivity service and voice service are prioritized when an urgent event happens in a residential community; small data transfer service is prioritized when an urgent event happens in an IoT based farmland. NOTE 3: Some situations can target the required network services to be provisioned within hours to serve certain users whose QoE is impacted by an urgent event. NOTE 4: Local control refers to the capability of part of the operator’s PLMN network to operate autonomously and independently, e.g. management of local subscription, local traffic, without interaction with the operator’s PLMN. | PR 5.6.2.6-1 PR 5.9.4.2-1 PR 5.9.5.6-1 PR 5.9.5.6-2 | On-demand/quick rollout for new/updated services PR 5.6.2.6-1, PR 5.9.5.6-1 and -2 are proposed to be moved to new Table Y.1.x-1 by S1-254191 Proposed to be changed by S1-254092 Proposed to be changed by S1-254250 Proposed to be changed by S1-4300r1 |
| Y.1.3-1-3 | Subject to operator policies and service level agreements, the 6G system shall enable a network operator to authorize a UE, that is subscribed to local network services, to access services from the PLMN of the same operator. NOTE 5: This applies to scenarios where a service is not available in the local network services that have been provisioned on-demand but is available from the PLMN of the same operator. | PR 5.6.2.6-2 | Same operator/cross PLMN service Proposed to be moved to new Table Y.1.x-1 by S1-254191 [QC] to be merged with other local NW PRs (in Verticals) from S1-254250 |
| Y.1.3-1-4 | Subject to operator policy and regulatory requirements, the 6G system shall be able to minimize service interruption or degradation during disasters taking into account the affected/not affected parts of a single or multiple network(s) serving an area impacted by a specific type of disaster. NOTE: In case of multiple networks serving the disaster area, it is assumed that under disaster conditions, the remaining available parts of the networks in that area can serve the UEs of other networks. | PR 5.6.4.6-1 |  |
| Y.1.3-1-5 | The 6G network shall provide a mechanism to minimize the signalling arising from recovery of network failure. NOTE: Massive signalling can happen e.g. when a network element fails and registrations or data sessions are re-routed to a backup network element. | PR 5.6.5.6-1 | Recovery |

Company Proposals:

- Y.1.3-1-2:

  - S1-254092 (China Telecom)

| Y.1.3-1-2 | Subject to operator policies and service level agreements, the 6G system shall support on-demand rollout (e.g. within hours) of new or updated services/capabilities - with minimal disruption to existing services, including the ability to efficiently rollback those services/capabilities, as needed (e.g. in case of failures or demand from other services), while minimizing the impact to the user experience | PR 5.9.5.6-1 PR 5.9.5.6-2 | On-demand/quick rollout for new/updated services |
| --- | --- | --- | --- |
| Y.1.3-1-x | Subject to operator policies and service level agreements, the 6G system shall enable operators to provision network services as part of the operator’s PLMN network on-demand, e.g. in response to an urgent event (e.g. disaster, emergency and DDoS events), with certain level of local control and specific functionalities in a given area during a specific time period. NOTE 1: The level of local control can be based on operator policies and agreements with 3rd party. For example, the authorization and policy control of users to access the provisioned services are not affected by the failure of the operator’s PLMN network. NOTE 2: The enabled functionalities can be based on operator policies and agreements with 3rd party. For example, data connectivity service and voice service are prioritized when an urgent event happens in a residential community; small data transfer service is prioritized when an urgent event happens in an IoT based farmland. NOTE 3: Some situations can target the required network services to be provisioned within hours to serve certain users whose QoE is impacted by an urgent event. NOTE 4: Local control refers to the capability of part of the operator’s PLMN network to operate autonomously and independently, e.g. management of local subscription, local traffic, without interaction with the operator’s PLMN. | PR 5.6.2.6-1 | Fast network provisioning to improve resilience |

  - S1-254250 (Qualcomm)

| Y.1.3-1-2 | Subject to operator policies and service level agreements, the 6G system shall support on-demand rollout (e.g. within hours) of new or updated services/capabilities - with minimal disruption to existing services, including the ability to efficiently rollback those services/capabilities, as needed (e.g. in case of failures or demand from other services), while minimizing the impact to the user experience - without reducing service quality NOTE 2: The enabled functionalities can be based on operator policies and agreements with 3rd party. For example, data connectivity service and voice service are prioritized when an urgent event happens. NOTE 3: Some situations can target the required network services to be provisioned within hours to serve certain users whose QoE is impacted by an urgent event. | PR 5.6.2.6-1 PR 5.9.4.2-1 PR 5.9.5.6-1 PR 5.9.5.6-2 | On-demand/quick rollout for new/updated services [local control should be merged with other Local NW PRs (in Verticals) |
| --- | --- | --- | --- |

  - S1-254300r1 (Huawei)

| Y.1.3-1-2 | Subject to operator policies and service level agreements, the 6G system shall support on-demand rollout (e.g. within hours) of new or updated services/capabilities - with minimal disruption to existing services, including the ability to efficiently rollback those services/capabilities, as needed (e.g. in case of failures or demand from other services), while minimizing the impact to the user experience | PR 5.9.5.6-1 PR 5.9.5.6-2 | On-demand/quick rollout for new/updated services |
| --- | --- | --- | --- |
| Y.1.3-1-x1 | Subject to operator policies and service level agreements, the 6G system shall enable operators to provision network services as part of the operator’s PLMN network on-demand, e.g. in response to an urgent event (e.g. disaster, emergency and DDoS events), with certain level of local control and specific functionalities in a given area during a specific time period. NOTE 1: The level of local control can be based on operator policies and agreements with 3rd party. For example, the authorization and policy control of users to access the provisioned services are not affected by the failure of the operator’s PLMN network. NOTE 2: The enabled functionalities can be based on operator policies and agreements with 3rd party. For example, data connectivity service and voice service are prioritized when an urgent event happens in a residential community; small data transfer service is prioritized when an urgent event happens in an IoT based farmland. NOTE 3: Some situations can target the required network services to be provisioned within hours to serve certain users whose QoE is impacted by an urgent event. NOTE 4: Local control refers to the capability of part of the operator’s PLMN network to operate autonomously and independently, e.g. management of local subscription, local traffic, without interaction with the operator’s PLMN. | PR 5.6.2.6-1 | Fast network provisioning to improve resilience |
| Y.1.3-1-x2 | The 6G system shall be able to support means to simplify network operation and service delivery compared to 5G system, e.g. in order to reduce signalling, minimise connection setup time. NOTE: This simplification should not mean reducing service quality provided. | PR 5.9.4.2-1 | simplify network operation and service delivery |

- Y.1.3-1-5

  - S1-254159 (NTT DOCOMO)

| Y.1.3-1-5 | The 6G network shall provide a mechanism to minimize connection setup time and the signalling arising from recovery of network failure compared to the 5G System. NOTE 1: Massive signalling can happen e.g. when a network element fails and registrations or data sessions are re-routed to a backup network element. NOTE 2: The mechanism to minimize should not mean reducing service quality provided. | PR 5.6.5.6-1 PR 5.9.4.2-1 | Recovery and simplification |
| --- | --- | --- | --- |

* * * Next Change: Table Y.1.4-1 – Energy-related aspects * * *

### Y.1.4	Energy-related aspects

Editor’s Note: This clause contains consolidated potential requirements for 6G that address energy efficiency and sustainability. This includes (but not limited to) PRs from clause 5.8.

Proposed Comments/Changes:

S1-254160 (ZTE):  Modify the EN above to:

Editor’s Note: This clause contains consolidated potential requirements for 6G that address energy efficiency and sustainability. This includes (but not limited to) PRs from clauses 5.8, 6 and 7.

NEW Editor’s Note:  PR 5.8.2.6-2 has an editor’s note associated with it (concerning energy consumption information), so it is not included in consolation into Table Y.1.4-1 at this time.

Nokia comment (via email reflector on 10 Nov): With respect to energy-related aspects we’d like to suggest to split CPR Y.1.4-1-2: do not merge PR 5.8.4.6-2 and PR 5.8.5.6-1, and instead only merge these 2 separately (1 focused on user preference, the other on the optimization of UE energy consumption). We believe merging 2+2 instead of 4 in this case will ease readability & interpretation of the requirements.

See Y.1.4-1-2 below for proposed changes from Nokia.

ZTE response to Nokia comments (via email reflector on 10 Nov): ZTE has submitted a proposal on the CPRs of EE (S1-254060).  Regarding the PR5.8.1.6-3, I agree with you that it is targeting the user's preference for UE's EE and not merge it into original CPRY.1.4-1-2 is better.  But for PR5.8.4.6-2, PR5.8.5.6-1, and PR5.8.1.6-2 which are for improving UE EE.  So, I think merging PR5.8.1.6-2 with PR5.8.1.6-3 is not good.

S1-254250 (Qualcomm):  [QC]: comments will be provided later

Table Y.1.4-1 – Energy-related aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.4-1-1 | The 6G system shall improve energy efficiency of 6G system compared to 5G system. | PR 5.8.1.6-1 | System Energy Efficiency |
| Y.1.4-1-2 | The 6G system shall support mechanisms to improve UE energy efficiency when providing services to the subscriber, optimizing the UE energy saving considering the current traffic conditions of the UE (subject to local regulations and user consent), enabling task offloading from the UE to the Service Hosting Environment, considering battery life and thermal issue of UE, and provide means for a user to provide a preference for UE energy efficiency. NOTE: The 6G network uses this preference when improving UE energy efficiency. Examples of preference include: - user preference on prioritizing service performance, - prioritizing UE energy efficiency, or - prioritizing essential service (e.g. voice call) while improving UE energy efficiency. | PR 5.8.1.6-2 PR 5.8.1.6-3 PR 5.8.4.6-2 PR 5.8.5.6-1 | UE Energy Efficiency Proposed to be modified by S1-254160 Proposed to be modified by Nokia email reflector comments and in subsequent email reflector discussions |
| Y.1.4-1-3 | The 6G network with multiple access networks (e.g. terrestrial, satellite) shall provide means to determine (e.g. through measurement and/or calculation) the energy consumed by the access networks for the provision of a one-to-many service (e.g. PWS [62], broadcast, multicast) characterised by at least the QoS, targeted service area and/or targeted set of users. | PR 5.8.2.6-1 | Access Energy Consumption Proposed to be modified by S1-254160 |
| Y.1.4-1-4 | Subject to operator’s policy, the 6G system shall support network slice-specific energy management policies that - define a maximum slice energy credit limit for services without QoS criteria, - support a mechanism to perform slice energy credit limit control for services without QoS criteria. NOTE: The result of the credit control is not specified by this requirement. | PR 5.8.3.6-1 PR 5.8.3.6-2 | Slice Energy Management Proposed to be modified by S1-254160 |
| Y.1.4-1-5 | Subject to local regulation and user consent, the 6G system shall be able to optimize network and UE energy saving jointly, while meeting the service performance requirements. | PR 5.8.4.6-3 | System Energy Saving Proposed to be modified by S1-254160 |
| Y.1.4-1-6 | Subject to operator’s policy, regulation and user’s consent, the 6G network shall provide suitable energy saving methods targeting different scenarios (e.g. different working time) and optimize the network energy saving considering the current traffic conditions of the network (subject to local regulation). | PR 5.8.6.6-1 PR 5.8.4.6-1 | Network Energy Saving Proposed to be modified by S1-254160 |
| Y.1.4-1-7 | Subject to regulation and operator’s policy, the 6G network shall be able to expose to a trusted third-party the network energy consumption information including the energy consumption related to sensing, AI, and computing services, over a specific time period (e.g. month etc.). | PR 5.8.6.6-2 | Network Energy Consumption Proposed to be modified by S1-254160 |
| Y.1.4-1-8 | Subject to user consent and operator policy, the 6G system shall support a means for a group of cooperating UEs to reduce the energy consumption for communication of the group of UEs whilst meeting requested service performance. | PR 5.8.8.6-1 | Cooperating UEs Energy Consumption Proposed to be modified by S1-254160 |
| Y.1.4-1-9 | The 6G system should support energy-efficient sensing operations. | PR 7.5.6-2 | Energy Efficient Sensing Proposed to be modified by S1-254160 |

Company Proposals:

- Y1.4-1-2:

  - S1-254160: (ZTE)

| Y.1.4-1-2 |  | The 6G system shall support mechanisms to improve UE energy efficiency when providing services to the subscriber, optimizing the UE energy saving considering the current traffic conditions of the UE (subject to local regulations and user consent), enabling task offloading from the UE to the Service Hosting Environment, considering battery life and thermal issue of UE. | PR 5.8.1.6-2 PR 5.8.4.6-2 PR 5.8.5.6-1 | UE Energy Efficiency [ZTE] move PR5.8.1.6-3 to seperate CPR |
| --- | --- | --- | --- | --- |
| Y.1.4-1-3 (new) | The 6G system shall provide means for a user to provide a preference for UE energy efficiency. NOTE: Examples of preference are user preference on prioritizing service performance, prioritizing UE energy efficiency, or prioritizing essential service (e.g. voice call) while improving UE energy efficiency. | PR 5.8.1.6-3 | [ZTE] it is for user’s preference, better to seperate. |

  - Nokia (10 Nov reflector comments)

| Y.1.4-1-2 | The 6G system shall be able to support mechanisms (including a mechanism to consider user preference) to improve UE energy efficiency when providing services to subscribers. Examples of such preferences including - prioritizing service performance, - prioritizing UE energy efficiency, or - prioritizing essential service (e.g. voice call) while improving UE energy efficiency. | PR 5.8.1.6-2 PR 5.8.1.6-3 | UE Energy Efficiency |
| --- | --- | --- | --- |
| Y.1.4-1-3 (new) | Subject to regulatory requirements and user consent, the 6G system shall optimize UE energy consumption by considering the UE's current traffic conditions and by enabling task offloading (e.g., XR rendering/AI inference) from the UE to the Service Hosting Environment, considering the UE's battery life and thermal issues. | PR 5.8.4.6-2 PR 5.8.5.6-1 | UE Energy Efficiency |

  - Last reflector comments (13 Nov)

| Y.1.4-1-2 | The 6G system shall support mechanisms to improve UE energy efficiency and saving when providing services to the subscriber, at least considering the current traffic conditions of the UE (subject to local regulations and user consent), by enabling task offloading from the UE to the Service Hosting Environment, also based on battery life and thermal issue of UE. | PR 5.8.1.6-2 PR 5.8.4.6-2 PR 5.8.5.6-1 | UE Energy Efficiency [ZTE] move PR5.8.1.6-3 to seperated CPR Nokia: rewording to: - Factorise efficiency & saving - use “at least by” instead of “ie” (exact list) or “eg” (examples only) - change “considering” to “also based on” (also removing the comma) to avoid another verb in -ing which may be confused with another “by”, and better link it with the offloading only |
| --- | --- | --- | --- |
| Y.1.4-1-3 (new) | The 6G system shall provide means for a user to provide a preference for UE energy efficiency. NOTE: Examples of preference are user preference on prioritizing service performance, prioritizing UE energy efficiency, or prioritizing essential service (e.g. voice call) while improving UE energy efficiency. | PR 5.8.1.6-3 | [ZTE] it is for user’s preference. UE Energy Efficiency Preference |

- Y1.4-1-3:

  - S1-254160: (ZTE)

| Y.1.4-1-4 | The 6G network with multiple access networks (e.g. terrestrial, satellite) shall provide means to determine (e.g. through measurement and/or calculation) the energy consumed by the access networks for the provision of a one-to-many service (e.g. PWS [62], broadcast, multicast) characterised by at least the QoS, targeted service area and/or targeted set of users. | PR 5.8.2.6-1 | Access Energy Consumption |
| --- | --- | --- | --- |

- Y1.4-1-4:

  - S1-254160: (ZTE)

| Y.1.4-1-5 | Subject to operator’s policy, the 6G system shall support network slice-specific energy management policies that - define a maximum slice energy credit limit for services without QoS criteria, - support a mechanism to perform slice energy credit limit control for services without QoS criteria. NOTE: The result of the credit control is not specified by this requirement. | PR 5.8.3.6-1 PR 5.8.3.6-2 | Slice Energy Management |
| --- | --- | --- | --- |

- Y1.4-1-5:

  - S1-254160: (ZTE)

| Y.1.4-1-6 | Subject to local regulation and user consent, the 6G system shall be able to optimize network and UE energy saving jointly, while meeting the service performance requirements. | PR 5.8.4.6-3 | System Energy Saving |
| --- | --- | --- | --- |

- Y1.4-1-6:

  - S1-254160: (ZTE)

| Y.1.4-1-7 | Subject to operator’s policy, regulation and user’s consent, the 6G network shall provide suitable energy saving methods targeting different scenarios (e.g. different working time) and/or the current traffic conditions of the network (subject to local regulation). | PR 5.8.6.6-1 PR 5.8.4.6-1 | Network Energy Saving |
| --- | --- | --- | --- |

- Y1.4-1-7:

  - S1-254160: (ZTE)

| Y.1.4-1-8 | Subject to regulation and operator’s policy, the 6G network shall be able to monitor and expose to a trusted third-party the network energy consumption information including the energy consumption related to sensing, AI, and computing services, over a specific time period (e.g. month etc.). | PR 5.8.6.6-2 [PR 6.37.6-1] [PR 6.37.6-2] | Network (Sensing, AI, computiner) related Energy Consumption |
| --- | --- | --- | --- |
|  | Based on operator's policy and agreement with 3rd party, the 6G network shall support monitoring energy consumption for an AI service (e.g. inference) requested by 3rd party. | [PR 6.37.6-1] | [ZTE] move from AI section |
|  | Based on operator's policy and agreement with 3rd party, the 6G network shall support exposing energy consumption information of an AI service to 3rd party. | [PR 6.37.6-2] | [ZTE] move from AI section |

- Y1.4-1-8:

  - S1-254160: (ZTE)

| Y.1.4-1-9 | Subject to user consent and operator policy, the 6G system shall support a means for a group of cooperating UEs to reduce the energy consumption for communication of the group of UEs whilst meeting requested service performance. | PR 5.8.8.6-1 | Cooperating UEs Energy Consumption |
| --- | --- | --- | --- |

- Y1.4-1-9:

  - S1-254160: (ZTE)

| Y.1.4-1-10 | The 6G system should support energy-efficient sensing operations. | PR 7.5.6-2 | Energy Efficient Sensing |
| --- | --- | --- | --- |

- Move into this table:

  - S1-254160: (ZTE)

| Y.1.4-1-11 | According to the use case of 6.24, revise the “energy related data” to “energy related information”: According to the use case of 6.24, revise the “energy related data” to “energy related information”: The 6G network shall be able to collect energy related information of the Service Hosting Environment, from energy management system, which is outside of 3GPP networks. PR 6.24.6-1 [ZTE] SHE EE, move from AI section | PR 6.24.6-1 | [ZTE] SHE EE, move from AI section |
| --- | --- | --- | --- |
| Y.1.4-1-12 | The 6G network shall provide suitable means for communicating with the NDT of operator controlled compute resources that are in the Service Hosting Environment or the cloud, for the purpose of energy consumption analysis and optimisation. NOTE: it only applies if the 6G network supports a Network Digital Twin | PR 5.8.7.6-1 | [ZTE] NDT for EE |
| Y.1.4-1-13 | The 6G network shall support the use of AI for the OAM of the 6G network, to assist with its energy efficiency and carbon emissions reduction. | PR 6.16.6-1 | [ZTE] AI in OAM for EE |
| Y.1.4-1-14 | According to the use case of 6.24, revise it as: The 6G network shall be able to support mechanisms to effectively use the supplied energy from the energy supplier, considering energy related information from energy management system, which is outside of 3GPP networks. | PR 6.24.6-3 | [ZTE]: not sure what is the exactly impact on 6G network |
|  | The 6G network shall be able to consider energy related information from energy management system, which is outside of 3GPP networks, to support mechanisms in order to effectively use the energy, e.g. efficiently use the supplied energy from the energy supplier. | PR 6.24.6-3 |  |

* * * Next Change: Table Y.1.5-1 – Data Collection and Consumption * * *

### Y.1.5	Data Collection and Consumption

Editor’s Note: This clause contains consolidated potential requirements for 6G that address data collection, management, and consumption. This includes (but not limited to) PRs from clauses 5.9.2 and W.

NEW Editor’s Note:  PR 5.9.2.2-2, -3, and -6 have editor’s notes associated with them (concerning user consent), so they are not included in consolation into Table Y.1.5-1 at this time.

Table Y.1.5-1 – Data Collection and Consumption

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.5-1-1 | The 6G system shall support mechanisms for 6G System Data collection and consumption minimizing the impact to 6G services. | PR 5.9.2.2-1 |  |
| Y.1.5-1-2 | Subject to regulation and operator policy, the 6G system shall support transfer 6G System Data between different data providers and data consumers within the 6G system. | PR 5.9.2.2-7 |  |
|  |  |  | Proposed to be moved to Table Y.1.2-1 – Security and Privacy by S1-254166 |

Company Proposals:

- Y.1.5.1-x:

  - S1-254131 (NEC): add the following to Table Y.1.5-1

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.5-1-x | Subject to operator’s policy, regulation and user consent, the 6G system shall support secure means to expose 6G System Data and derived/processed information to authorized trusted third-party, authorized network function or authorized UE. | PR 5.9.2.2-3 PR W.4.6-1 | Proposed merged CPR, data exposure |
|  | Subject to user consent, regulation and operator's policy, the 6G system shall support secure means to expose 6G System Data to authorized trusted third-party, authorized network function or authorized UE. | PR 5.9.2.2-3 | Data exposure |
|  | Subject to regulation and operator(s) policy, the 6G network shall support a mechanism to expose information derived/processed from the 6G System Data based on the requirement of a third-party application (e.g., user intent). | PR W.4.6-1 | Data exposure |

Editor’s Note: PR 5.9.2.2-3 has an associated EN, so not eligible for consolidation at this time.  If agreed, align w/Computing CPRs.

- Y.1.5-1-x

  - S1-254250 (Qualcomm): ad the folloiwng requirment to Table Y.1.5-1

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.5-1-y | Subject to operators’ policies, regulations and user consent, the 6G system shall provide means for a network operator to monitor network coverage and/or traffic usage, including collection of information from UEs (with subscription to the network operator). NOTE 1: Monitoring and collection of information from a UE is assumed to be authorized and configured by the UE’s home operator, for certain geographical area(s) and/or time(s), NOTE 2: The traffic usage information collected from a UE is for traffic associated with that UE. | PR 5.7.9.3-1 | NW coverage/usage verification Moved from below (Y.1.7) |

* * * Next Change: Table Y.1.6-1 – Charging  * * * *

### Y.1.6	Charging

Editor’s Note: This includes (but not limited to) PRs on charging throughout the document.

NEW Editor’s Note: PR 7.17.6-4 is for FFS, so it is not included in consolation into Table Y.1.6-1 at this time.

Table Y.1.6-1 – Charging

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.6-1-1 | Subject to operator policy and user consent, the 6G system shall support means to provide users with differentiation of QoS and charging based on users’ digital identity information issued by a third party and users’ subscription information. | PR 5.5.9.6-1 | Digital Identity |
| Y.1.6-1-2 | The 6G system shall be able to provide charging and accounting mechanisms for 6G System Data. | PR 5.9.2.2-5 | System Data |
| Y.1.6-1-3 | The 6G system shall provide appropriate charging support for differentiated services per media component (e.g. to meet SLA requirements). | PR-5.9.8.2-3 | Service Awareness Differentiated Svcs SLA support |
| Y.1.6-1-4 | The 6G network shall support charging for the usage of network resources by third parties. | PR 6.2.6-2 | 3rd party charging |
| Y.1.6-1-5 | The 6G network shall support charging information collection for the intelligent communication assistant service. | PR 6.11.6-2 | Intelligent Communication Assistant From S1-254250 [QC] Merge with 8 &15? |
| Y.1.6-1-6 | The 6G system shall be able to collect charging information for the computing resource(s) coordinated by the 6G network e.g. per request, per UE. | PR 6.14.6-1 | Computing |
| Y.1.6-1-7 | The 6G network shall support charging information collection per UE and/or for the usage of network resources related to multiple 3GPP services (e.g. communication service, sensing service, AI service) to satisfy the intents received from the UE. | PR 6.21.6-5 | UE charging collection Charging for multiple services |
| Y.1.6-1-8 | The 6G network (e.g. in conjunction to IMS) shall support charging information collection for the intelligent calling service. | PR 6.22.6-2 | IMS Intelligent Calling Service From S1-254250 [QC] Merge with CPR -15. May clarify what “in conjunction with IMS” mean. Is it that both the 6G NW and IMS shall be able to support? |
| Y.1.6-1-9 | Subject to operator’s policy, the 6G network shall support the collection of charging information for the AI/ML model training, AI model inference and management which are within the service hosting environment for authorized third parties. | PR.6.25.6-3 | AI/ML model training AI model inference & Mgt |
| Y.1.6-1-10 | The 6G network shall be able to collect charging information based on unique traffic characteristics pertinent to e.g. specific AI applications, or enhanced/guaranteed user experience. | PR 6.26.6-4 | Charging for unique traffic characteristics or enhanced/guaranteed user experience Proposed to be changed by S1-254250 |
| Y.1.6-1-11 | The 6G network shall be able to collect charging information for AI service per user. | PR 6.29.6-2 | AI service |
| Y.1.6-1-12 | The 6G network shall be able to collect charging information for the usage of AI/ML models that are stored or generated within the Service Hosting Environment. | PR 6.36.6-3 | AL/ML model usage |
| Y.1.6-1-13 | Based on operator policy and user consent, the 6G network shall support mechanism to collect charging information for customized service based on received intent(s) from user. | PR 6.44.6-3 | Customized Service based on received intent(s) |
| Y.1.6-1-14 | Based on regulatory requirements, operators’ policy and agreement with 3rd party, 6G network shall support charging for services (e.g. combination of communication, computing service, sensing service) provided to 3rd party applications (e.g. AI agents). | PR 6.55.6-2 | 3rd party AI agents Proposed to be changed by S1-254250 |
| Y.1.6-1-15 | The 6G system shall support collection of charging information associated with initiating and terminating an intelligent immersive calling service. | PR 9.10.6-3 | Intelligent Immersive Calling Service From S1-254250 ]QC]: Merge with CPR -8 |

* * * Next Change: Table Y.1.7-1 – NDT * * *

### Y.1.7	Other operational aspects

Editor’s Note: This includes (but not limited to) PRs from clause 5.5.8, 5.8.7, 5.9.3, throughout the document.

Table Y.1.7-1 – NDT

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.7-1-1 | Subject to operator’s policy, the 6G network should be able to support a Network Digital Twin for network and service management, enabling autonomous networks (including network and OAM) for better service offering, e.g. through simulating in the network characteristics and behaviours provide services to users (e.g. user traffic running on the virtual replica of mobile network or part of it). NOTE 2: The network characteristics can be NFs configuration, UE throughput in NFs, the number of current subscribers, UE QoS, user experience data, fault prediction and non 3GPP data such as user complaint data etc. | PR 5.5.8.6-1 PR 5.9.3.6-1 | Basie NDT functionality |
| Y.1.7-1-2 | The 6G network shall provide suitable means for communicating with the NDT of operator controlled compute resources that are in the Service Hosting Environment or the cloud, for the purpose of energy consumption analysis and optimisation. | PR 5.8.7.6-1 | Network Energy Consumption (note) Could also go into Table Y.1.4-1 – Energy-related aspects |
| Y.1.7-1-3 | The 6G system shall expose an API for the 6G network to transfer network function related information (e.g. NF configuration, UE context related data such as UE QoS etc.) to NDT to replicate 6G network in NDT. | PR 5.9.3.6-2 | (note) |
| Y.1.7-1-4 | The 6G system shall expose an API for the 6G network to receive feedback (e.g. optimal QoS parameter values, NF configuration etc.) from NDT. | PR 5.9.3.6-3 | NDT Exposure (note) |
| Y.1.7-1-5 | Subject to operator’s policy, the 6G system shall be able to transfer the data (e.g. related to service outage) from 6G Network to NDT. | PR 5.9.3.6-4 | (note) |
| NOTE: These requirements only apply if the 6G network supports a Network Digital Twin. |  |  |  |

Company Proposals:

- S1-254250 (Qualcomm): [QC]: comments will be provided later.

* * * Next Change: Table Y.1.7-2 – OAM * * *

Table Y.1.7-2 – OAM

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
|  |  |  | Could also go into Table Y.1.5-1 – Data Collection and Consumption From S1-254250 [QC]: do not agree to add this PR under OAM. Propose moving it to Y.1.5-1 (or Y.1.1-2) |
| Y.1.7-2-2 | Subject to operator’s policy, the 6G network (OAM) should support autonomous detection, isolation, and recovery of failed service with minimum human intervention, to ensure continuous operation. | PR 5.6.1.2-1 PR 5.6.3.6-1 | Resilience From S1-254250 [QC]: prefer to move it under resilience |

Company Proposals:

- Y.1.7.7-2:

  - S1-254130 (NEC): add the following to Table Y.1.5-1

| Y.1.7-2-2 | Subject to regulatory requirements and operator’s policy, the 6G network (OAM) should support autonomous detection, isolation, and recovery from network or service failure with minimum human intervention, to ensure continuous operation. | PR 5.6.1.2-1 PR 5.6.3.6-1 | Proposed merged CPR Resilience |
| --- | --- | --- | --- |
|  | Subject to regulatory requirements and operator’s policy, 6G network should support suitable means to detect, isolate and recover from network failure. | PR 5.6.1.2-1 | Resilience |
|  | Subject to operator’s policy, the 6G network (OAM) should support autonomous detection and recovery of failed service with minimum human intervention, to ensure continuous operation. | PR 5.6.3.6-1 | Resilience |

* * * End of Changes * * * *