3GPP TSG SA WG 1 Meeting #112 	S1-254416

17-21 November 2025, Dallas, Texas, USA	(revision of S1-254238r1)

Source:	Huawei, HiSilicon

pCR Title:	Pseudo-CR on Update of clause 6.7 use case on AI-Agents communication

Draft Spec:	3GPP TR 22.870 v0.4.1

Agenda item:	8.1.3.2

Document for:	Approval

Contact:	Shuang ZHANG, zhang.shuang2 AT Huawei DOT com

Abstract: Update clause 6.7’s existing PR with clarification.

1. Introduction

Some aspects of the use case are clarified by pointing out that in reality different 3rd party AI Agents can support their own specific protocols, and this should be considered when providing communication services to them. Accordingly, the corresponding PR is updated.

2. Reason for Change

The use case 6.7 on AI Agents communication captures the scenario for AI Agents to complete a task together. In reality, more often than not the 3rd party AI Agents are from different vendors, hence the 6G network shall support communication between/among multiple 3rd party AI Agents by supporting the cross-protocol interaction aspects of the multi-vendor 3rd party AI Agents.

3. Conclusions

Complement the existing PR with the identified aspects.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870, v041.

Changes in r1:

1. simplified the update for PR4 – focus on providing efficient communication services among multiple 3rd party AI Agents considering each communicating via their own specific protocols

2. shortened NOTE2: keeping the last part to be crisp.

Changes in S1-254416:

Simplified wording in PR.

* * * First Change * * * *

### 6.7.3	Service Flows

Figure 6.7.3-1: Task-Oriented Multi-Group Communication Network for multiple user AI-agents communication

- Bob, Alice, and Charlie each request the operator to create AI agent groups for them. They then invite their respective AI agents, such as their AI assistants, smart cars, drones, etc., to join their individual groups. In other words, there are separate groups for Bob, Alice, and Charlie, allowing the AI agents within the same group to communicate with each other. In addition to owner information (e.g. related user, etc.), examples attributes for Bob’s AI agent in this use case are as follows:

Table 6.7.3-1: Illustrative attributes of AI agents

|  | Service Features | Capabilities | Permission |
| --- | --- | --- | --- |
| AI assistant | - Service description: (Voice and Text-Based Interaction, Information Retrieval, Personalized Recommendations) - Service area: wide - Moving speed: N/A | WiFi & cellular connection; Tools (Perception + Action); Reasoning & Decision making; Memory + Reflection; | InvitationAllowed: true; DiscoveryAllowed: true ShareAllowed: true; CreatGroupAllowed: true |
| Drone | - Service description (e.g. Radar, camera, Grasping, navigation); - Service area: wide - Moving speed: Medium | WiFi & cellular & V2V connection; Tools (Perception + Action); | InvitationAllowed: true; DiscoveryAllowed: true ShareAllowed: true; CreatGroupAllowed: true |
| Intelligent vehicle | - Service description (e.g. Radar, camera, driver assistance, and improved navigation); - Service area: wide - Moving speed: High | WiFi & cellular connection; Tools (Perception + Action); Reasoning & Decision making; Sensing; | InvitationAllowed: true; DiscoveryAllowed: true ShareAllowed: true; CreatGroupAllowed: false |
| Home robot | - Service description (e.g. voice interaction, cleaning, security monitoring); - Service area: Local - Moving speed: Low | WiFi connection; Tools (Perception + Action); | InvitationAllowed: true; DiscoveryAllowed: true ShareAllowed: true; CreatGroupAllowed: true |

As the table illustrates, AI agents vary in their service features, capabilities and permission, etc. This results in different network requirements. For example, an AI assistant might require cloud storage for extensive data storage, while a drone may need network-provided sensing and computing capabilities for obstacle identification.

- The newly purchased home robot joins Bob’s group after authentication and is associated with Bob’s identity. Then, the home robot reports its capability to the group.

- Bob sends the request to his AI-assistant and requests the AI-assistant to arrange the gathering as task coordinator. The AI-assistant received Bob’s request. Based on the local or cloud-based knowledge base, Bob’s AI-assistant retrieves the history of past family gatherings, determines the list of sub tasks and distributes the sub tasks to corresponding AI agents.

- Bob’s AI-assistant communicates with the discovered cleaning robot based on its capabilities and sends the request to the cleaning robot to carry out a full house cleaning. When the cleaning robot needs collaboration from other robots, e.g. to move some heavy furniture, it will setup a cleaning group with others to enable efficient communication within the cleaning group. After the cleaning, the communication group will be released accordingly.

- Bob’s AI-assistant communicates with the discovered and selected restaurant’s AI assistant by matching the requirements and AI agents’ attributes. Bob’s AI-assistant orders food for tomorrow and negotiate the food delivery time and location with restaurant’s AI assistant. The restaurant’s AI assistant selects a drone based on its capabilities from restaurant’s group, to do the food delivery. Then the restaurant’s AI assistant sends the determined time and location to the food delivery drone and sends the information of selected drone to Bob’s AI-assistant, so that Bob’s AI-assistant can communicate with drone.

- During food delivery, the drone offloads a portion of the computational tasks to the network, to assist it in obstacle identification, by assisting in the process of modal transformation, for example, by pre-processing the images captured by drones, and obtaining alert data. When drone arrives at the pick-up point, they will setup a temporary communication group with the home robot to enable secure and cooperative food delivery. Considering the drone and home are from different vendors, they support different protocols and information modalities, the communication is supported even if the drone and home robot transmit different modality information, e.g. video/text: the 6G network transform the video taken by drone and pick-up point address in text format to semantic maps with guideline marks that the robot can recognize to retrieve the delivery, etc. The communication group will be released after the delivery. In addition, if the interaction between the drone and the home robot is frequent in 6G network, a novel specific protocol will be generated for efficient transmission.

- Bob’s AI-assistant communicates with Alice’s AI assistant and intelligent vehicle. Bob’s AI-assistant negotiates with Alice’s AI assistant for the time and place to pick up Alice’s and sends the request to the intelligent vehicle to pick up Alice’s with negotiation result (i.e. the pick-up time and place).

- Bob’s AI-assistant communicates with Charlie’s AI assistant and intelligent vehicle. Bob’s AI-assistant negotiates with Charlie’s AI assistant for the time and place to pick up Charlie’s and sends the request to the intelligent vehicle to pick up Charlie’s with negotiation result (i.e. the pick-up time and place).

* * * Next Change * * * *

### 6.7.6	Potential New Requirements needed to support the use case

[PR 6.7.6-1] Based on regulatory requirements, operators’ policy and user consent, 6G network shall support trusted network access for 3rd party AI Agent, and support a mechanism to expose 3rd party AI Agent’s attributes (e.g. related users, sensing capabilities, AI capabilities, service features) to other 3rd party AI Agents.

[PR 6.7.6-2] Based on regulatory requirements, user consent, operators’ policy and agreement with authorized 3rd party, the 6G network shall be able to support security identification for 3rd party AI Agents provided by authorized 3rd party associated with a user (e.g. AI Agents belonging to a customer).

[PR 6.7.6-3] Based on regulatory requirements, operators’ policy and user consent, 6G network shall support mechanisms for 3rd party AI Agents to provide/register their attributes (e.g. sensing capabilities, AI capabilities, service features, associated authorized users) to 6G network, and discover other authorized 3rd party AI Agents to achieve collaborative tasks.

[PR 6.7.6-4] Based on regulatory requirements and operators’ policy, the 6G network shall provide means to support efficient and secure communication (including multi-modality exchange) between multiple 3rd party AI Agents considering each communicating via their own specific protocols on UEs over a target area.

NOTE 1:	This requirement can apply to 3rd party AI Agents of same users or different users. It is expected that the required communication service would be provisioned in the range of minutes to days, depending on use case. Lower for temporary task and higher for long term task.

NOTE 2:  It’s expected that the required communication service would be provisioned so that 3rd party AI Agents with different protocols can communicate with each other efficiently.

[PR 6.7.6-5] Based on operator policy, the 6G network shall be able to support secure means to expose different services, e.g. computing offloading service in Service Hosting Environment, to the authorized third-party AI Agent.

* * * End of Change * * * *