3GPP TSG-SA WG1 Ad-hoc on SMARTER (5G)	S1-153040

Vancouver, Canada, 19-21 October 2015

Title:	Discuss grouping of SMARTER requirements

Agenda Item:	3.1

Source:	Huawei

Contact:	Laurence Meriau laurence.meriau@huawei.com

Abstract: SA1 needs to identify requirements for SMARTER, some grouping will help to clarify the main directions to focus on for network evolution.

Note: this is a re-submission of Belgrade S1-152087, with clarifications and improvements due to emails discussions on grouping:

- Criteria used for grouping

- Grouping proposal

- Matrix of requirements showing the detailed reasons for grouping, it distinguishes Verticals (eMBB, mMTC... ) and consider all new use cases agreed in Belgrade

- Matrix to compare different companies grouping proposals (subject to change)

- Proposals for TR changes: it is proposed to show groups (so the expected SID) at the end of the TR

- Criteria used for Grouping

There is a large variety of use cases in the TR. We used following criteria for Grouping:

Criteria#1:

- we considered each individual requirement

- and created 2 detailed Matrix in section III & IV to distinguish the groups of Verticals and Horizontal:

  - Matrix of section III: the 279 requirements of the TR show commonalities/difference between use cases,

  - Matrix of section IV: gives an overview of some other companies grouping (Vodafone, Alcatel-Lucent, Qualcomm), to help to reach a common SA1 visibility.

This matrix shows naturally certain groups

Criteria#2: We considered SA1 has some benefit to identity each Vertical group when some requirements differ:

- To ensure SA1 floor share the same understanding on needs

- To ensure groups support specific business model: for ex separate V2X from eMBB as this could be 2 separate business: with a MVNO deploying for example V2X, with no MBB business at all;

- To allow flexibility in Stage2:

  - It is expected that if a UE needs support for 2 types of Verticals (like a UE using eMBB service in a Vehicle), then 2 groups of characteristics have to be considered for the slice creation. While if a UE supports only 1 Vertical, there is no need for the operator to consider characteristics of the other Verticals for this UE for the slice supporting the UE.

  - For ex: V2X differs from eMBB due to its high reliability, lower rate, high speed and high positioning accuracy, V2X group also needs to consider the migration of the V2X service e.g. specified in 3GPP R14 while eMBB group does not need to consider this aspect.

- Grouping proposal

- Groups obtained from Matrix for Verticals (eMBB, V2X, Critical Communication, mMTC):

The following table shows Groups for verticals from Matrix of section III.

The column on the right highlights the main differences between this group and others.

| Verticals Summary | QoE and Peak data rate | Latency | Reliability | Comm. efficiency | Traffic density | Conn. density | UE speed | Position accuracy | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Group#1 eMBB | Very High data rate (Up to 10 Gbps) | Low latency |  |  | High traffic density Tbps/ km2） | High density for UE |  |  | Characterised by very high traffic, high bit rate. No Reliability needed, no accuracy need |
| Group#3 Critical Comm. |  | Realtime low latency | Ultra high Reliability |  | High Density |  |  | Precise position within [10 cm] | Characterised by low latency, ultra high reliability |
| Group#4 mMTC |  |  |  | Coverage enhancement, Efficient resource and signalling to support low power | High Density | Low mobility (for majority of MTC cases except for inventory) |  | High positioning accuracy in both outdoor and indoor (e.g., 0.5m) | Important difference with CC is: No Latency, No reliability |
| Group#2 V2X | Medium Rate (10 of Mbps per device) | Low latency | High Reliability |  | Medium traffic density | Medium connection Density | High Speed(up to 500 KM/h) | High positioning accuracy (e.g. 0.1 meters) | V2X differs from eMBB due to its high reliability , lower rate , high speed and high positioning accuracy |

- Group#1: eMobile Broadband (eMBB)

  - 5.5 Mobile broadband for indoor scenario

  - 5.6 Mobile broadband for hotspots scenario

  - 5.7 On demand networking (support high density of terminals)

  - 5.10 Mobile broadband services with seamless wide area coverage

  - 5.11 Virtual presence

  - 5.30 Connectivity Everywhere

- Group#2: Critical Communication (CC):

  - 5.1  Ultra-reliable communications

  - 5.12 Connectivity for drones

  - 5.13 Industrial control

  - 5.14 Tactile Internet

  - 5.15 Localized real-time control

  - 5.17 Extreme real-time communications and tactile internet

  - 5.18 Remote control

  - 5.44  Cloud Robotics

  - 5.45  Industrial Factory Automation

  - 5.46  Industrial Process Automation

  - 5.50  Low-delay speech coding

  - 5.54  Local UAV Collaboration

- Group#3: mMTC:

  - 5.19 Light weight device configuration

  - 5.20 Wide area sensor monitoring and event driven alarms

  - 5.24  Bio-connectivity

  - 5.25  Wearable Device Communication

  - 5.40  Devices with variable data

  - 5.41  Domestic Home Monitoring

  - 5.42  Low mobility devices

  - 5.43  Materials and inventory management and location tracking

  - 5.59  Massive Internet of Things M2M and device identification

- Group#4: V2X:

  - 5.10  Mobile broadband services with seamless wide-area coverage (500 km/h speed in V2X)

  - 5.29  Higher User Mobility

  - 5.32  Improvement of network capabilities for vehicular case

  - 5.33  Connected vehicles

  - 5.53  Vehicular Internet& Infotainment

B) Horizontal Group#5: Network Operation for foundation for a new architecture:

This groups some “horizontal” requirements that are used to support Verticals.Network operation looks to be an important group to be considered in SMARTER to allow the operator to have framework and the flexibility to select the network functions and capabilities it needs to provide the diverse verticals, while avoiding other unnecessary functionalities and to support performances needed for future services and  reduce the time to market for new services.

A single group is proposed with some particular “families”. A split in independent groups looks to be complex with no clear benefit (see Comparison of companies’ position in Table of section III).

  - 5.2   Network Slicing

  - 5.4   Migration of Services from earlier generation

  - 5.8   Flexible application routing

  - 5.9   Flexibility and scalability

  - 5.16 Coexistence with legacy systems

  - 5.21  IoT Device Initialization

  - 5.22  Subscription security credentials update

  - 5.35  Context Awareness to support network elasticity

  - 5.47  SMARTER Service Continuity

  - 5.49  Network capability exposure

  - 5.51  Network enhancements to support scalability and automation

  - 5.58  Use Case for Green Radio

The following use cases consider the access types:

  - 5.23 Access from less trusted networks

  - 5.26 Best Connection per Traffic Type

  - 5.27 Multi Access network integration

  - 5.28 Multiple RAT connectivity and RAT selection

And some use cases focussed on minimum service capability by the network:

  - 5.3   Lifeline communications / natural disaster

  - 5.31 Temporary Service for Users of Other Operators in Emergency Case

  - 5.48  Provision of essential services for very low-ARPU areas

Some other use cases are to attract new services/create new values

Note: either the service will be defined by 3GPP (like done for Public Safety, MCPTT) or 3GPP will have to consider services defined in other SDOs. SA1should define the relationship with these services, for example considering currently ETSI ISG MEC work on this subjects and how 3GPP can support this work to offer value added with the operator’s network to attract these service. We consider it is important SA1 gives a clear message that future network has to attract new service, that this is a target also for the new architecture discussions.

  - 5.36       In-network caching

  - 5.37       Routing path optimization when server changes

  - 5.38       ICN Based Content Retrieval

  - 5.39       Wireless Briefcase

And use cases for network optimisation:

  - 5.34       Mobility on demand

  - 5.52       Wireless Self-Backhauling

  - 5.55      High Accuracy Enhanced Positioning (ePositioning)

  - 5.56       Broadcasting Support

  - 5.57       Ad-Hoc Broadcasting

Also some verticals use cases impact the Network Operation (see table below for detailed requirements):

  - eMBB

  - V2X

  - mMTC

  - CC

- Matrix of all requirements of the TR

This matrix below list all the TR use cases and all the requirements of each use case (lines) which are put in appropriate column based on their purpose (columns).

When characteristics are similar, different groups are visible and a grouop # and name is addedas well as line summarizing the main characteristics.

Note: in the following tables:

- QoE and Peak data rate includes: “QoE Data Rate” and “Peak Data Rate”

- Latency includes “UE-to-network Latency” and “UE-UE Latency”

- Reliability includes “Reliability”, “Availability”, “Jitter”

- Communication efficiency includes “Communication Efficiency” , “Traffic Freq.”, “Comm. Range” and “Battery”

- Network operation contains: (Slice, provisioning, roaming, broadcast, security, service exposure, legacy service support, LI/Charging, Minimum service, Multiple access)

- Network Optimization contains: Backhaul, Mobility, Relay, Traffic Multiplex, UE-UE Networking, Energy saving)

| Group #1 eMBB | QoE and Peak data rate | Latency | Reliability | Comm efficiency | Traffic density | Connection density | UE speed | Positioning accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| eMBB summary | Very High data rate (Up to 10Gbps) | Low latency |  |  | High traffic density Tbps/ km2） | High density for UE/traffic |  |  |
| 5.5 Mobile broadband for indoor scenario | Up to Gbps of level |  |  |  | Tbps/ km2 |  |  |  |
|  | Peak rate: 10 of Gbps |  |  |  |  |  |  |  |
|  |  | very low |  |  |  |  |  |  |
| 5.6 Mobile broadband for hotspots scenario | Up to Gbps of level while the user is moving slowly |  |  |  | Tbps/ km2 |  |  |  |
|  | Peak rate:10 of Gbps while moving slowly |  |  |  |  |  |  |  |
| 5.7 On-demand Networking | DL: 300Mbps; UL: 50Mbps |  |  |  |  | in 200-2500 /km2 |  |  |
|  | DL:50Mbps; UL: 25Mbps |  |  |  |  | in 2000/km2 |  |  |
| 5.10 Mobile broadband services with seamless wide-area coverage | anytime and anywhere, e.g., 100Mbps. |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | e.g., 500km/h. |  |
|  |  |  |  |  |  | 500 active UEs simultaneously |  |  |
|  |  | low latency for high speed |  |  |  |  |  |  |
| 5.11 Virtual presence | high bandwidth (bidirectional) | low latency |  |  |  |  |  |  |
| 5.30 Connectivity Everywhere |  |  | aerial object with reliable mobile broadband connectivity. |  |  |  |  |  |
|  |  |  | nautical object with reliable mobile broadband connectivity. |  |  |  |  |  |
|  |  | reliable low-latency connectivity between aerial objects |  |  |  |  |  |  |

| Group #2 Critical Communication | QoE and Peak data rate | Latency | Reliability | Comm. efficiency | Connection density | UE speed | Position accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CC summary |  | Realtime /low latency | Ultra high Reliability | Comm. Range:10-20m | High Density |  | ## Precise position within [10 cm] |
| 5.1 Ultra-reliable communications |  |  | limit the duration of service interruption for mission critical traffic |  |  |  |  |
|  | “Substation protection and control”: ~12.5Mbps per MU at 256 samples/cycle | “Substation protection and control”: as low as 1 ms end-to-end; the case “Smart grid system”: less than 8 ms | “Substation protection and control”: Packet loss rate: as low as 1e-04; the case “Smart grid system”: reliably delivered in 8 ms |  |  |  |  |
| 5.12 Connectivity for drones |  | Round trip latency less than [150 ms] |  |  |  |  |  |
|  |  |  | [near 100%.] |  |  |  |  |
|  |  |  | the same level for current aviation Air Traffic Control (ATC) |  |  |  |  |
|  |  |  | Realibity with Priority, Precedence, Preemption (PPP) mechanisms |  |  |  |  |
|  |  |  |  |  |  |  | within [10 cm] |
|  | high speed uplink bandwidth at least [20Mbps], for a flying UE at low altitude of [10-1000] meters |  |  |  |  | high speed as maximum as [300km/h]. |  |
| 5.13 Industrial Control |  | low latency (~1 ms) |  |  |  |  |  |
|  |  |  | high reliability |  |  |  |  |
|  |  |  | high availability |  |  |  |  |
|  | UL:10 of Mbps per device |  |  |  | in a dense environment |  |  |
| 5.14 Tactile Internet |  | low latency (~1 ms) |  |  |  |  |  |
|  |  |  | high reliability |  |  |  |  |
| 5.15 Localized real-time control |  | UE-UE latency: low latency [1-10 ms] | Extremely high reliability |  |  |  |  |
| 5.17 Extreme real-time communications and the tactile internet |  | 0.5ms one-way delay |  |  |  |  |  |
| 5.18 Remote Control |  | Round trip latency less than [150 ms] |  |  |  |  |  |
|  |  |  | near 100% |  |  | 120km/h |  |
|  |  |  | Reliability to be at the same level for current aviation Air Traffic Control (ATC) |  |  |  |  |
|  |  |  | With Priority, Precedence, Preemption (PPP) mechanisms |  |  |  |  |
|  |  |  |  |  |  |  | < [10 cm] in densely populated areas. |
| 5.44 Cloud Robotics | UL:synchronized audio, video and data in real time |  |  |  | high density distribution |  |  |
|  |  | end to end latency lower than [10ms] |  |  |  |  |  |
| 5.45 Industrial Factory Automation |  | cycle times of [1ms to 2ms.] |  |  |  |  |  |
|  |  |  | [below 10usecs.] |  |  |  |  |
|  |  |  | [below 10-9.] |  |  |  |  |
|  |  |  |  | Comm. Range:10-20m |  |  |  |
| 5.46 Industrial Process Automation |  |  |  |  | 10k sensor /10sqkm |  |  |
|  |  |  | Packet loss <10-5 |  |  |  |  |
|  |  | 50-100ms two way |  |  |  |  |  |
|  |  |  |  | Battery: multiple years, transaction rate of one every few seconds |  |  |  |
| 5.50 Low-delay speech coding |  | speech with very low one-way service latency [10 ms] |  |  |  |  |  |
| 5.54 Local UAV Collaboration |  | [10 ms] |  |  |  |  |  |
|  |  |  | Near [100%] |  |  |  |  |
|  |  |  | Priority, Precedence, Preemption (PPP) |  |  |  |  |
|  |  |  |  |  |  |  | < [10 cm] due to multiple UAVs that may need to collaborate in close proximity to one another. |

| Group #3 massive MTC | QoE and Peak data rate | Latency | Reliability | Comm. efficiency | Connection density | UE speed | positioning accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mMTC summary |  |  |  | Coverage enhancement, Efficient resource and signalling to support low power | High Density | Majority expected with low mobility (location tracking use case for V2X group) | High positioning accuracy in both outdoor and indoor (e.g., 0.5m) |
| 5.19 Light weight device configuration |  |  |  | support devices (e.g., smart meter) with limited communication requirements and capabilities (e.g., devices without an IMS client). |  |  |  |
|  |  |  |  | support a lighter weight signalling for device configuration (i.e., service parameters) than is currently available in EPS. |  |  |  |
| 5.20 Wide area sensor monitoring and event driven alarms | UL only |  |  | low power devices , infrequent uplink data |  |  |  |
|  |  |  |  | power efficiency (e.g., battery life up to more than 10 years |  |  |  |
|  |  |  |  | efficient data transmission with limited resource and signalling cost |  |  |  |
|  |  |  |  |  | high density massive connections (e.g.1 million connections per square kilometre) |  |  |
|  |  |  |  | coverage enhancement (e.g., 20dB better coverage than Rel 99 GPRS system). |  |  |  |
| 5.24 Bio-connectivity |  |  |  |  |  |  |  |
|  |  |  |  | efficient transfer of data for UEs which only support 3GPP device-to-device communication |  |  |  |
| 5.25 Wearable Device Communication |  |  |  | an UE (e.g wearable device) to access to the 3GPP network directly, or via an authorized UE (e.g. smart phone) when the two UEs are within a short range communication (e.g., less than [TBD]m) |  |  |  |
| 5.40 Devices with variable data |  |  |  | efficient and flexible for both low throughput short data bursts and high throughput data transmissions (e.g., streaming video) from the same device. |  |  |  |
|  |  |  |  | support efficient signaling mechanisms (e.g., signaling is less than payload). |  |  |  |
|  |  |  |  | reduce signalling overhead for security needed for short data burst transmission, without reducing the security protection provided by 4G 3GPP Systems. |  |  |  |
| 5.41 Domestic Home Monitoring |  |  |  | Payload size: messages up to 256 octets per message. |  |  |  |
| 5.42 Low mobility devices |  |  |  | accept information from large numbers of stationary devices with reduced mobility management (e.g., handover support, idle mode mobility management). |  |  |  |
|  |  |  |  | support a resource efficient mechanism to provide information to a stationary device |  |  |  |
|  |  |  |  | provide resource efficient support for stationary devices with reduced mobility management (e.g., handover support, idle mode mobility management). |  |  |  |
| 5.43 Materials and inventory management and location tracking |  |  |  | a resource efficient mechanism | to accept information from large numbers of locally dense devices, possibly simultaneously. |  |  |
|  |  |  |  | enable sufficient indoor and outdoor coverage (e.g., 20dB better coverage than legacy Rel 99 GPRS system) for a large number of locally dense low power devices. |  |  |  |
|  |  |  |  |  | a mechanism to manage resource (e.g., radio resources) sharing by large numbers of locally dense devices efficiently. |  |  |
|  |  |  |  | with reduced battery consumption. | communication service for high density of devices up to (e.g., 1 million devices per km2) | with high mobility at minimum of 100 km/h |  |
|  |  |  |  |  |  |  | high positioning accuracy in both outdoor and indoor scenarios (e.g., 0.5m) |
| 5.59 Massive Internet of Things M2M and device identification |  |  |  |  | connection densities of [200,000 / km2] |  |  |

| Group #4 V2X (high mobility access) | QoE and Peak data rate | Latency | Reliability | Comm . efficiency | Traffic density | Connection density | UE speed | Position accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X summary | Medium Rate (tens of Mbps per device) | Low Latency | High Reliability |  | Medium traffic density | Medium connection Density | High Speed High Speed (up to 500 KM/h) | High positioning accuracy (e.g. 0.1 meters) |
| 5.10 Mobile broadband services with seamless wide-area coverage |  |  |  |  |  |  | e.g., 500km/h. |  |
| 5.29 Higher User Mobility |  |  |  |  |  |  | support enhanced mobile broadband services in fast moving vehicles (e.g. up to 500 km/h) |  |
|  |  |  |  |  |  |  | support enhanced connectivity services in fast moving airplanes (e.g. up to 1000 km/h) |  |
|  |  |  |  |  |  |  | provide the mobile broadband service in fast moving vehicles with enhanced system experience. |  |
|  |  |  |  |  |  |  | provide the airplanes connectivity service with enhanced system experience. |  |
| 5.32 Improvement of network capabilities for vehicular case | Streaming:[100] Mbps (DL) |  |  |  |  |  |  |  |
|  |  | (end-to-end) is less than [1]s |  |  |  |  |  |  |
|  |  | (over the air) is less than [200] ms |  |  |  |  |  |  |
|  |  |  |  |  |  |  | up to 100 km/h |  |
|  | Video call: > [10] Mbps (DL and UL) |  |  |  |  |  |  |  |
|  |  | (end-to-end) is less than [150]ms |  |  |  |  |  |  |
|  |  | (over the air) is less than [30] ms |  |  |  |  |  |  |
|  |  |  |  |  |  |  | up to 100 km/h |  |
|  | Gaming: > [50-120] Mbps (DL or UL) |  |  |  |  |  |  |  |
|  |  | (end-to-end) is less than [7.5]ms |  |  |  |  |  |  |
|  |  | (over the air) is less than [1.5] ms |  |  |  |  | up to 100 km/h |  |
| 5.33 Connected vehicles |  | 1 millisecond end-to-end latency |  |  |  |  |  |  |
|  |  |  | nearly 100% |  |  |  |  |  |
|  | tens of Mbps per device |  |  |  | in a dense environment |  |  |  |
|  |  |  |  |  |  |  | high mobility (e.g., absolute speed more than 200 km/h while relative speed more than 400 km/h). |  |
|  |  |  |  |  |  |  |  | high positioning accuracy (e.g. 0.1 meters) |
|  |  |  |  |  |  | high density of connections for vehicles (e.g. the number of vehicles can exceed 10000 in scenarios with multiple lanes and multiple levels and types of roads) |  |  |
| 5.53 Vehicular Internet& Infotainment | Internet access: at least [0.5Mb/sec] |  |  |  |  |  |  |  |
|  | Music streaming at least [1Mb/sec] |  |  |  |  |  |  |  |
|  | video streaming at least [5Mb/sec] |  |  |  |  |  |  |  |
|  | UHD streaming at least [15 Mb/sec] |  |  |  |  |  |  |  |
|  |  | no more than [100ms] for internet browsing |  |  |  |  |  |  |
|  |  |  |  |  |  |  | up to 200km/hr. |  |
|  |  |  |  |  |  | up to [2000] vehicles in a given service area [1km2] | ranging from 0km/h (e.g. in a traffic jam) to 200km/h. |  |

Group #5: Network operation

Note: the 1st column also list use cases already existing in Vertical groups tables, because they have requirements that have to be considered for the group on network Operation:

| Use case | Network operation | Network Optimization |
| --- | --- | --- |
| 5.6 Mobile broadband for hotspots scenario |  | flexible and efficient backhaul especially outdoor |
| 5.7 On-demand Networking | adjust the network capacities dynamically |  |
| 5.1 Ultra-reliable communications |  | multiplexing of mission critical traffic and nominal traffic |
| 5.14 Tactile Internet | very difficult to block, modify, or hijack |  |
| 5.15 Localized real-time control |  | self-organized dynamic networking for multi-hop localized network |
| 5.18 Remote Control |  | Seamless connection for fast-moving |
| 5.45 Industrial Factory Automation | sufficiently integrity- and confidentiality |  |
| 5.46 Industrial Process Automation | sufficiently integrity- and confidentiality-protected |  |
| 5.54 Local UAV Collaboration | at the level for current aviation Air Traffic Control (ATC |  |
| 5.33 Connected vehicles | data transmission from one point to multipoint (e.g. multicast and/or broadcast) |  |
| 5.20 Wide area sensor monitoring and event driven alarms | a resource efficient mechanism to provide service parameters and activate groups of low power devices |  |
|  | an appropriate and efficient authentication mechanism for low power devices. |  |
|  | provides appropriate and efficient confidentiality and integrity protection for mobile originated transfer from low power devices. |  |
| 5.24 Bio-connectivity |  | allows the network to be aware of the UEs operating with assistance from a UE-to-network relay. |
| 5.25 Wearable Device Communication |  | service continuity for an UE when it changes from direct access to indirect access via another UE, and vice versa. |
|  | Data transmission in secure manner for an UE, when it’s connected to the network via another UE. |  |
|  | wearable device should have a subscription associated to its own subscriber’s identity (e.g. IMSI) with mobile operator. |  |
| 5.41 Domestic Home Monitoring | support of a UE connected to remote monitoring (that suits home monitoring/control) and includes: Static mobile, Group connection, Normal data service and service for low volume, low frequency updates. |  |
|  | IoT Security is provided only by the connected UE. Further security measures are for implementation at the IoT level applications. |  |
| 5.43 Materials and inventory management and location tracking | efficient authentication for low power devices. |  |
|  | confidentiality and integrity protection for data from low power devices. |  |
|  | efficiently manage one or more devices associated with a service (e.g., inventory tracking service). |  |
|  | support roaming |  |
| 5.59 Massive Internet of Things M2M and device identification | support network servers/applications and devices to identify, address and reach other devices, in a consistent manner independently of how these devices are connected. |  |
| 5.2 Network Slicing | x |  |
| 5.4 Migration of Services from earlier generations | x |  |
| 5.8 Flexible application traffic routing | x |  |
| 5.9 Flexibility and scalability | x |  |
| 5.16 Coexistence with legacy systems | x |  |
|  | x |  |
|  | x |  |
| 5.21 IoT Device Initialization | x |  |
|  | x |  |
| 5.22 Subscription security credentials update | x |  |
|  | x |  |
| 5.35 Context Awareness to support network elasticity | x |  |
| 5.47 SMARTER Service Continuity | x |  |
| 5.49 Network capability exposure | x |  |
| 5.51 Network enhancements to support scalability and automation | x |  |
| 5.58 Use Case for Green Radio |  | [1000] times energy efficiency compared to legacy system. |
| 5.23 Access from less trusted networks | x |  |
| 5.26 Best Connection per Traffic Type | x |  |
| 5.27 Multi Access network integration | x |  |
| 5.28 Multiple RAT connectivity and RAT selection | x |  |
| 5.3 Lifeline communications / natural disaster | maintain an appropriate level of communications security for temporary service. |  |
| 5.31 Temporary Service for Users of Other Operators in Emergency Case |  | support temporary service for users of other than home operators as temporary users in emergency case by serving operator policy |
|  |  | support defining the limited set of necessary communication services and acceptable terminal features for temporary users by serving operator policy |
|  | maintain an appropriate level of communications security for temporary service |  |
| 5.48 Provision of essential services for very low-ARPU areas | x |  |
|  | x |  |
|  |  | minimise as much as possible the traffic (Data and signalling) on the interfaces between the access network and the core network in order to reduce the amount of backhaul traffic. |
|  |  | be activated/deactivated either manually (e.g., on demand), or automatically (e.g., reaching a threshold). |
|  |  | Service may be restricted to authorized users |
|  |  | The base station may be in listen mode |
|  |  | Traffic density: provide the essential services with traffic density of 16 Mbps / km2 |
|  | x |  |
|  | inform UEs what capabilities are supported (e.g., to allow UEs to determine if the network provides the required capabilities). |  |
| 5.36 In-network caching | x |  |
| 5.37 Routing path optimization when server changes | x |  |
| 5.38 ICN Based Content Retrieval | x |  |
| 5.39 Wireless Briefcase | x |  |
| 5.34 Mobility on demand |  | define different levels of mobility support for different UEs. |
|  |  | update the level of mobility support provided for a UE. |
| 5.52 Wireless Self-Backhauling |  | x |
| 5.55 High Accuracy Enhanced Positioning (ePositioning) | support co-existence with legacy 3GPP positioning service and migration to higher accuracy positioning service |  |
| 5.56 Broadcasting Support | x |  |
| 5.57 Ad-Hoc Broadcasting | x |  |

- Comparison of companies grouping proposals

## We considered inputs received from VF, QC and ALU before the deadline, so subject to change.

## So this is subject to change.

## Comparison of grouping for Verticals:

| Color | group |
| --- | --- |
|  | eMBB |
|  | Critical Communication (CC) |
|  | V2X |
|  | Massive MTC |
|  | Network operation |
|  | Network optimization |
|  | Backhaul (use of radio freq for backhaul transport) |
|  | Broadcast |
|  | Access Agnostic |
|  | New capability/service support |
|  | ePositioning |

| Use cases | QC use case grouping | VF use case grouping | ALU use case grouping | HW Use case grouping | Comments |
| --- | --- | --- | --- | --- | --- |
| 5.5 Mobile broadband for indoor scenario |  |  |  |  |  |
| 5.6 Mobile broadband for hotspots scenario |  |  |  |  |  |
| 5.7 On-demand Networking |  |  |  |  | Part of requirement is about QoE data rate and connection density |
|  |  |  |  |  | Part of requirement is: adjust the network capacities dynamically |
| 5.10 Mobile broadband services with seamless wide-area coverage |  |  |  | 5.10 Mobile broadband services with seamless wide-area coverage | Part of this use case requirements is V2X: High mobility requirement (e.g., 500km/h.) and low latency for high speed scenario |
|  |  |  |  | 5.10 Mobile broadband services with seamless wide-area coverage | Part of this use case requirements is eMBB: QoE data rate e.g. 100MB/s anytime and anywhere |
| 5.30 Connectivity Everywhere |  |  |  | 5.30 Connectivity Everywhere |  |
| 5.11 Virtual presence | 5.11 Virtual presence |  | 5.11 Virtual presence |  | Requirement on high bandwidth and low latency |
| 5.32 Improvement of network capabilities for vehicular case | 5.32 Improvement of network capabilities for vehicular case |  | 5.32 Improvement of network capabilities for vehicular case |  | Requirement on QoE data rate and latency with high mobility (up to 100KM/h) so adapted to V2X group |
| 5.50 Low-delay speech coding |  |  | “miscellaneous” group | 5.50 Low-delay speech coding | Only 1 requirement on very low one-way service latency [10 ms] so this makes the use case belongs to CC |
| 5.15 Localized real-time control | 5.15 Localized real-time control |  |  | 5.15 Localized real-time control |  |
| 5.45 Industrial Factory Automation |  |  |  |  |  |
|  |  |  |  |  | Part of requirement is about network operation |
| 5.46 Industrial Process Automation |  |  |  |  |  |
|  |  |  |  |  | Part of requirement is about network operation |
| 5.1 Ultra-reliable communications |  |  |  |  |  |
| 5.12 Connectivity for drones |  |  |  |  |  |
| 5.13 Industrial Control |  |  |  |  |  |
| 5.14 Tactile Internet | 5.14 Tactile Internet |  |  |  |  |
|  |  |  |  |  | Part of requirement is about network operation: self-organized dynamic networking for multi-hop localized network |
| 5.17 Extreme real-time communications and the tactile internet | 5.17 Extreme real-time communications and the tactile internet |  |  |  |  |
| 5.18 Remote Control |  |  |  |  |  |
| 5.44 Cloud Robotics | 5.44 Cloud Robotics |  |  |  |  |
| 5.54 Local UAV Collaboration |  |  | 5.54 Local UAV Collaboration |  |  |
| 5.41 Domestic Home Monitoring |  |  |  |  |  |
|  |  |  |  |  | Part of requirement is about network operation |
| 5.59 Massive Internet of Things M2M and device identification |  |  |  |  |  |
|  |  |  |  |  | Part of requirement is about network operation |
| 5.33 Connected vehicles |  |  |  | 5.33 Connected vehicles | Requirements about high mobility and reliability |
| 5.29 Higher User Mobility |  |  |  | 5.29 Higher User Mobility | Requirements about high mobility should be considered in V2X group |
| 5.53Vehicular Internet& Infotainment |  |  |  |  | Requirements about high mobility |
| 5.19 Light weight device configuration |  |  |  |  |  |
| 5.20 Wide area sensor monitoring and event driven alarms |  |  |  |  |  |
| 5.24 Bio-connectivity |  |  |  |  |  |
| 5.42 Low mobility devices |  |  |  |  |  |
| 5.40 Devices with variable data |  |  |  |  |  |
| 5.25 Wearable Device Communication |  |  |  |  |  |
| 5.43 Materials and inventory management and location tracking |  |  |  |  |  |
| 5.37 Routing path optimization when server changes |  |  |  |  | New service support |
| 5.36 In-network caching |  |  |  | 5.36 In-network caching | New service support |
| 5.38 ICN Based Content Retrieval | (phase2) |  |  | 5.38 ICN Based Content Retrieval | New service support |
| 5.3 Lifeline communications / natural disaster |  |  |  |  | Sub group: minimum service capability |
| 5.31 Temporary Service for Users of Other Operators in Emergency Case |  |  |  |  | Sub group: minimum service capability |
| 5.48 Provision of essential services for very low-ARPU areas |  |  |  |  | Sub group: minimum service capability |
| 5.52 Wireless Self-Backhauling |  |  |  | 5.52 Wireless Self-Backhauling | Sub group: network optimization |
| 5.2 Network Slicing |  |  |  |  |  |
| 5.9 Flexibility and scalability |  |  |  |  |  |
| 5.35 Context Awareness to support network elasticity |  |  |  |  |  |
| 5.51 Network enhancements to support scalability and automation |  |  |  |  |  |
| 5.8 Flexible application traffic routing |  |  |  |  |  |
| 5.49 Network capability exposure |  |  |  |  |  |
| 5.4 Migration of Services from earlier generations |  |  |  |  |  |
| 5.16 Coexistence with legacy systems |  |  |  |  |  |
| 5.21 IoT Device Initialization |  |  | IOT Security |  | Requirement about security |
| 5.22 Subscription security credentials update |  |  | IOT Security |  |  |
| 5.34 Mobility on demand |  |  |  |  |  |
| 5.47 SMARTER Service Continuity |  |  |  |  |  |
| 5.58 Use Case for Green Radio |  |  |  |  |  |
| 5.39 Wireless Briefcase | (Phase2) |  | IOT Security |  |  |
| 5.56 Broadcasting Support |  |  |  |  |  |
| 5.57 Ad-Hoc Broadcasting |  |  |  |  |  |
| 5.55 High Accuracy Enhanced Positioning (ePositioning) |  |  |  |  |  |
| 5.27 Multi Access network integration |  |  |  | 5.27 Multi Access network integration |  |
| 5.28 Multiple RAT connectivity and RAT selection |  |  |  | 5.28 Multiple RAT connectivity and RAT selection |  |
| 5.26 Best Connection per Traffic Type |  |  |  | 5.26 Best Connection per Traffic Type |  |
| 5.23 Access from less trusted networks |  |  |  | 5.23 Access from less trusted networks |  |

- Proposals for TR changes

It is proposed to modify the section 6.3.1 in order to keep trace of groups in the TR in line with agreed groups:

- It is currently proposed to keep the current split into “service requirement” and “operational requirement”, but this may be see as useless after SA1 discussion on grouping.

- No change is proposed for section 6.3.2, as consolidation of requirements is expected to be done in adhoc SID of each group.

- When Section 6 groups requirements, we propose to remove section 7.

It looks also important to ensure that Stage2/3 considers all groups together, as for example there is interaction between vertical services performances and new architecture possibilities.

So, the structure of 6.3 is modified as following:

| Initial 6.3 structure | New structure to show the different groups |
| --- | --- |
| 6.3.1 Collected potential requirements - 6.3.1.1 Potential service requirements - 6.3.1.2 Potential operational requirements | 6.3.1 Collected potential requirements - 6.3.1.1 Potential service requirements for eMBB - 6.3.1.2 Potential operational requirements for eMBB - 6.3.1.3 Potential service requirements for CC - 6.3.1.4 Potential operational requirements for CC - 6.3.1.5 Potential service requirements for mMTC - 6.3.1.6 Potential operational requirements for mMTC - 6.3.1.7 Potential service requirements for V2X - 6.3.1.8 Potential operational requirements for V2X - 6.3.1.7 Potential service requirements for Network Operation - 6.3.1.8 Potential operational requirements for Network Operation |

***** BEGIN 1st CHANGE *****

## 6.3	Considerations on grouping of requirements

The requirements are grouped with the goal to provide network evolutions to be considered by Stage2/3:

- Requirements to support Vertical services:

  - eMBB

  - Critical Communication (CC)

  - mMTC

  - V2X

- Requirements for Network Operation: it is expected that network architecture should be foundation of future system so should be considered rapidly by Stage2.

### 6.3.1	Collected potential requirements

#### 6.3.1.1	Potential service requirements on eMBB

The following table collects potential service requirements for each use case related to eMBB.

Table 6.3.1.1

| <use case 1> |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| <use case 2> |  |  |  |  |  |  |  |  |  |  |

#### 6.3.1.2	Potential operational requirements on eMBB

The following table collects potential operational requirements for each use case related to eMBB.

Table 6.3.1.2

| <use case 1> |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| <use case 2> |  |  |  |  |  |  |  |  |  |  |

#### 6.3.1.3	Potential service requirements on CC

The following table collects potential service requirements for each use case related to Critical Communication.

Table 6.3.1.3

| <use case 1> |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| <use case 2> |  |  |  |  |  |  |  |  |  |  |

#### 6.3.1.4	Potential operational requirements on CC

The following table collects potential service requirements for each use case related to Critical Communication.

Table 6.3.1.4

| <use case 1> |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| <use case 2> |  |  |  |  |  |  |  |  |  |  |

#### 6.3.1.5	Potential service requirements on mMTC

The following table collects potential service requirements for each use case related to mMTC.

Table 6.3.1.5

| <use case 1> |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| <use case 2> |  |  |  |  |  |  |  |  |  |  |

#### 6.3.1.6	Potential operational requirements on mMTC

The following table collects potential service requirements for each use case related to mMTC.

Table 6.3.1.6

| <use case 1> |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| <use case 2> |  |  |  |  |  |  |  |  |  |  |

#### 6.3.1.7	Potential service requirements on V2X

The following table collects potential service requirements for each use case related to V2X.

Table 6.3.1.7

| <use case 1> |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| <use case 2> |  |  |  |  |  |  |  |  |  |  |

#### 6.3.1.8	Potential operational requirements on V2X

The following table collects potential service requirements for each use case related to V2X.

Table 6.3.1.8

| <use case 1> |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| <use case 2> |  |  |  |  |  |  |  |  |  |  |

#### 6.3.1.9	Potential service requirements for Network Operation

This section collects potential service requirements for each use case related to new architecture.

#### 6.3.1.10	Potential operational requirements for Network Operation

This section collects potential service requirements for each use case related to new architecture.

### 6.3.2	Consolidated potential requirements

#### 6.3.2.1	Consolidated potential service requirements

The following table collects consolidated potential service requirements for each use case group.

Table 6.3.2.1

| <use case group 1> |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| <use case group 2> |  |  |  |  |  |  |  |  |  |  |

#### 6.3.2.2	Consolidated potential operational requirements

The following table collects consolidated potential operational requirements for each use case group.

Table 6.3.2.2

| <use case group 1> |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| <use case group 2> |  |  |  |  |  |  |  |  |  |  |

***** END OF CHANGE *****