3GPP TSG-SA WG1 Meeting #73	S1-160385

Okinawa, Japan, 1-5 February 2016	(revision of S1-150385)

Title:	eMBB: Potential consolidation for eMBB non high speed families 1,2,3

Agenda Item:	8.4

Source:	Huawei, Hisilicon

Contact:	Laurence Meriau laurence.meriau@huawei.com

Abstract: This document proposes discussion to work on the consolidation of requirements for eMBB families 1, 2 and 3 for the non high speed users (i.e. not for High Mobility scenario of family 4)

# Discussion

## Family 1,2 and 3 are all for non-high Mobility Users

Requirements defined in family 4 cover all aspects (expected speed, expected density, bitrate values per user  and aggregated bitrate). It has no link with family 1 (as family 4 bitrates are far from the Gbps expected in family 1),  no link with family 2 (as family 4 values are far from Tbps/km2 expected in family 2) and no link with family 3 (as no “support everywhere”).

So family 4 gives a complete picture of traffic caracteristcs expected for users with High Mobility.

In balance to this high mobility family 4, families 1, 2 and 3 appear then all for non-high mobility devices, i.e. for pedestrian or users in urban/suburban vehicles with no such high mobility requirements.

## Family 1,2 and 3 requirements could be consolidated together

In the first figure below:

- We provided all the current requirements of eMBB families 1, 2 and 3 in “black”, to give a complete picture of requirements for non-high mobility users,

- We highlighted in “green”some missing values/KPIs we consider eMBB TR should clarify for a consolidation work,

- We highlighted in “blue” the NGMN’s values/KPIs.

| MBB family | Higher Data Rate | Higher density | Deployment and Coverage |
| --- | --- | --- | --- |
| Speed | Pedestrian (upto10 km/h) in small areas: offices, stadium | Pedestrian or vehicle up to 60 km/h | Pedestrian or vehicle up to 120 km/h |
| Data Rate | Everywhere: minimum 100 Mbps (“DL” is missing) (NGMN: 50+ Mbps) | Everywhere: minimum 100 Mbps (“DL” is missing) (NGMN: 50+ Mbps) | Everywhere: minimum 100 Mbps (“DL” is missing) (NGMN: 50+ Mbps) |
|  | - Peak rate: 10 Gbps, experience rate: up to Gbps (value missing) - Missing: experienced rate 1 Gbps DL, total 17 Tbps/km2 (UL+DL) | Guaranteed exper. rate: 300Mbps DL/50Mbps UL (same in NGMN) Total: 1 Tbps/ km2 |  |
| Density | - Missing : <200/km2 connections - 75000/km2 connections | 200-2500/km2 connections (in line in NGMN) | 400 /km2 connections (suburban) 100/km2 connections (rural) |
| NGMN category | - Not in NGMN - Matches with #2 “Indoor Ultra-high Broadband Access” | #1 “Broadband access in dense area” | #4 “50+ Mbps everywhere” |

From this 1st figure and table, we deducte that consolidation of eMBB TR for non high mobility users like pedestrian or user in vehicles with low speed, could be done as following:

- Minimum 100 Mbps everywhere should apply to any scenario, this is the minimum service expected anywhere

- A scenario B from NGMN can be introduced for perdestrian

- Some KPIs from NGMN white paper could be added

We also noticed that NGMN provides another scenario#3 for pedestrian in crowded area that is curently not clearly distinguished in eMBB TR as NGMN’s KPIs are not in eMBB TR. It has been added as case C in blue in the second table below:

| MBB family | Higher Data Rate |  | Higher density | Deployment and Coverage |
| --- | --- | --- | --- | --- |
| Speed | Pedestrian (upto10 km/h) in small areas: offices, stadium |  | Pedestrian or vehicle up to 60 km/h | Pedestrian or vehicle up to 120 km/h |
| Data Rate | Everywhere: minimum 100 Mbps (“DL” is missing) (NGMN: 50+ Mbps) |  | Everywhere: minimum 100 Mbps (“DL” is missing) (NGMN: 50+ Mbps) | Everywhere: minimum 100 Mbps (“DL” is missing) (NGMN: 50+ Mbps) |
|  | - Peak rate: 10 Gbps, experience rate: up to Gbps (value missing) - Missing: experienced rate 1 Gbps DL, total 17 Tbps/km2 (UL+DL) | Experienced rate (stadium) DL 25Mbps UL 50Mbps missing in eMBB TR Total 12 Tbps/km2 (UL+DL) | Guaranteed exper. rate: 300Mbps DL/50Mbps UL (same in NGMN) Total: 1 Tbps/ km2 |  |
| Density | - Missing : <200/km2 connections - 75000/km2 connections | 150000/km2, 30000/stadium missing in eMBB TR | 200-2500/km2 connections (in line in NGMN) | 400 /km2 connections (suburban) 100/km2 connections (rural) |
| NGMN category | - Not in NGMN - Matches with #2 “Indoor Ultra-high Broadband Access” | Matches with #3 “Broadband Access in a Crowd “ | #1 “Broadband access in dense area” | #4 “50+ Mbps everywhere” |

We propose to add this in eMBB TR. We currently propose text for consolidation requirement.

# Proposal

Text to eMBB Section 7 “Consolidation of Potential Requirments” is proposed below to summarize the above figures/KPIs for non-High mobility users to illustrate current eMBB TR families, add missing KPIs and clarify the distinct missing NGMN family.

***** START OF CHANGE *****

# 7	Consolidation of Potential Requirements

For wide area coverage, the system shall support a minimum experienced data rate per user for mobile broadband services anytime and anywhere, e.g., 100Mbps, for pedestrians (up to 10 km/h) and users in urban vehicle (up to 120 km/h), except for very high user density from150000/km2 (e.g. stadium)

For pedestrian (speed up to 10km/h) in area with connection density lower than 200/km2 (e.g. in office, streets), the 3GPP system shall support the peak data rate at [10] Gbps DL and user experienced data rate up to [TBD] Gbps UL and [TBD] Gbps DL.

For pedestrian (speed up to 10km/h) in high connection density scenario (75000/km2) (e.g. in hotspot), the 3GPP system shall support user experienced data rate up to [TBD] Gbps UL and [1] Gbps DL, resulting into maximum traffic density of 17 Tbps/km2 (UL+DL).

For pedestrian (speed up to 10km/h) in very high density scenario (150000/km2 connection density, like in stadium with 30000 users per stadium) the 3GPP system shall provide user experienced data downlink rate of 25 Mbps and uplink rate of 50Mbps, resulting into traffic density of 12 Tbps/km2 (UL+DL).

For pedestrian (speed up to 10km/h) in high density scenario (200-2500 /km2 connection density, like in city centre) or for users moving in public transportation (buses) with speed 0-60km/h, the 3GPP system shall provide user experienced data downlink rate of 300Mbps and uplink rate of 50Mbps, resulting into traffic density of 1 Tbps/km2 (UL+DL).

***** NEXT CHANGE *****

Annex A:
Comparison of use cases (informative)

The Figure and tables below provide a comparison of use cases: Use cases from eMBB families 1, 2 and 3 are in “black” text,New values are in “green”,Use cases from NGMN are in “blue” text.

Table A.2: eMBB use cases and NGMN use cases

| MBB family | Higher Data Rate |  | Higher density | Deployment and Coverage |
| --- | --- | --- | --- | --- |
| Speed | Pedestrian (upto10 km/h) in small areas: offices, stadium |  | Pedestrian or vehicle up to 60 km/h | Pedestrian or vehicle up to 120 km/h |
| Data Rate | Everywhere: minimum 100 Mbps (“DL” is missing) (NGMN: 50+ Mbps) |  | Everywhere: minimum 100 Mbps (“DL” is missing) (NGMN: 50+ Mbps) | Everywhere: minimum 100 Mbps (“DL” is missing) (NGMN: 50+ Mbps) |
|  | - Peak rate: 10 Gbps, experience rate: up to Gbps (value missing) - Missing: experienced rate 1 Gbps DL, total 17 Tbps/km2 (UL+DL) | Experienced rate (stadium) DL 25Mbps UL 50Mbps missing in eMBB TR Total 12 Tbps/km2 (UL+DL) | Guaranteed exper. rate: 300Mbps DL/50Mbps UL (same in NGMN) Total: 1 Tbps/ km2 |  |
| Density | - Missing : <200/km2 connections - 75000/km2 connections | 150000/km2, 30000/stadium missing in eMBB TR | 200-2500/km2 connections (in line in NGMN) | 400 /km2 connections (suburban) 100/km2 connections (rural) |
| NGMN category | - Not in NGMN - Matches with #2 “Indoor Ultra-high Broadband Access” | matches with #3 “Broadband Access in a Crowd “ | #1 “Broadband access in dense area” | #4 “50+ Mbps everywhere” |