3GPP TSG-SA WG1 Meeting #76Bis	S1-170260

# Spokane, US, 14-20 Jan, 2017 												(revision of S1-170179)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.886 | CR | 0002 | rev | 2 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify use case associated to platooning size in section 7 2 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, KRRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_eV2X |  |  |  |  |  |  | Date: |  | 2016-01-19 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | 2 different platooning sizes are given with no explanation of why there is a difference. |  |  |  |  |  |  |  |  |
| Summary of change: |  | propose to describe the two conditions/scenariosin for the platooning size in platooning use case.Platoon size is assumed larger on the condition of fully automated driving compared to the size on the condition of driver control (limited automated driving) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The platooning size requirement is not clear and will confuse the downstream GWs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 7.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |

***************************** Start of Change  ***************************************

7.2.2	Requirements for Platooning

[CPR.P-001]	The 3GPP system shall be able to support up to 5 UEs for a group of UEs supporting V2X application, within maximum communication range,  with driver control or limited automated driving.

Notes:  The amount of UEs doesn’t necessarily depend on the automisation level, even for driver controlled or limited automated driving the amount may be larger than 5 UEs.

[CPR.P-002]     The 3GPP system shall be able to support a group of up to 20 UEs supporting V2X applications, within maximum communication range, with fully automated driving.

Notes:  for truck platooning, the number of UEs in a platoon will be smaller, because only limited numbers of vehicles due to communication range, the length of truck, inter-truck distance, etc.

[CPR.P-002]	For Vehicle Platooning, the 3GPP system shall be able to support reliable V2V communications between a specific UE supporting V2X applications and up to 19 other UEs supporting V2X applications.

[CPR.P-003]	The 3GPP system shall support relative longitudinal position accuracy of less than 0.5 m for UEs supporting V2X application for platooning in proximity.

# Table 7.2.2-1 Performance requirements for Platooning

| Communication Scenario |  |  |  | Payload (Bytes) | Tx Rate (Message/ Sec) | Max Latency (ms) | Reliabi-lity (%) | Data Rate (Mbps) | Commu-nication Range (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Section # | Description |  | CPR # |  |  |  |  |  |  |
| 5.1 | Among a group of UEs (or two UEs) supporting V2X application |  | [CPR.P-004] | 50-1200 (NOTE 1) | 30 | 10 (NOTE 2) |  |  |  |
|  |  |  | [CPR.P-005] | 300-400 | 30 | 25 | 90 |  |  |
| 5.2 | Between UE supporting V2X application and RSU via another UE supporting V2X application |  | [CPR.P-006] | [50-1200] | 2 | 500 |  |  |  |
| 5.5 | Between UEs supporting V2X application | Driver control | [CPR.P-007] | 300-400 (NOTE 3) |  | 25 | 90 |  |  |
|  |  | Fully automated driving | [CPR.P-008] | 1200 |  | 10 | 99.99 |  | 80 |
| 5.12, 5.13 | Between UEs supporting V2X application | Driver control/ Limited automated driving | [CPR.P-009] | [6500] | 50 | [20ms] |  |  | [10] sec * (max. relative speed) [m/s] |
|  |  | Fully automated driving | [CPR.7.P-010] |  |  | [20ms] |  | [65] | [5] sec * (max. relative speed) [m/s] |
| 5.12, 5.13 | Between UE supporting V2X application and RSU | Driver control/ Limited automated driving | [CPR.7.P-011] | [6000] | 50 | [20ms] |  |  | [10] sec * (max. relative speed) [m/s] |
|  |  | Fully automated driving | [CPR.7.P-012] |  |  | [20ms] |  | [50] | [5] sec * (max. relative speed) [m/s] |

NOTE 1:	This value does not including security related messages component.

# NOTE 2:	This value is end-to-end latency.

NOTE 3:	This value is applicable for both triggered and periodic transmission of data packets.

***************************** End of Change  ***************************************