3GPP TSG-SA1 Meeting #91e	S1-203192

Electronic Meeting, 24 August - 2 September 2020	(revision of S1-20xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.042 | CR | 0006 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Human-Readable Network Name for NPN via NITZ |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | LS from CT in S1-203197 / CP-201361 highlights a gap in the requirements for defining a method to enable the UE to receive a Human-Readable Network Name from the network for a NPN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The existing mechanism Network Identity and Timezone (NITZ) can provide the PLMN name to UEs. It is proposed to extend NITZ to also provide the NPN name to UEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Capability to enable UE to receive a Human-Readable Network Name is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 3, 4, 5, 6.1 and 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 1	Scope

The present document describes the feature Network Identity and Timezone (NITZ).

This feature provides the means for serving networks to transfer current identity, time, Daylight Saving Time and the local timezone to Mobile Stations (MS)s, and for the MSs to store and use this information. This enhances roaming by permitting accurate indication of network identities that are either newer than the Mobile Equipment (ME) or have changed their name since the ME was sold. Additionally, time, Daylight Saving Time and timezone information can be utilised by MEs as desired.

* * * Next Change * * * *

# 3	Definitions and abbreviations

In addition to the following definitions, abbreviations used in the present document are listed in 3GPP TR 21.905 [1].

NITZ	The feature Network Identity and Timezone as described in the present document.

UCS2	Universal Character Set 2

UT	Universal Time

LTZ	Local Time Zone, the offset from UT applying in that locality, including any adjustments for summer time, etc.

DST	Daylight Saving Time. Adjustment for summer time.

NPN	Non-Public Network

# 4	Description

The feature Network Identities and Timezone shall make it possible for a serving network to transfer its current identity, universal time, DST and LTZ to MSs, and for the MS to store and use this information. Each one of these elements is optional. The feature significantly enhances roaming as itenables the accurate indication of network identities that are either newer than the ME or have changed their name since the ME was manufactured or sold. Additionally, time and timezone information can be utilised by MEs as desired.

When using the default character set (see 3GPP TS 23.038 [3]), the serving network shall make both a "short" and a "long" name available to the MS. As an alternative or, in addition, to the default character set, the serving network can make a name available in UCS2. The MS shall be free to choose one of these names depending upon its own characteristics and/or limitations, such as those of its display.

NOTE:	Guidance is sought, particularly from non-European operators, as to whether long and short name is required in UCS2 format.

The Network Operator may change the network identity at any time. However the change of network identity need not force immediate transfer of information to the MS.

As a network option, it shall be possible to send universal time (UT) by the network. Time information shall include: Year, Month, Day, Hour, Minute, Second, Timezone and DST. The expected accuracy of the time information is in the order of minutes.

NOTE:	Universal time indicates the time at which this information element (see 3GPP TS 24.008 [2]) may have been sent by the network. Thus it can be assumed that the accuracy of the time information  when it arrives at the MS is usually within a couple minutes.

The serving network shall make Local Time Zone (LTZ) available to the MS as an offset from Universal Time in units of 15 minutes.

When the LTZ is compensated for DST (summertime), the serving network shall provide a DST parameter to indicate this. The adjustment for DST can be +1h or +2h.

For networks which cover more than one timezone, it is assumed that the Network Operator will arrange for boundaries between subsets of the network service area to be approximately aligned with timezone boundaries. When an MS changes Local Time Zone the network is not required to immediately transfer new time zone information. Similarly, the network will transfer the LTZ changes arising from summer/winter adjustments when convenient to the network operator.

The MS will implement the new time zone information at an appropriate time following receipt.

The information passed to MSs supporting the NITZ feature is controlled by the serving network Operator through administrative interaction. The interface necessary to support this administrative interaction is outside the scope of the present document.

* * * Next Change * * * *

# 5	Applicability

Network Identity and Timezone is both an optional network feature and an optional MS feature.

The NITZ feature is not intended to replace the existing method of network Indication, nor is it intended to discharge the administration and maintenance of the associated MoU Permanent Document, SE13.

* * * Next Change * * * *

# 6	Normal procedure

## 6.1	Transfer of NITZ information

Network name, time, DST and timezone information can be transferred from the serving network to the MS:

1)	Upon registering on the network.

2)	When the MS geographically relocates to a different Local Time Zone.

3)	When the network changes its Local Time Zone, e.g. between summer and winter time.

4)	When the network changes its identity.

5)	At any time during a signalling connection with mobile station.

Transfer of relevant information shall not unduly consume scarce network resources.

## 6.2	Use of NITZ information

Relevant information shall be presented to the MS user at the earliest opportunity.

It is expected that the MS will display the most up to date information transferred to it.

Switching off the MS should not cause the updated name of the network(s) to be deleted.

UE moving between NPNs and between NPN and PLMN should not cause the updated name of the network(s) to be deleted.

Removal of the SIM/USIM should not cause the updated name of the network(s) to be deleted.

However, the number of different network identities retained in the ME is a manufacturer issue.

Usage of time information in MS is a ME manufacturer issue. For example, time information can be utilised to time stamp transactions for logging purposes.

The UE shall update the network name presented to the user when:

-	the UE moves from the coverage of one network to another, and UE receives a network name;

-	the UE moves from the coverage of one network to another, and UE does not receive a network name; or

-	the user requests NITZ information.

* * * End of Changes * * * *