3GPP TSG-SA WG Meeting #91-e	S1-203031r2

Electronic Meeting, Aug. 24- 28, 2020

Title:		Use of Multi-RATs for network slices

Agenda Item:	7.6.1 FS_EASNS

Source:	LG Electronics

Contact:	sungduck.chun@lge.com

Abstract: This contribution describes a new use case for EASNS

1. Proposal

It is proposed to agree on the following text proposal for FS_EASNS.

*****************************START of TEXT PROPOSAL (All New Text) *****************************

5.x.	Use of Multi-RATs for network slices

5.x.1	Description

5GC is connected to NG-RAN and the NG-RAN supports both E-UTRA and NR. However, for a third party who wants to utilize a network slice, the different potential and flexibility of E-UTRA and NR is not something that can easily be disregarded.

Also, the unequal possibility of radio resource slicing for NR and E-UTRA leads to a unique use case scenario.

5.x.2	Pre-conditions

Figure 5.x.2-1 shows the use case scenario where a NG-RAN is supporting both NR and E-UTRA radio resources.

Figure 5.x.2-1 Initial condition

In this figure, it is assumed that

- Subscription:

  - UE A1 and A4 have subscription to slice M.

  - UE A2 and A4 have subscription to slice N.

  - UE A3 has subscription to slice O.

- Deployment:

  - E-UTRA is configured to serve slice M, e.g. for general eMBB services, including a slice for IMS service.

  - NR is configured to serve slice N and O, e.g. for URLLC or V2X. Due to demanding requirements of V2X application, V2X application providers (e.g. OEM) requests to use only NR for slice O.

Frequency used for E-UTRA and NR can be same or different. For example, when DSS is used, same frequency is used for both E-UTRA and NR, but in different time slots.

5.x.3	Service Flows

Following is an example service flow based on the setting in previous section:

-	After power-on, UE A1, A2, A3 and A4 start searching for cells.

- 	UE A1, A2, A3 and A4 start registration in the found cells.

-	Depending on factors such as e.g., use of Dual Connectivity, priorities of network slices, different UEs may camp on different RATs. For example, for UE A1 which has subscription to only Slice M, the UE A1 may camp on E-UTRA.

- 	Based on the various inputs, for the transport of user traffic for a network slice, the RAT and radio resources can be configured. For example, when user traffic for Slice M is generated for UE A4, the network may configure E-UTRA resource for the UE to deliver the traffic. For this UE A4, when user traffic for Slice N is generated, the network may configure NR resource for the UE. When both NR and E-UTRA resource are configured, the user traffic for the network slice only flows over the allowed RATs.

-	When service policy changes, e.g. when slice M is now set to use NR instead of E-UTRA, UEs camp NR cells instead of E-UTRA cells. For a UE with ongoing active communication, handover occurs to move the UE from E-UTRA to NR.

5.x.4	Post-conditions

Following figure 5.x.4-1 shows the status at the end of service flow. For the transport of user traffic, UE A1 is served by E-UTRA, UE A2 and A3 are served by NR. UE A4 may camp on either E-UTRA or NR during Idle mode and be configured with E-UTRA, NR or both depending on the active application during Connected mode.

Figure 5.x.4-1 End result

5.x.5	Existing features partly or fully covering the use case functionality

5.x.6	Potential New Requirements needed to support the use case

Following new requirements can be derived from this use case:

- 3GPP system shall support a third party to indicate which specific RATs are allowed for a network slice.

- 3GPP system shall support to route user traffic over RATs which are allowed for the network.

- 3GPP system shall minimize signaling exchange and service interruption to a network slice when there is a change in the allowed radio resources (e.g. RATs) for the network slice.