3GPP TSG-SA WG1 Meeting #92e	S1-204096

Online, 09 November – 18 November 2020

Title:	 FS_Ranging KPI analysis on Smart Transportation Metro/Bus Validation

Agenda Item:	7.10.1

Source:	Xiaomi

Contact:	Xiaowei Jiang (jiangxiaowei@xiaomi.com)

1. Background

In last SA1 meeting, the numbers in the KPI table of Transportation Metro/Bus Validation were marked in square bracket due to concerns on the value. In this contribution, we try to analyze and justify these values, and suggest to remove the square brackets.

2. Discussion

The KPIs for Transportation Metro/Bus Validation is defined in Table 5.8.6-1 in 22.855, as cited below:

Table 5.8.6-1 – KPIs for smart transportation metro/bus validation

| Ranging distance (meters) | Ranging distance accuracy (cm) | Latency (ms) | Availability (%) | Relative UE velocity (km/h) |
| --- | --- | --- | --- | --- |
| [0.1] - [2] | < [10] | < [50] | [99] | < [3] |
| [2] - [10] | < [10] | < [50] | [95] | < [5] |

Distance accuracy:

A typical metro entrance is shown in figure below. There is one ranging module for each aisle. The ranging module can only perform ranging with devices falling within the angle of view α, which is to avoid for performing ranging with devices at backside. The threshold distance for triggering access to an aisle should be no more than 0.3+0.55=0.85m to avoid mistakenly triggering aisle access by UE at another aisle. A safer distance threshold would be 0.6m with 0.1m distance accuracy.

Availability:

99% availability will ensure a user friendly experience for smart home control.

Relative UE velocity:

The typical UE velocity is 3km/h.

Service Latency & Ranging Interval:

For 3km/h user movement, 50ms will result in 4cm distance change. Thus, 50ms service latency and ranging interval is able to ensure the distance change is well tracked.

Effective ranging distance:

Considering the 0.6m distance threshold for access, effective Ranging distance 2m would be enough.

3. Proposal

Proposal：Analysis is added to the description part. Potential new requirements and KPI table are updated based on the analysis and the square brackets are removed.

**************** First Change ******************

## 5.8	Smart Transportation Metro/Bus Validation

### 5.8.1	Description

Nowadays, heavy passenger flow shock the current public transportation system. However, passengers have to actively touch the billing device using public transportation card, which is so slow at rush time that it is easy to meet congestion. This problem can be effectively solved through the hands free metro/bus validation. It determines whether the user gets on or off the bus according to the relative distance, and then provides the information about getting on or off the bus/metro to a billing node. The billing node automatically deducts the corresponding fee from the user's account.

At rush time, crowd get close to an entrance simultaneously, and the Ranging module inside the entrance performs Ranging with lots of smart phones (e.g. 20 smart phones) at the same time. Since entrance machines are always built in a row closely, one smart phone may perform Ranging procedures with 2 or more entrances simultaneously while it appears within their effective ranging distances (e.g. ranging distance < 2m).  In this way, smart phones can distinguish which entrance is the nearest one to enter. A typical distance based metro entrance access is shown in figure below. There is one ranging module for each aisle. The ranging module can only perform ranging with devices falling within the angle of view α, which is to avoid for performing ranging with devices at backside. The threshold distance for triggering access to an aisle should be no more than 0.3+0.55=0.85m to avoid mistakenly triggering aisle access by UE at another aisle. A safer distance threshold would be 0.6m with 0.1m distance accuracy. Then, effective Ranging distance 2m would be enough.

Figure 5.8.1-1 Illustration of distance based metro entrance access

Typically, UE will move at 3km/h speed, it will result in 4cm distance change in 50ms. Thus, 50ms service latency and ranging interval is able to ensure the distance change is well tracked.

************* End of First Change ***************

**************** Second Change ******************

### 5.8.6	Potential New Requirements needed to support the use case

[PR 5.8.6-1] The 5G system shall be able to support for a UE to discover other UEs supporting Ranging.

[PR 5.8.6-1] The 5G system shall be able to provide Ranging service with following performance KPIs.

Table 5.8.6-1 – KPIs for smart transportation metro/bus validation

| Ranging scenario | Ranging Accuracy (95 % confidence level) |  | Availability | Latency | Effective Ranging distance, Coverage, NLOS/LOS, Relative UE velocity, Ranging interval, Number of concurrent ranging operation for a UE, Number of concurrent ranging operation in an area |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Distance Accuracy | Direction Accuracy |  |  | Effective Ranging distance | Coverage | NLOS/LOS | Relative UE velocity | Ranging interval | Number of concurrent ranging operation for a UE | Number of concurrent ranging operation in an area |
| Smart Transportation Metro/Bus Validation | 10cm | - | 99 % | 50ms | 2m | IC/PC/OOC | LOS | Static/ Moving (3km/h) | 50ms | - | - |

************* End of Second Change ***************