3GPP TSG-SA WG Meeting #94-e	S1-211477

Electronic Meeting, May 10-20, 2021                                   	(revision of S1-211230)

Title:	Update to FS_PALS: steering based on signal quality.

Agenda Item:	7.13.1

Source:	Qualcomm

Contact:	Lola Awoniyi-Oteri (oawoniyi at qti.qualcomm.com)

Abstract: Remove one of the proposed requirements for PALS study.

Introduction:

A new requirement that links home network steering with UE received signal strength was introduced at last SA1 meeting.  This is the subject of another potential study and should not be included in the PALS TR.  Generally speaking, the home network may not know specifics of local networks.  Also suggest to revise the other potential requirement in this use case.

-------- Start first change --------

### 5.11.5	Existing features partly or fully covering the use case functionality

The existing service requirements for roaming users assume that roaming policies are in place for UEs to use their home network services.

TS22.011, 3.2.2.8:

Steering of roaming allows the HPLMN to steer a UE to a VPLMN on which the HPLMN wants the UE to register, when the UE registers on another VPLMN. This capability may be needed for reasons e.g., reselection to a higher priority PLMN based on business arrangements.

### 5.11.6	Potential New Requirements needed to support the use case

[PR.5.11.6-1]: The 5G network shall be able to allow the home network to steer its UE(s) to a hosting network with the consideration of the location, times, coverage of the hosting network and services offered by the home network and hosting network.

-------- End first change --------