3GPP TSG-SA1 Meeting #96e	S1-214028

Electronic Meeting, 8-18 November 2021	(revision of S1-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.989 | CR | 0009 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Arbitration: automatic answer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_eFRMCS |  |  |  |  |  | Date: |  |  | 14-10-2021 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Alignment between latest revisions of UIC FRMCS Specifications and 3GPP TR 22.989 including the need to enable and disable automatic answering of communications based on user identities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Automatic answering of communications |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between 3GPP TR 22.9xx series and UIC FRMCS Specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.16.1, 9.16.3, 9.16.3.1, 9.16.3.2, 9.16.3.3, 9.16.3.4, 9.16.3.5. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************************************Change**************************************************

### 9.16.1	Introduction

In this chapter the use cases related to arbitration are defined. The following use cases are defined:

- Arbitration

- Arbitration for communication auto-connection

*************************************End of Change***********************************************

****************************************Change**************************************************

### 9.16.3	Use case: Arbitration for communication auto-connection

#### 9.16.3.1	Description

The FRMCS System shall be able to automatically accept incoming communications without asking the FRMCS User, based on configuration (i.e., configured arbitration rules).

#### 9.16.3.2	Pre-conditions

The FRMCS System is authorised to perform arbitration on communication on the FRMCS Equipment (i.e., end user device).

#### 9.16.3.3	Service flows

FRMCS User is using a single FRMCS Equipment

When the communication is received by the FRMCS User on any of its identities (including registered functional identities), the FRMCS System automatically accepts the incoming private or group communication if arbitration rules are configured to do so.

FRMCS User is using multiple FRMCS Equipment simultaneously

If arbitration rules allow configuration for automatic answering and if an FRMCS User is logged in multiple FRMCS Equimpent simultaneously, the FRMCS System automatically accepts the incoming private or group communication to the FRMCS User’s default FRMCS Equipment when the communication is received by the FRMCS User on any of its registered functional identities.

The FRMCS User’s default FRMCS Equipment is set up by either the FRMCS User or by the FRMCS System Administrator.

If the FRMCS System fails to automatically accept the incoming private or group communication to the FRMCS User’s default FRMCS Equipment, the FRMCS System automatically accepts the incoming private or group communication to another FRMCS Equipment the FRMCS User is logged in.

Arbitration rules

Arbitration rules allows configuration for automatic answering based on:

- The FRMCS User identities;

- The FRMCS User registered functional identities;

- The FRMCS Application identities;

- The FRMCS User’s default FRMCS Equipment.

#### 9.16.3.4	Post-conditions

The incoming communication is automatically accepted to the relevant FRMCS Equipment the FRMCS User is logged in.

#### 9.16.3.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-9.16.3-001] | When an incoming private communication is received by the FRMCS User, the FRMCS System shall be able to automatically accept the communication if arbitration rules are configured to do so (based on FRMCS User identities, registered functional identities or FRMCS Application types). | A | 22.280 | Partly covered by private call setup in automatic commencement mode (request for automatic commencement mode – automatic answering – is trigerred by the calling party, not by the System). In addition, configuration for automatic commencement mode is based on MC User ID only. Arbitration rules based on FRMCS Application types (i.e., MCPTT, MCData or MCVideo) is supported but not on Functional Alias(es). Editor’s note: CR is needed. |
| [R-9.16.3-002] | When an incoming group communication is received by the FRMCS User, the FRMCS System shall be able to automatically accept the communication if arbitration rules are configured to do so (based on FRMCS User identities, registered functional identities or FRMCS Application types). | A | 22.280 | Covered by affiliation and functional alias mechanisms. |
| [R-9.16.3-003] | When the FRMCS User is using multiple FRMCS Equipment simultaneously, the FRMCS System shall be able to automatically accept an incoming private communication to the default FRMCS Equipment. | A | 22.280 | Covered by 5.10 Support for multiple devices (i.e., multiple MCX UEs) and [R-6.7.2-005] default device designated by the receiving user. Editor’s note on multiple devices support: default UE covered by stages 2&3? (didn’t find it) |
| [R-9.16.3-004] | The default FRMCS Equipment is configured by either the FRMCS User or by the FRMCS System Administrator. | A | 22.280 | Partly covered by 5.10 Support for multiple devices (i.e., multiple MCX UEs) and [R-6.7.2-005]: only the receiving user can designate the default device. |
| [R-9.16.3-005] | When the FRMCS User is using multiple FRMCS Equipment simultaneously, the FRMCS System shall be able to automatically accept an incoming private communication to other FRMCS Equipment if communication to the default FRMCS Equipment fails to be established. | A | 22.280 | Not covered by [R-6.7.2-005]: it is up to the receiving user to detect that something is wrong with default UE and then to select another UE to receive incoming call. Editor’s note on multiple devices support: selection of default UE “on the fly” supported or only during MC Registration? |

*************************************End of Change***********************************************