3GPP TSG-SA1 Meeting #96e	S1-214029

Electronic Meeting, 8-18 November 2021	(revision of S1-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.989 | CR | 0010 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment between Changing conditions of Railway Emergency Alert and Merging Railway Emergency Alert |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_eFRMCS |  |  |  |  |  | Date: |  |  | 14-10-2021 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TBD |  |  |  |  |  |  |  |  |
| Summary of change: |  | TBD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TBD |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.5.1, 6.4.5.2, 6.4.5.3, 6.4.5.4, 6.4.5.5. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************************************Change**************************************************

### 6.4.5	Use case: Changing of the railway emergency alert

#### 6.4.5.1	Description

An authorised Controller or an External System is able to change the conditions selecting the appropriate recipients of the railway emergency alert. The change may result in an expansion or a reduction of FRMCS Users that shall be informed about the emergency.

#### 6.4.5.2	Pre-conditions

The FRMCS User is authorised to change the conditions of the railway emergency alert. This is managed by the application authorisation of application.

There is an ongoing railway emergency alert.

#### 6.4.5.3	Service flows

The Controller or an External System shall be able to change the conditional criteria selecting the appropriate recipients of the railway emergency alert.

The FRMCS System shall continuously check if FRMCS Users still meet the conditions of the railway emergency alert. The FRMCS User can be e.g., a mobile FRMCS User, a Driver, maintenance staff, a member of a shunting team or a Controller.

The FRMCS System shall forward the railway emergency alert to all additional FRMCS Users within a setup time specified as IMMEDIATE (see 12.10). The alert notification can include additional information (e.g., text, voice prompts) which are forwarded to the FRMCS User.

The priority of the railway emergency alert is managed by the prioritisation application. The alert has the priority that matches the application category of CRITICAL DATA (see 12.10) within the FRMCS System.

In case of an ongoing railway emergency voice communication, the additional FRMCS Users are added.

In case FRMCS User is already active in another railway emergency alert, the new alert is not forwarded by the FRMCS System to this particular FRMCS User. If the FRMCS User is no longer involved in an ongoing railway emergency alert, the waiting alert is forwarded by the FRMCS System.

The FRMCS System shall update the FRMCS Users for whom the railway emergency alert is no longer applicable and terminates the alert. The termination notification of the alert can include additional information (e.g., text, voice prompts) which are forwarded to the FRMCS User.

The priority of the railway emergency alert is managed by the prioritisation application. The alert has the priority that matches the application category of CRITICAL DATA (see 12.10) within the FRMCS System.

In case railway emergency alert conditions does not match any longer to a FRMCS User and if there is an ongoing railway emergency voice communication, the FRMCS User will remain in the ongoing railway emergency voice communication.

FRMCS System informs the Controller about the FRMCS Users who have left and have joined the railway emergency alert.

A mobile FRMCS User, e.g., a Driver, maintenance staff or a member of a shunting team, is not able to leave or terminate the Railway emergency alert.

The communication is recorded by the Data recording and access to recorded data application.

#### 6.4.5.4	Post-conditions

The appropriate recipients remain as alerted, while non-appropriate recipients are not alerted anymore.

#### 6.4.5.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.4.5-001] | If alert conditions are changed by an authorised Controller or an External System, the FRMCS System shall check if additional FRMCS Users match the conditions of the railway emergency alert. | A | 22.280 | R-6.6.4.2-002b |
| [R-6.4.5-002] | The FRMCS System shall forward the railway emergency alert to new FRMCS Users that match the conditions and inform the initiating Driver and Controller(s) accordingly. | A | 22.280 | [R-5.2.2-014] |
| [R-6.4.5-003] | The FRMCS System shall withdraw the alert from those FRMCS Users who do not match the conditions anymore and inform them and the initiating Driver and Controller(s) accordingly. | A | 22.280 | R-5.3-XXX R-5.6.2.4.1-004 R-6.8.8.4.1-006 (Using User regroup, not 5.6.2.4.1-005) R-6.6.4.1-XXX User regroup |
| [R-6.4.5-004] | The FRMCS System shall be able to forward the railway emergency alert to new FRMCS User(s) within a setup time specified as IMMEDIATE (see 12.10). | A&T | N/A | See section 12.10 below |
| [R-6.4.5-005] | Void. |  |  |  |
| [R-6.4.5-006] | The FRMCS System shall be able to forward additional information (e.g., text, voice prompts) in the railway emergency alert to new FRMCS Users. | A | 22.280 | [R-5.2.2-014] |
| [R-6.4.5-007] | For railway emergency alert the application layer priority of the communication shall be managed by the prioritisation application. | A | 22.280 | 5.4.2-XXX 5.6.X, 6.8.7.X |
| [R-6.4.5-008] | The FRMCS System shall be able to prevent new FRMCS User(s), other than a Controller, involved in an ongoing railway emergency alert, to receive an additional railway emergency alert. In this case when the ongoing railway emergency alert has ended the FRMCS System shall forward the railway emergency alert to the new FRMCS User. | A | 22.280 | 5.4.2-XXX [R-5.4.2-007a] |
| [R-6.4.5-009] | Void. |  |  |  |

*************************************End of Change***********************************************