3GPP TSG-SA1 Meeting #96e	S1-214032

Electronic Meeting, 8-18 November 2021

Title:	Clean up references

Agenda Item:	7.5.1

Source:	UIC

Contact:	Guillaume Gach gach@uic.org

Abstract: Some references are wrongly identified. This contribution proposes to clean up those references for Rail Off-Network TR 22.990.

--------------------------------------- Start of Change ----------------------------------------

#### 5.2.6.2	Requirements related to the Transport layer

Editor’s note : the following potential new requirements are relevant for SA2 and RAN.

[PR 5.2.6.2-1] The FRMCS System shall support the following traffic characteristics of data transfer for data communication for Trackside Maintenance Warning System:

Note:	This table is intended to be aligned with TS 22.289 table 5.2.2-2 [2]

| Scenario | End-to-end latency | Reliability (Note 1) | UE speed | User experienced data rate (UL and DL) | Payload size (Note 2) | Area traffic density (UL and DL) | Overall UE density | Communication range (note 6) | Service area dimension (note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Trackside maintenance warning system Communication | ≤500 ms | 99,9999% (note 7) | Stationary (warning system entities) and pedestrians (workers). Option: ≤500kmph for train detection via onboard system | 10 kbps up to 500 kbps | Small to medium | Up to 10 Mbps/km | ≤80/worksite | ≥8.5 km along rail tracks including tunnels, bad weather conditions and unfavourable geographical conditions (note 4) | ≥12 km along rail tracks including tunnels, bad weather conditions and unfavourable geographical conditions (note 5) |
| NOTE 1: Reliability as defined in TS 22.289 sub-clause 3.1. NOTE 2: Small: payload ≤ 256 octets, Medium: payload ≤512 octets; Large: payload 513 -1500 octets. NOTE 3: Estimates of maximum dimensions. NOTE 4: Distance is equal to the distance for train of maximum speed of 500 kmph to enter the worksite + the maximum distance of a worksite NOTE 5: Minimum distance of the warning zone NOTE 6: Supported via a single or a combination of 3GPP capabilities of 5GS that best serve these use case in areas with no FRMCS RAN nodes/no FRMCS network coverage. NOTE 7: Reliability figures could be achieved using multiple 3GPP 5GS-compatible spectrum bands simultaneously. |  |  |  |  |  |  |  |  |  |

Table 5.2.6.2-1: Traffic characteristics for Trackside Maintenance Warning System

--------------------------------------- End of Change ----------------------------------------

--------------------------------------- Start of Change ----------------------------------------

#### 5.3.6.2	Requirements related to the Transport layer

Editor’s note : the following potential new requirements are relevant for SA2 and RAN.

[PR 5.3.6.2-1] If the FRMCS Equipment is served by a 3GPP RAT, using NR for direct communication for Remote Control of Engines shall not interfere with on-network services.

[PR 5.3.6.2-2] The FRMCS System shall support the following traffic characteristics for communication for Remote Control of Engines:

Note:	This table is intended to be aligned with TS 22.289 table 5.2.2-2 [2]

| Scenario | End-to-end latency | Reliability (Note 1) | UE speed | User experienced data rate (UL and DL) | Payload size (Note 2) | Area traffic density (UL and DL) | Overall UE density | Communication range (note 4) | Service area dimension (note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Remote control of engines data communication | ≤10 ms | 99,9999% (note 4) | Stationary (trackside remote driver) and ≤40kmph (onboard remote control system and onboard remote driver) | 100 kbps up to 1 Mbps | Small to Medium | Up to 100 Mbps/km | 2 (remote driver and onboard system) | ≤1000m along rail tracks including depots and tunnels with clear line of sight | ≤2000m along rail tracks including depots and tunnels with clear line of sight |
| Remote control of engines video communication | ≤100 ms | 99,9% (note 4) | Stationary (trackside remote driver) and ≤40kmph (onboard remote control system and onboard remote driver) | 10 Mbps | Medium | Up to 1 Gbps/km | 2 (remote driver and onboard system) | ≤1000m along rail tracks including depots and tunnels with clear line of sight | ≤2000m along rail tracks including depots and tunnels with clear line of sight |
| NOTE 1: Reliability as defined in TS 22.289 sub-clause 3.1. NOTE 2: Small: payload ≤ 256 octets, Medium: payload ≤512 octets; Large: payload 513 -1500 octets. NOTE 3: Estimates of maximum dimensions. NOTE 4: Supported via a single or a combination of 3GPP capabilities of 5GS that best serve these use case in areas with no FRMCS RAN nodes/no FRMCS network coverage. NOTE 5: Reliability figures could be achieved using multiple 3GPP 5GS-compatible spectrum bands simultaneously |  |  |  |  |  |  |  |  |  |  |

Table 5.3.6.2-1: Traffic characteristics for Remote Control of Engines

--------------------------------------- End of Change ----------------------------------------

--------------------------------------- Start of Change ----------------------------------------

#### 5.7.6.2	Requirements related to the Transport layer

[PR 5.7.6.2-1] The FRMCS Service in Off-Network mode shall support the following traffic characteristics of data transfer for direct communication for autonomous train control and operation:

| Scenario | End-to-end latency | Reliability (Note 1) | UE speed | User experienced data rate (UL and DL and SL) | Payload size (Note 2) | Area traffic density (UL and DL and SL) | Overall UE density | Communication range | Service area dimension (Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Autonomous train control and operation data communication (Korea, urban railway) | ≤100 ms | 99.99 % | ≤100 km/h | ≤1 Mbps | Small to large | ≤5 Mbps/km | ≤15 (3000 m) | ≤3000 m (Note 5) | ≤3000 m along rail tracks including bad weather conditions (Note 4) |
|  | NOTE 1: Reliability as defined in TS 22.289 sub-clause 3.1. NOTE 2: Small: payload ≤ 256 octets, Medium: payload ≤512 octets; Large: payload 513 -1500 octets. NOTE 3: Estimates of maximum dimensions. NOTE 4: Non-Line-of-Sight (NLOS) between UEs shall be supported. NOTE 5: UEs are assumed to be located at the tail of the leading train and the front of the following train |  |  |  |  |  |  |  |  |

Table 5.7.6.2-1: Traffic characteristics for autonomous train control and operation communication – off network communication

Note:	This table is intended to be included in Section 6.2.2.3-1 of TS 22.289 [2]

--------------------------------------- End of Change ----------------------------------------

--------------------------------------- Start of Change ----------------------------------------

#### 5.8.6.2	Requirements related to the Transport layer

[PR 5.8.6.2-1] The FRMCS system shall be able to provide integrity protection for off-network train control communication

[PR 5.8.6.2-2] The FRMCS Service in Off-Network mode shall support the following traffic characteristics of data transfer for direct communication for virtual coupling:

Note:	This table is intended to be included in Section 6.2.2.3-1 of TS 22.289 [2]

| Scenario | End-to-end latency | Reliability (Note 1) | UE speed | UE Relative Speed | User experienced data rate (UL and DL and SL) | Payload size (Note 2) | Area traffic density (UL and DL and SL) | Overall UE density | Communication range | Service area dimension (Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Virtual coupling, critical data communication (Korea, urban railway) | ≤ 100 ms | 99.99% | ≤100 km/h | ≤ 50km/h | ≤1Mbps | Small to large | ≤5 Mbps/km | ≤15 (3000 m) | ≤3000 m (Note 6) | ≤3 000 m along rail tracks including bad weather conditions |
| Virtual coupling, Very critical data communication (Korea, urban railway) (Note 5) | ≤ 10 ms | 99.9999% | ≤100 km/h | ≤ 50km/h | ≤1Mbps | Small to large | ≤20 Mbps/km | ≤6 (300 m) | ≤ 300 m (Note 6) | ≤300 m along rail tracks including bad weather conditions (Note 4) |
|  | NOTE 1: Reliability as defined in TS 22.289 sub-clause 3.1. NOTE 2: Small: payload ≤ 256 octets, Medium: payload ≤512 octets; Large: payload 513 -1500 octets. NOTE 3: Estimates of maximum dimensions. NOTE 4: Non-Line-of-Sight (NLOS) between UEs shall be supported. NOTE 5: Data link between trains can work as like wireless train backbone NOTE 6: UEs are assumed to be located at the tail of the leading train and the front of the following train |  |  |  |  |  |  |  |  |  |  |

Table 5.8.6.2-1: Traffic characteristics for Virtual coupling communication

[PR 5.8.6.2-3] The FRMCS System shall establish an immediate communication session between entities for virtual coupling.

Editor’s note: The requirement above is FFS

[PR 5.8.6.2-4] The FRMCS System shall support seamless service continuity between on-network based connection and off-network based connection.

Editor’s note: The requirement above is FFS

[PR 5.8.6.2-5] The FRMCS System shall support UEs capable of utilizing off-network and on-network communications at the same time.

Editor’s note: The requirement above is FFS

--------------------------------------- End of Change ----------------------------------------