3GPP TSG-SA1 Meeting #96e	S1-214289

Electronic Meeting, 8 – 18 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.847 | CR | 0012 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of editorial issues |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_TACMM |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The figure in 5.1.1 lacks a caption. - Wrong spelling in 5.1.6. - Wrong number of potential requirements in 5.5.6 and 5.6.6. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a caption for figure in 5.1.1 Correct the spelling in 5.1.6 Correct the number of PRs in 5.5.6 and 5.6.6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The editorial issues may lead to misunderstandings. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------- The 1st change ---------

# 5	Use case

## 5.1	Immersive multi-modal Virtual Reality (VR) application

### 5.1.1	Description

Immersive multi-modal VR application describes the case of a human interacting with virtual entities in a remote environment such that the perception of interaction with a real physical world is achieved. Users are supposed to perceive multiple senses (vision, sound, touch) for full immersion in the virtual environment.  The degree of immersion achieved indicates how real the created virtual environment is. Even a tiny error in the preparation of the remote environment might be noticed, as humans are quite sensitive when using immersive multi-modal VR applications. Therefore, a high-field virtual environment (high-resolution images and 3-D stereo audio) is essential to achieve an ultimately immersive experience.

One of the major objectives of VR designers and researchers is to obtain more realistic and compelling virtual environments. As the asynchrony between different modalities increases, users’ sense of presence and realism will decrease. There have been efforts (since 1960s or even earlier) in multi-modal-interaction research regarding the detection of synchronisation thresholds. The obtained results vary, depending on the kind of stimuli and the psychometric methods employed. Hirsh and Sherrick measured the synchronisation thresholds regarding visual, auditory and tactile modalities [23].

Figure 5.1.1-1 The synchronisation thresholds regarding visual, auditory and tactile modalities measured by Hirsh and Sherrick

M.E. Altinsoy and co. believe the audio-tactile synchronization has to be at least within an accuracy of ±40 ms [22]. More results have been reported based on extensive theoretical and experimental efforts. [24] further indicated the perceptual threshold values were 50 ms for the audio lag and the 25 ms for audio lead.

As to the visual-tactile synchronisation threshold, Massimiliano Di Luca and Arash Mahnan provided test results in [25] that indicate that none of the participants could reliably detect the asynchrony if haptic feedback was presented less than 50ms after the view of the contact with an object. The asynchrony tolerated for haptic before visual feedback was instead only 15ms.

--------- Next change ---------

### 5.1.6	Potential New Requirements needed to support the use case

[PR 5.1.6-1] The 5G System shall provide the network connection to address the KPIs for immersive multi-modal VR applications, see table 5.1.6-1.

Table 5.1.6-1 – Potential key performance requirements for immersive multi-modality VR applications

| Use Cases | Characteristic parameter (KPI) |  |  | Influence quantity |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability | Message size (byte) | # of UEs | UE Speed | Service Area |  |
| Immersive multi-modal VR (UL: device application sever) | 5 ms (note 2) | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | [99.9%] (without haptic compression encoding) [99.999%] (with haptic compression encoding) | 1 DoF: 2-8 3 DoFs: 6-24 6 DoFs: 12-48 More DoFs may supported by the haptic device | - | Stationary or Pedestrian | several km2 (note 3) | Haptic feedback |
|  | 5 ms | < 1Mbit/s | [99.99%] | MTU | - | Stationary or Pedestrian | several km2 (note 3) | Sensor information e.g. user position and view |
| Immersive multi-modal VR (DL: application sever device) | 10 ms (note1) | 1-100 Mbit/s | [99.9%] | 1500 | - | Stationary or Pedestrian | several km2 (note 3) | Video |
|  | 10 ms | 5-512 kbit/s | [99.9%] | 50 | - | Stationary or Pedestrian | several km2 (note 3) | Audio |
|  | 5 ms (note 2) | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | [99.9%] (without haptic compression encoding) [99.999%] (with haptic compression encoding) | 1 DoF: 2-8 3 DoFs: 6-24 6 DoFs: 12-48 | - | Stationary or Pedestrian | several km2 (note 3) | Haptic feedback |
| NOTE 1: Motion-to-photon delay (the time difference between the user’s motion and corresponding change of the video image on display) should be less than 20ms, the communication latency for transfer the packets of one audio-visual media is less than 10ms, e.g. the packets corresponding to one video/audio frame are transferred to the devices within 10ms. NOTE 2: Refer to IEEE 1918.1 [3] as for haptic feedback, the latency should be less than 25ms for accurately completing haptic operations. As rendering and hardware introduce some delay, the communication delay for haptic modality should be reasonably less than 5ms, i.e. the packets related to one haptic feedback are transferred to the devices within 10ms. NOTE 3: In practice, the service area can vary depends on the actual deployment. In some cases a local approach (e.g. the application servers are hosted at the network edge) is preferred in order to satisfy the requirements of low latency and high reliability. |  |  |  |  |  |  |  |  |

Editor's Note: Service Area is FFS.

[PR 5.1.6-2] The 5G system shall enable means to meet a synchronization threshold for flows of multiple UEs associated with an application based on input received from an authorized 3rd party.

Editor's Note: this requirement is FFS.

[PR 5.1.6-3] To support immersive multi-modal VR applications, the 5G system shall support the following synchronisation thresholds in order to avoid having a negative impact on the user experience (i.e. viewers detecting lack of synchronisation).

Table 5.1.6-2: Potential Key performance requirements for synchronization thresholds for immersive multi-modality VR applications

|  | synchronisation threshold |  |
| --- | --- | --- |
| audio-tactile | audio delay: [50 ms] | tactile delay: [25 ms] |
| visual-tactile | visual delay: [15 ms] | tactile delay: [50 ms] |

Editor's Note: this requirement is FFS.

--------- Next change ---------

5.5.6	Potential New Requirements needed to support the use case

[PR. 5.5.6-1] The 5G network shall support a mechanism to allow an authorized 3rd party to provide QoS policy for flows of multiple UEs associated with an application. The policy may contain e.g. the expected 5GS handling and the associated triggering event.

[PR. 5.5.6-2] The 5G system shall support a mechanism to apply QoS policy for flows of multiple UEs associated with an application received from an authorized 3rd party.

[PR 5.5.6-3] The 5G system shall provide a network connection to address the KPIs for immersive multi-modal navigation applications, see Table 5.5.6-1.

--------- Next change ---------

5.6.6	Potential New Requirements needed to support the use case

[PR 5.6.6-1] The 5G system shall support a mechanism to allow an authorized 3rd party to provide QoS policy for flows of multiple UEs associated with an application. The policy may contain e.g. the expected 5GS handling and the associated triggering event.

[PR 5.6.6-2] The 5G system shall support a mechanism to apply QoS policy for flows of multiple UEs associated with an application received from an authorized 3rd party.

--------- End of changes ---------