3GPP TSG-SA WG1 Meeting #96-e 	S1-21xxxx

Electronic Meeting, 08-18 Nov 2021	(revision of S1-21xxxx)

Source:	Huawei

pCR Title:	FS_TACMM resolving the ENs in clause 5.1

Draft Spec:	3GPP TR 22.847 V18.0.0

Agenda item:	7.11.1

Document for:	Approval

Contact:	Alice Li, alice.li1@huawei.com

Abstract: There are several editor’s notes in clause 5.1 “Immersive multi-modal Virtual Reality (VR) application”. This document discusses the remaining issues and proposes the way forward to resolve these ENs.

Discussion

There are several editor’s notes in clause 5.1 “Immersive multi-modal Virtual Reality (VR) application”.

Service Area

The service area for the immersive multi-modal VR application is set as “several km2” with a note:

NOTE 3:	In practice, the service area can vary depends on the actual deployment. In some cases a local approach (e.g. the application servers are hosted at the network edge) is preferred in order to satisfy the requirements of low latency and high reliability.

Considering the end-to-end latency requirement of one way < 5ms, it is suggested to update the service area to “typically < 100 km2”

Proposal 1: To update the service area as “typically < 100 km2”.

Requirements related to “synchronization threshold”

There are a couple of ENs related to the synchronization threshold. During the course of the consolidation, it seems acceptable to update the related requirements as following.

Proposal 2:

Service exposure –

The 5G system shall support a means for an authorized 3rd party to provide 5GS with synchronization threshold(s) between the multiple flows (e.g., haptic, audio and video) associated with a multi-modal communication session.

The 5G system shall support a means to assist a 3rd party application to coordinate the transmission of multiple flows (e.g., haptic, audio and video) of a multi-modal communication session to enable presenting the related tactile and multi-modal data to the user within a certain time.

Performance KPIs –

To support immersive multi-modal VR applications, the 5G system shall support the following synchronisation thresholds in order to avoid having a negative impact on the user experience (i.e. viewers detecting lack of synchronisation).

Table: Potential Key performance requirements for synchronization thresholds for immersive multi-modal VR applications

|  | synchronisation threshold |  |
| --- | --- | --- |
| audio-tactile | audio delay: [50 ms] | tactile delay: [25 ms] |
| visual-tactile | visual delay: [15 ms] | tactile delay: [50 ms] |

A CR is provided to update the TR accordingly.