3GPP TSG-SA WG1 Meeting #96e 	S1-214037r1

Electronic Meeting, 8 – 18 Nov. 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.874 | CR | 0003 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FS_AMMT update to CPR tables with new format |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMMT |  |  |  |  |  | Date: |  |  | 2021-11-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Update the CPR table in the TR to align with the new requirements of SA1 TR format. Add some newly agreed CPRs using the same table |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CPRs in the table are not changed. Only the format of the table is updated. Add some newly agreed CPRs (i.e. from 4079r1 and 4081r1) using the same table |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The format of the CPR table is not aligned with the required format. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the Change---

8.2	Functional requirements for AMMT services

Editor’s Note: 	This section summarizes the available requirements from the various approved use cases based on v18.1.0. The requirements in the tables may be corrected/modified aligned with the newly-added use-cases and the updates to the approved use-cases.

Table 8.2-1 Functional requirements for AMMT services

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 8.2-1 | Based on operator policy, the 5G network shall provide the means to allow an authorized third-party to monitor the resource utilisation of the network service that is associated with the third-party. NOTE: Resource utilization in the preceding requirement refers to measurements relevant to the UE’s performance such as the data throughput provided to the UE. | PR.5.5-001 |  |
| CPR 8.2-2 | Based on operator policy, the 5G system shall be able to provide an indication about a planned change of bitrate, latency, or reliability for a QoS flow to an authorized 3rd party so that the 3rd party AI/ML application is able to adjust the application layer behaviour if time allows. The indication shall provide the anticipated time and location of the change, as well as the target QoS parameters. | PR.5.5-002 |  |
| CPR 8.2-3 | Based on operator policy, 5G system shall be able to provide the means to predict (to the extent possible) and expose predicted network condition changes (i.e. bitrate, latency, reliability) per UE to the authorized third party. | PR.5.5-003 |  |
| CPR 8.2-4 | Subject to user consent, operator policy and regulatory constraints, the 5G system shall support a mechanism to expose monitoring and status information of an AI-ML session, (e.g. measured data rate/delay and other traffic analytics information), to a 3rd party AI/ML application. NOTE: such mechanism is needed for AI/ML application to determine an in-time transfer of AI/ML model | PR.6.4-001 PR.6.7-001 | The two PRs are proposing to make 5GS provide information including measured data rate, delay, analytics result, and prediction for communication to 3rd party, for AI model downloading and training. It is proposed to merge the two PRs |
| CPR 8.2-5 | 5G system shall provide a means to supply event alerting to an authorized 3rd party, together with a predicted time of the event. (e.g., alerting about traffic congestion or UE moving into/out of a different geographical area). NOTE: A 3rd party AI/ML application may use the prediction information to minimize disturbance in the transfer of learning data and AI/ML model data. | PR.7.3-003 | It is proposed to adopt it with the modification “learning agent –> 3rd party” and rewording |
| CPR 8.2-6 | The 5G system shall be able to support an authorised 3rd party to change aggregated QoS parameter values associated with a group of UEs, e.g. UEs of a FL group. | PR 7.4-001 |  |
| CPR 8.2-7 | Subject to user consent, operator policy and regulatory requirements, the 5G system shall be able to expose information to an authorized 3rd party to support the 3rd party to determine members of a group of UEs, e.g. UEs of a FL group, based upon criteria provided in the request from the 3rd party. | PR 7.4-001 |  |
| CPR 8.2-8 | The 5G system shall be able to expose aggregated QoS parameter values for a group of UEs to an authorized service provider. | PR.7.4-002 PR.7.4-003 PR.7.4-004 | It is proposed to adopt the three potential requirements into CPRs |
| CPR 8.2-9 | The 5G system shall be able to support collection of charging information for a group of UEs, e.g. UEs of a AI/ML FL group. | PR 7.4-004 | The CPR is changed from the original PR to support the charging for FL group where the member may be dynamically changed. |

## ---End of the Change---