3GPP TSG SA WG 1 Meeting #99e 	S1- 222183

Electronic Meeting, 22 August – 1 September 2022

Source:	China Mobile

pCR Title:	pCR 22.882 - New use case of reuse location information for PLMN and NPN of the same operator to save energy

Draft Spec:	3GPP TR 22.882-000

Agenda item:	7.12

Document for:	Approval

Contact:	Xiaonan Shi  shixiaonan@chinamobile.com

Abstract: This contribution proposes a new use case for reuse location information for different PLMN and NPN under the same operator. 5GS can reduce the number of location calculation times and save energy efficiency in system level.

* * * First Change * * * *

6.Y	Reuse location information to reduce location calculation times to save energy efficiency in system level

6.y.1	Description

Application sends UE location request to 5GS. AMF select a LMF, then LMF will calculate the UE location based on distance information measured by several RANs. LMF reports the information to GMLC, and AF. This procedure cost several signalling message and energy consumption. So if 5GS can reuse location information as much as possible, 5G network can save energy in system level.

UE is a personal user of sim 1 belong to PLMN A, and is also a vertical user of sim 2 belong to NPN B. PLMN A and NPN B belong to the same operator and share the same 5G system. Application x requires for UE location of sim 1. Application y requires for UE location of sim 2. The 5G system can aware that sim 1 and sim 2 are in the same equipment and can share location information. 5GS can send UE location based on sim 1 information and reduce times of location calculation.

SIM 1

SIM 2

5GS

PLMN A

NPN B

App x

App y

6.y.2	Pre-conditions

Yuzu has one terminal equipment with two sim card. Sim 1 is personal user of PLMN A, while sim 2 is vertical user of NPN B. PLMN A and NPN B belong to the same operator and share the same network infrastructure. Yuzu is in the enterprise campus where PLMN A and NPN B both can provide network service. He interact with app x through PLMN A, and interact with app y through NPN B.

6.y.3	Service Flows

1.App x requires for UE location of sim 1 to PLMN A.

2.PLMN A trigger procedure of location calculation and response result to app x. 5GS stores result of UE location, validity based on user calculation accrancy.

3.App y requires for UE location of sim 2 to NPN B. 5GS aware that sim 1 and sim 2 has the same location information because they are in the same terminal equipment. 5GS checks location of sim 1 from location management network function (e.g. GMLC). 5GS finds the valid result and return to app y without calculate UE location again.

4.App y receive the location of sim 2.

6.y.4	Post-conditions

App x and app y both get the location information of Yuzu with only one-time location calculation, and provides application service to Yuzu.

6.y.5	Existing features partly or fully covering the use case functionality

none

6.y.6	Potential New Requirements needed to support the use case

[PR 6.Y.6-1] The 5G system shall provide the capability of UE location information sharing between PLMN and NPN of the same operator.

* * * End of Changes * * * *