TSG-SA Working Group 2 meeting #3	TSGS2(99)107

Nynäshamn 15-19 March 1999

Agenda item: common TSG-S2 + RAN WG3

Agenda Item: 	23.20, Key Issue: MM and Iu

Source: 	Alcatel

Title: 	Principles of SRNS relocation

Document for:	23.20 Change

# Introduction

The aim of this document is to put in 23.30 and 23.20 some clarifications on the principles to be used for SRNS relocation procedures.

# Discussion

To be able to carry out SRNS relocation only when this procedure has the least adverse effect on user traffic, and as it is not the new SRNC but the old SRNC that knows the current services of an user, only the old SRNS can decide to launch a SRNS relocation procedure.

The SRNC relocation procedures shall ensure that there is only one Serving RNC for an user even if this user has services through more than one (IP or ISDN) domain.

The SRNS relocation procedure is split in 2 parts. In the first one resources are reserved on the new Iu interfaces and (if needed) inside the CN between anchor and drift MSC. Only when this first part has been successfully carried out for all domains on which thye user has currently some services, the old SRNS can launch the second part i.e. hand-over the role of SRNC to the new SRNC.

# Proposal

It is proposed that the content of section 2 of this Tdoc is included in (new) section 7.3.11.2.0 chapter of 23.20 (on "SRNS relocation principles") and that it is also included in a new section 4.2.i of 23.30 (to be created) on "SRSNS relocation"

# References

1	UMTS 23.20, version 1.5.0

[2]	UMTS 23.30 v1.1.0