SA WG2 Meeting #107	S2-150497

26 - 30 January 2015, Sorrento, Italy	(revision of S2-150146, S2-150454)

Source:	China Telecom, Intel

Title:	Architectural Assumption for Roaming Scenario

Document for:	Approval

# Agenda Item:	7.17

# Work Item / Release:	FMSS/Rel-13

Abstract of the contribution: Proposes architectural assumptions for roaming scenario.

Discussion

The following requirement is excerpted from clause 30.2 of TS 22.101:

-	In case of roaming, the HPLMN shall be able to apply traffic steering policies for home routed traffic.

Note: Traffic steering policies for local breakout is not specified.

The requirement of traffic steering policies for home routed is specified, but the traffic steering policies for local breakout haven’t been specified yet.

Subject to HPLMN operator’s policy and the prior agreements between PLMN’s, the VPLMN may use its own traffic steering policies in the case of local breakout for roaming users.

Proposal

It is proposed to add the following text for architecture assumption for roaming scenario.

* * * * Start of 1st Change * * * *

4	Architectural Assumptions

### 4.1	Traffic steering policy generator – PCRF

In order to realize traffic steering in the operator deployed (S)Gi-LAN, the network operator uses various parameters, related to the subscriber session and the application traffic, to define traffic steering policies. These policies are used to control the steering of the subscriber’s traffic to appropriate service enablers (e.g. NAT, antimalware, parental control, DDoS protection) in the (S)Gi-LAN.

The PCRF is extended to support generation of traffic steering policy for the support of flexible service steering in the (S)Gi-LAN. This will ensure a central function controlling all the subscriber related policies within the 3GPP core network architecture. In the home routed scenario, H-PCRF shall be able to generate the traffic steering policy for home routed traffic.

Editor’s Note: The traffic steering in local-breakout scenario is FFS.

The PCRF may use one or more pieces of information from the IP-CAN session, subscription information and network operator's policies as input for generation of traffic steering policies.

* * * * End of Changes * * * *