3GPP TSG-SA2 Meeting #111	S2- 153156

# Chengdu, P.R. China, 19 - 23 October 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.246 | CR | 0397 | rev | - | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cell ID in MBMS bearer context table for EPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MBMS_enh_st2?? |  |  |  |  |  |  | Date: |  | 2015-10-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | CR 0296 added the possibility to send cell ID list to support granularity smaller than the entire MBMS service area. The MBMS bearer context table (Table 3 in section 6.2) needs to be updated as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the liste of cell IDs to table 3 as part of the MBMS bearer context |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment within specification – create confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | - |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

############################# FIRST CHANGE START #############################

## 6.2	MBMS Bearer Context

The MBMS Bearer Context, which is referred to as MBMS Service Context in RAN, contains all information describing a particular MBMS bearer service and is created in each node involved in the delivery of the MBMS data.

In MBMS multicast mode a MBMS Bearer Context is created in the SGSN and GGSN when the first MBMS UE Context is created in the node or when a downstream node requests it. The MBMS Bearer Context is statically configured in the BM-SC Proxy and Transport Function; how this is done is out of the scope of this specification. The MBMS Bearer Context is created in the Iu mode BSC and in SRNC when a first MBMS UE Context is created in BSC/SRNC. MBMS Session Start procedure may create MBMS Bearer Context in a BSC/RNC which has no MBMS Bearer Context yet.

In MBMS broadcast mode, an MBMS Bearer Context is created in the MME/SGSN, MBMS GW/GGSN and RAN as a result of the MBMS Session Start procedure.

An MBMS Bearer Context, once created, can be in one of two states reflecting the bearer plane resource status of the corresponding MBMS bearer service.

Figure 6: MBMS Bearer Context State Model

'Active' reflects the state of an MBMS Bearer Context in which bearer plane resources are required in the network for the transfer of MBMS data. This state is maintained as long as there is a corresponding MBMS session ongoing.

'Standby' reflects the state of an MBMS Bearer Context in which no bearer plane resources are required in the network for the transfer of MBMS data. This state is maintained as long as there is no corresponding MBMS session ongoing.

In MBMS broadcast mode, the MBMS Bearer Context is always implicitly in state 'active' (since it is created as a result of an MBMS Session Start procedure) and does not use the MBMS Bearer Context State Model in Figure 6.

The content of the MBMS Bearer Context is described in Table 2.

Table 2: MBMS Bearer Context for GPRS

| Parameter | Description | RAN | SGSN | GGSN | BM-SC |
| --- | --- | --- | --- | --- | --- |
| Multicast/broadcast mode | MBMS bearer service in broadcast or multicast mode | X | X | X | X |
| IP multicast address( multicast mode only) | IP multicast address identifying the MBMS bearer described by this MBMS Bearer Context. | X | X | X | X |
| APN (multicast mode only) | Access Point Name on which this IP multicast address is defined. | X | X | X | X |
| TMGI | Temporary Mobile Group Identity allocated to the MBMS bearer service. | X | X | X | X |
| Flow Identifier (broadcast mode only) | Location dependent subflow of the MBMS bearer service. When present, the Flow Identifier together with the TMGI uniquely identify the MBMS Bearer Context. |  | X (note 1) | X (note 1) | X (note 1) |
| GGSN TEID-C | Tunnel Endpoint Identifier of GGSN for control plane. |  | X |  |  |
| GGSN IP Address for Control Plane in use | The IP address of the GGSN used for the control plane. |  | X (note 4) |  |  |
| State | State of bearer plane resources ('standby' or 'active') | X | X | X | X |
| Required MBMS Bearer Capabilities (multicast mode only) | Minimum bearer capabilities the UE needs to support |  | X | X | X |
| QoS | Quality of Service required for the MBMS bearer service. | X | X | X | X |
| MBMS Service Area | Area over which the MBMS bearer service has to be distributed. | X | X | X | X |
| List of downstream nodes | List of downstream nodes that have requested the MBMS bearer service and to which notifications and MBMS data have to be forwarded. |  | X (note 5) | X (notes 3, 4,6) | X |
| Number of UEs (multicast mode only) | Number of UEs hosted by the node that have joined the multicast MBMS bearer service. |  | X | X |  |
| List of PMM-CONNECTED UEs | List of PMM-CONNECTED UEs which have activated an MBMS service. | X2) |  |  |  |
| List of RAs (multicast mode only) | List of RAs, each of which contains at least one UE that has joined the MBMS bearer service. | X1) | X1) |  |  |
| IP multicast and Source address for distribution | IP addresses identifying the SSM channel used for user plane distribution on the backbone network | X | X | X |  |
| MBMS HC indicator | Flag set by BM-SC if it is using compressed header for the payload. |  | X | X | X |
| NOTE 1: It is an optional parameter. NOTE 2: It is available only for UTRAN, not for GERAN. NOTE 3: For GGSN, the list at least includes the couples of the SGSN IP addresses and TEIDs for control plane, and the couples for user plane. NOTE 4: GSN that supports both IPv4 and IPv6 address types, stores only the IP address in use. NOTE 5: For SGSN, the list includes the registered DRNC. NOTE 6: The GGSN shall include SGSN Address for user data and TEID-U for downstream SGSNs that do not accept the IP multicast backbone distribution. |  |  |  |  |  |

Table 3: MBMS Bearer Context for EPS (E-UTRAN/UTRAN) broadcast mode only

| Parameter | Description | RAN | MME/ SGSN | MBMS-GW | BM-SC |
| --- | --- | --- | --- | --- | --- |
| MBMS GW TEID-C | Tunnel Endpoint Identifier of MBMS GW for control plane. |  | X |  |  |
| TMGI | Temporary Mobile Group Identity allocated to the MBMS bearer service. | X | X | X | X |
| Flow Identifier | Location dependent subflow of the MBMS bearer service. When present, the Flow Identifier together with the TMGI uniquely identify the MBMS Bearer Context. |  | X (note 1) | X (note 1) | X (note 1) |
| MBMS GW IP Address for Control Plane in use | The IP address(es) of the MBMS GW used for the control plane. |  | X (note 2) |  |  |
| MBMS GW IP Address for User Plane in use | The IP address of the MBMS GW used for the user plane. |  | X (note 2) |  |  |
| C-TEID | Common Tunnel Endpoint Identifier of MBMS GW for user plane | X |  | X |  |
| QoS parameters | Quality of Service required for the MBMS bearer service. | X | X | X | X |
| MBMS Service Area | Area over which the MBMS bearer service has to be distributed. | X | X | X | X |
| List of downstream nodes | List of downstream nodes that have requested the MBMS bearer service and to which notifications have to be forwarded. |  | X (note 3) | X (note 4) | X |
| IP multicast address(es) and IP Source address(es) for distribution | IP addresses identifying the SSM channel used for user plane distribution on the backbone network. The IP multicast address and paired IP address of the multicast source shall be of the same IP type. | X | X (note 6) | X (note 6) |  |
| MBMS HC indicator | Flag set by BM-SC if it is using compressed header for the payload. |  | X (note 5) |  | X (note 5) |
| SGSN IP Address and TEID for User Plane over Sn-U | The IP address and TEID of SGSN used for the user plane over Sn-U when some RNCs have not accepted IP multicast distribution. |  |  | X |  |
| List of cell ID(s) | Cell(s) for which the MBMS bearer service has to be distributed. | X |  |  | X |
| NOTE 1: It is an optional parameter. NOTE 2: An MBMS GW that supports both IPv4 and IPv6 address types, stores both IP addresses. NOTE 3: For SGSN, the list includes the registered DRNC. NOTE 4: For MBMS GW, the list includes the couples of the SGSNs and MMEs IP addresses and TEIDs for control plane. NOTE 5: Header Compression is only supported for UTRAN for this Release. NOTE 6: An MBMS-GW or an MME that supports IPv4 and IPv6 M1 multicast address types stores both IPv4 and IPv6 pairs of source and multicast addresses. The alternative IP multicast address and paired IP address of the multicast source are of the same IP type. |  |  |  |  |  |

########################### CHANGE END ############################