SA WG2 Meeting #112	S2-153861

16 - 20 November 2015, Anaheim, California, USA	(revision of S2-15xxxx)

Source:	Telekom Deutschland

Title:	Discussion on solution 6.3 Determination of PLMN-ID via HSS interaction

Document for:	 Discussion

Agenda Item:	6.6

Work Item / Release:	FS_V8/Rel-14

Abstract of the contribution: For the solution to retrieve the VPLMN ID via HSS some open topics might need more description. It would be desirable to consider these in the proposed solution description and describe solutions for it.

# Discussion

The first topic for discussion addresses the dual attach and the solution where location information is retrieved from HSS (Steps 0, 5):

Terminals might apply dual attach to the mobile network. In parallel there might be an attach via WiFi/Internet to an ePG (non3GPPaccess). The attach status for the different access types might be reported via D, Gr/S6a, S6d or Wx interfaces to appropriate HSS components.  But how shall the HSS derive from the network attach status whether the IMS-APN is established via 2/3G(incl. HSPA), LTE or WiFi to provide the location of the access via which the IMS-APN is established ? Does it require new parallel/sequential requests to SGSN, MME, WiFi-ePG nodes to retrieve this information?

3GPP allows HSS implementation based on different dedicated frontends which serve the various CS/PS interfaces. E.g. GSM-HLR-FE serving D/Gr reference point to MSC/SGSN and EPC-HSS-FE serving S6a/S6d reference points to MME/SGSN and IMS-HSS-FE serving Cx/Dx reference point to I/S-CSCF and Sh interface to application servers. Such Front-Ends might use a shared user profile in a common profile store, but there are no standardised interfaces between these entities.

How shall the request for location cope with these different front-ends? Shall P-CSCF send sequential requests for PS location and non3GPPaccess location?

The second topic is lawful interception (LI), step 5: In the proposed solution the VPLMN ID becomes available when the message was already processed by P-CSCF and S-CSCF (steps 1, 2, and 4). Shall LI be skipped for the registration? Or shall LI be delayed in these entities until VPLMN ID is available. It would be unlawful to intercept a subscriber when he is roaming in another jurisdiction (e.g. law in Germany).