SA WG2 Meeting #115	S2-163206

23 - 27 May 2016, Nanjing, P.R. China	(e-mail revision 3 of S2-163184)

Source:	NTT DOCOMO

Title:	Re-selection of user-plane path based on UE traffic analysis

Document for:	Approval

Agenda Item:	6.10.5

Work Item / Release:	FS_NextGen / 14

Abstract of the contribution:

This contribution proposes a solution for re-selection of user plane path based on information from CN-U to CN-P.

# Discussion

The main concept of the solution is that core-network anchor for the UE is changed by CN-CP based on information provided by user plane node in the core network. This information can be information about the destination IP addresses/servers that the UE is accessing.

# Proposal

***** Start of Change # 1 *****

### 6.5.x	Solution 5.x: Re-selection of user-plane path based on UE traffic pattern

#### 6.5.x.1	Architecture Description

#### The solution proposed in this section is based on the architecture diagram in clause 6.2.2. The main concept of the solution is that core-network anchor for the UE is changed by CN-CP based on information provided by the user plane node in the core network. This information can be information about the destination IP addresses/servers that the UE is accessing.

#### Editor’s note:	Applicability to roaming scenario is FFS.

#### 6.5.x.2	Function description

The high-level flow for this solution is provided in the figure below.

#### Figure 6.5.x.2-1: User-plane path reselection based on UE traffic information from CN-U.

1.	The UE sends a PDU Session Establishment Request (Data Network Name (DNN)) message to CN-CP.

#### Editor’s note:	It is FFS if DNN is different from APN

#### 2.	The CN-CP selects CN-U Anchor-1, based on information e.g. DNN, subscription information of the UE, current mobility information of the UE (if available).

#### 3.	The CN-CP provides forwarding rules to CN-U Anchor 1. The CN-CP subscribes to specific UE traffic events such as access to specific applications/servers, summaries of destination IP address, etc.

#### Editor’s note:	It is FFS how to ensure that CN-U reporting traffic events to the CN-CP does not induce signaling congestion.

#### 4.	The CN-CP sends a PDU Session Establishment Accept message to the UE.

#### 5.	The UL and DL IP Traffic of the UE are via CN-U Anchor 1.

#### 6.	The CN-U Anchor 1 provides periodic/event-based information about the UE’s traffic to CN-CP. This information can include for example, information about destinations/servers that the UE is accessing.

#### 7.	Based on the IP traffic information provided from the CN-U. e.g. UE is accessing content hosted closer to another anchor, and other information such as mobility pattern information of the UE, the CN-CP decides to change the CN-U anchor for the UE such that the new CN-U anchor provides a more efficient user plane path for the UE.

8.	The CN-CP triggers the relocation of the CP-U Anchor by sending a PDU Session disconnection request with “reactivation request” to the UE to reconnect again to the same DNN. The existing PDU Session of the UE is torn down by CN-CP using PDU Session disconnection procedure (8b). The CN-CP keeps record of the new anchor to be selected for the UE for the DNN.

#### 9.	The UE sends PDU Session Establishment Request (DNN) to reconnect to the same DNN.

#### 10. The CN-CP selects CN-U Anchor 2, based on information stored in Step 8.

NOTE 1: 	If the network supports “make before break” for PDU sessions, this feature can be used to minimize disruption to existing IP connections, if needed.

Editor’s note:	The relationship and impact with SSC modes and mechanisms is FFS.

#### NOTE 2:	To minimize impact to user experience, the anchor reselection may occur when UE is idle or lull in UE traffic.

#### Editor’s note:	How this can be achieved in case SM and MM are separated or if the SM changes during the disconnection/reconnection is FFS.

#### 11.	The CN-CP provides forwarding rules to CN-U Anchor 2. The CN-CP subscribes to specific UE traffic events such as access to specific applications/servers, summaries of destination IP address, etc.

#### 12.	The CN-CP sends a PDU Session Establishment Accept message to the UE.

#### 13. The UL and DL IP Traffic of the UE are now via CN-U Anchor 2.

#### 6.5.x.3	Solution Evaluation

Editor's note:	This clause will contain evaluation on the system impacts, e.g., UE, access network and non-access network.

***** End of Change # 1 *****