SA WG2 Meeting #116	S2-163660

11 - 15 July 2016, Vienna, Austria	(revision of S2-16xxxx)

Source:	Deutsche Telekom AG

Title:	TR 23.719 corrections

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_SeDoc / Release 14

Abstract of the contribution: This document proposes corrections to TR 23.719

# Discussion

This document proposes error corrections for different clauses.

# Proposal

It is proposed to change following clauses in TR 23.719

Start of first change

### 6.1.1	Overview

This solution addresses key issue #3.

This solution targets the key issue where the VPLMN is fully supporting IMS and SeDoC but the HPLMN of the roamed in user does not support IMS or ICS. The basic enhancement is to provide VLR services towards the HLR in the HPLMN in an interworking function, which has the role of a centralized VLR, replacing MAP with Diameter in the MSC - Interworking Function communication.

The architecture would look similar to the normal ICS architecture with the difference that the ICS Interworking Function is shared by all MSCs enhanced for SeDoC to support inbound roamers without IMS roaming agreement. The inbound roamers will have a temporary IMS subscription for the VPLMN, all services are executed in the VPLMN.

CAMEL can be supported by collocating the IP Multimedia Service Switching Function (IM SSF) with the AS in the VPLMN. The IM SSF and can retrieve the CAMEL Subscription Information (CSI) from the ICS IWF at the time of the Attach of the UE. The IM SSF will perform the normal CAMEL requests to the gsmSCF in the HPLMN as specified in TS 23.278 [16].

Figure 6.1.1-1: Reference Architecture with ICS IWF

The ICS Interworking Function (ICS IWF) has the following tasks to support inbound roamers:

-	role as VLR towards HLR

-	role as HSS towards AS in VPLMN for local IMS registration, i.e. ICS IWF needs to convert the CS subscription profile and service settings into an IMS profile to be used in the VPLMN

-	role as HSS for S-CSCF for local IMS registration of the MSC enhanced for SeDoC.

Editor's note:	It if FFS where to store the paging area information for the MT call procedure.

The ICS MSC Server needs to be enhanced for SeDoC to support the legacy VLR communication for CS authentication/authorization with Diameter.

In order to support this functionality, 3 new reference points are introduced in above architecture:

I4:	based on Cx, authentication parameters can be provided in a similar way as S-CSCF – HSS communication

Sh'	based on Sh, functionality similar to the procedures of AS –HSS communication

ISC'	based on ISC, functionality similar to the procedures of AS - CSCF communication

Start of second change

### 6.2.1	Overview

This solution addresses key issue #4.

This solution targets the key issue where the HPLMN is fully supporting IMS and SeDoC but the VPLMN of the outbound roaming user does not support IMS or ICS. The basic enhancement is to provide HLR services towards the VLR in the VPLMN in an interworking function.

The architecture is depicted in the following figure:

Figure 6.2.1-1: Architecture for SeDoC outbound roamers support

The ICS Interworking Function (ICS IWF) has the following tasks to support outbound roamers:

-	role as HLR towards VLR

-	role as AS towards HSS in HPLMN for 3rd party IMS registration to enable terminating session routing to the VPLMN.

-	translates IMS service and subscription profile to a CS service and subscription profile

-	B2BUA capabilities for 3rd party IMS registration to enable terminating session routing

In order to support this functionality, 4 new reference points are introduced in above architecture:

Ics1:	based on D, functionality similar to the procedures of HLR towards VLR

Ics2:	based on Cx, authentication parameters can be provided in a similar way as S-CSCF – HSS communication

ISC'	based on ISC, functionality similar to the procedures of S-CSCF – AS communication

Sh'	based on Sh, functionality similar to the procedures of AS –HSS communication

Start of third change

### 6.2.2	Attach Procedure for SeDoC outbound roamer

Outbound roaming

UE

Non

-

ICS

MSC

-

S

ICS IWF

HSS

Legacy CS Protocols

SIP/Diameter

1. Location Update Request (IMSI, LAI)

6. Authentication Request(RAND)

7. Authentication Response(SRES)

<= VPLMN

=> HPLMN

8. Update

Location

(IMSI)

12. Insert Subscriber Data  (IMSI)

14. Location Update Accept

2. Authentication Parameter Request (IMSI)

5. Authentication Info (IMSI, Kc, RAND, SRES)

13. Start ciphering

15. Subscr. Data Insert Ack. /

Loc. Upd. Ack

16. Register to IMS as B2BUA for terminating

session setup towards VPLMN

3. Diameter Authentication Parameter Request (IMSI)

4. Diameter Authentication Info (IMSI, Kc, RAND, SRES)

9. Diameter User Data Request (IMSI)

10. Diameter User Data Response (IMSI)

11. Store Subscriber Data, map to CS Service Data

Figure 6.2.2-1: Procedure for SeDoC outbound roamers support

1.	The UE sends a Location Update Request message to the MSC-Server in the VPLMN and includes its IMSI and the LAI.

2.	The MSC detects based on the IMSI that the UE does not belong to the own network and is an inbound roamer. The MSC contacts the ICS IWF, acting as a HLR of the HPLMN, indicating that CS authentication is required with a Authentication Parameter Request.

3.	The ICS IWF acts like an AS and contacts the HSS, sending a Diameter Authentication Request, including the IMSI.

4.	The HPLMN HSS provides back the Authentication Info with the IMSI, Kc, RAND and SRES parameters. Depending on authentication vector provided by the HSS, the ICS IWF has to map it to a CS authentication vector.

5.	The ICS IWF (acting as HLR to the MSC-Server) provides the Authentication Info parameters to the MSC-Server. The MSC-Server stores the parameters and triggers the CS authentication procedure as specified.

6.	The MSC-Server sends an Authentication Request with the RAND value to the UE.

7.	The UE computes the SRES and provides it back to the MSC-Server in the Authentication Response.

8.	The MSC-Server sends an Update Location Request with the IMSI to the ICS-IWF, acting as the HLR of the HPLMN.

9.	The ICS IWF acts like an AS and contacts the HSS, sending a Diameter User Data Request, including the IMSI.

10.	The ICS IWF retrieves the subscription profile and service settings from the HSS in a Diameter User Data Answer.

11.	The ICS IWF stores the subscriber data and maps the IMS subscription profile and service settings into a CS profile with CS settings.

12.	The ICS IWF replies with the Insert Subscriber Data message according to normal VLR-HLR procedures.

13.	The MSC Server starts ciphering according to normal procedures.

14.	The MSC-Server provides a Location Update Accept message to the UE according to the normal procedures.

15.	The MSC-Server exchanges the Subscriber Data Insert Acknowledgement and Location Update Acknowledgement according to normal VLR-HLR procedures.

16.	The ICS IWF constructs an IMS identity for the UE based on the available information similar to an ICS MSC Server and performs a 3rd party registration to IMS in the HPLMN in order to enable terminating session redirection to the VPLMN.

Start of fourth change

### 6.4.1	Overview

This solution addresses key issue #2.

The MSC-Server enhanced for SeDoC shall be able to detect the Emergency Call setup based on the CS Setup Indication. The ICS MSC Server already is able to transfer CS calls into IMS sessions, but needs to be able to support some P-CSCF capabilities in case of emergency call.

-	Detection of non-UE detectable emergency calls

-	Allow IMS Emergency calls also when the UE is not registered with the IMS or does not have sufficient credentials

-	In the case that UE is not already IMS registered, it shall perform a registration for the support of emergency services (emergency registration).

-	Forwarding of the emergency call request to E CSCF in the same network

-	Since the MSC-Server is the mobility management entity of the CS UE, it can provide location information and can include it already in the request to the E-CSCF.

The following figure shows the basic architecture to support emergency sessions in the VPLMN for the inbound roaming UE.

Figure 6.4.1-1: Basic architecture for Emergency Call support

As a summary the MSC-Server enhanced for SeDoC main impact is to support detection of the CS Emergency Setup, the procedures of the P-CSCF in case of emergency according to TS 23.167 [15] and a new interface towards the E-CSCF.

Editor's note:	It is FFS whether the MSC-Server enhanced for SeDoC performs the IMS Emergency Registration.

Editor's note:	Call back from the PSAP is FFS

End of changes