SA WG2 Meeting #116	S2-164156

11 - 15 July 2016, Vienna, AT	(revision of S2-163982 was 3446)

Source:	NTT DOCOMO, LG Electronics Inc., ITRI

Title:	Update of Network Slicing Solution 1.3

Document for:	Approval

Agenda Item:	6.10.1

Work Item / Release:	FS_NextGen / Rel-14

Abstract of the contribution: This contribution proposes an update to the solution 1.3 for support of multiple connections to multiple network slices.

# Introduction

This paper discusses the Editor’s Notes added in this solution and proposes updates to the solution to resolve the Editor’s Notes.

# Discussions

1. Editor's note: Which C-Plane Functions that are common to multiple Core Network Instances and are dedicated for each Core Network Instance are FFS.

Below are potential C-Plane Functions in NextGen (not an exhaustive list).

- Authentication function (AU): AU is responsible for authenticating and authorizing the UE to attach to the operator’s network. It also provides security and integrity protection of NAS signaling.

- Mobility Management function (MM): MM is responsible for UE registration in the operator’s network (e.g., storing of UE context) and UE mobility support (e.g., providing mobility function when UE is moving across base stations within the operator’s network).

- Session Management function (SM): SM is responsible for PDU session establishment, PDU session modification and PDU session termination.

From the list of CP above, it is expected that the AU and MM functions are common C-Plane functions to multiple Core Network Instances, maintaining a single authentication context for access to the MNO network and a single MM context. This is due to the fact that

- The UE should be first authenticated before accessing the operator’s network regardless which service requested by the UE.

- Even though it might be connected to multiple services (i.e. simultaneous connections to multiple Core Network Instances), it is more efficient to have a single UE context (e.g., MM state) in a single network functions/network entities. This would also simplify mobility management for e.g. IDLE mode procedures such as paging, instead of having multiple separate entities performing separate MM procedures.

SM function(s) are expected to be dedicated to each Core Network Instance, since they are related to establishing a session to a specific service provided by the specific Core Network Instance.

Proposal: It is proposed to have the AU and MM as common C-Plane functions for multiple slices for the same UE, whereas the SM function is a dedicate C-Plane function. Whether other C-Plane functions (e.g., policy and charging control) can be considered as common C-Plane functions is FFS.

2. Editor's note: Whether to keep this figure in this solution or refer to another high-level network slice architecture captured in another clause is FFS.

Proposal: Due to the fact that there is no other clause that captures this similar network slice architecture with the concept of common C-Plane functions, it is proposed to keep this figure as it is.

3. Editor's note: It is FFS whether there is a direct interface between RAN and CP-functions of CN instance.

Based on the above proposal to have the Authentication Function (AU) being common to multiple slices, the UE security context is closely associated with the function that authenticates the UE (as also described by the current single solution for the authentication and security framework for key issue 12. This implies that the security context for protection of UE NAS signaling is in the common CP function, and that the NAS signaling sent by the UE is protected between the UE and the common CP function. Hence, in our views, the direct interface between RAN and CP-functions dedicated for a CN Instance is not needed.

Proposals:

- It is proposed to update the Figure 6.1.3.1-1 by removing the direct interface between RAN and dedicated CP function of CN instance, and to add a few sentences to explain the reasons for “not having such direct interface”.

4. Editor's note: How CPSF and NSSF interact with the RAN node and the C-Plane Function is FFS.

Editor's note: Location of the CPSF and NSSF is FFS.

In our solution, we assume that the RAN communicates with the common CP via a single CP interface regardless of which CP function within the common CP that the RAN is communicating with. A common CP is therefore considered as a black box.

Figure 1: Communication between the RAN and the common CP function via a single CP interface

Taking the above assumption into account, it is not necessary to select the function within the common CP that the RAN has to communicate with but instead it is only necessary to select the right common CP function that is appropriate to the UE, e.g. by taking the UE Usage Type into account. Hence, the term Control Plane Selection Function (CPSF) is to be replaced by the Common Control Plane Selection Function (CCPSF).

Regarding where to place this Common CP Selection Function, we assume that it is located within the common CP function. One of the reasons is to support the case, where the UE does not provide any assistant information or not sufficient information to the RAN. This usually happens during the initial ATTACH or when the UE registers to the PLMN for the first time.

As described in the solution, the NSSF is responsible for selecting which Core Network Instance for a specific service. This usually happens after determining which common CP function is appropriate for the UE. We propose to have the NSSF to be considered as a part of common CP function, such that when it receives a request from the UE to establish a connection with a specific Core Network Instance, the common CP function can select and route the UE’s request immediately without relaying to any other network nodes.

Proposals:

It is proposed to

- Add an assumption that the common CP function is considered as a black box, which consists of one or more CP function(s), and the RAN only uses a single CP interface to communicate with this common CP function.

- Update the CPSF such that it selects the common CP function rather than the CP function within the common CP function for the RAN to communicate with.

- Clarify that both the Common CP Selection Function (CCPSF) and the Network Slice Selection Function (NSSF) is considered as a part of common CP function.

5. Editor's note: Details of the Service Type whether to use existing parameters or new parameters is FFS.

In solution 1.3, the “UE Usage Type”, “DCN-ID”, “Service Type” and “DNN” are used for the purposes as described below:

- “UE Usage Type” is used for identifying the type of UE, e.g., Car UE Usage Type, Smartphone UE Usage Type.

- “DCN-ID” is used for identifying a dedicate CN that is specific for each UE Usage Type and (optionally) that is specific for the enterprise of the same UE Usage Type. In other words, DCN-ID is used to determine which common CP function that is specific for this UE Usage Type and (optionally) for a specific UE enterprise of this UE Usage Type. For instance, one could think of having a dedicate CN for different car enterprises, although they belong to the same type of UE.

- “Service Type” is used for identifying what type of service that the Core Network Instance (CNI) is supposed to support, e.g., the car UE may want to access multiple CNIs simultaneously that supports eMBB service and V2X service

- “Domain Network Name (DNN)” is used for identifying the Packet Data Network (PDN) that a UE wants to communicate with for a certain service type, e.g., DNN#1 for Internet service that requires

Taking the above explanation into account, selecting a proper CNI for the service type that is requested by the UE can be done by using the DNN parameter.

Figure 2 depicts an example of DCN-ID being configured for different UE Usage Type and for different UE enterprises and the CNI being configured for different DNN to support different service.

Figure 2: Communication between the RAN and the common CP function via a single CP interface

Proposals: It is proposed to

- List “UE Usage Type”, “DCN-ID”, “Service Type” and “DNN” parameters that are relevant for selecting common CP function and for selecting CNI and describe the usage of these parameters

# Proposal

Based on the discussions above, it is proposed to update the solution in the TR23.799 accordingly.

* * * * Start of changes * * * *

### 6.1.3	Solution 1.3: Support of multiple connections to multiple Network Slices

This solution is to solve the key issue#1 on support of network slicing.

#### 6.1.3.1	Architecture description

To enable a UE to simultaneously obtain services from multiple Network Slices of one network operator, the Core Network Instances can be set up as depicted in Figure 6.1.3.1-1. That is:

-	A single set of C-Plane Functions that are in common among Core Network Instances is shared across multiple Core Network Instances;

-	Other C-Plane Functions that are not in common reside in their respective Core Network Instances, and are not shared with other Core Network Instances.

Common C-Plane Functions to multiple Core Network Instances can be:

-	Authentication function (AU): AU is responsible for authenticating and authorizing the UE to attach to the operator’s network. It also provides security and integrity protection of NAS signaling.

-	Mobility Management function (MM): MM is responsible for UE registration in the operator’s network (e.g., storing of UE context) and UE mobility support (e.g., providing mobility function when UE is moving across base stations within the operator’s network).

Dedicated C-Plane Functions for each Core Network Instances can be:

-	Session Management function (SM): SM is responsible for PDU session establishment, PDU session modification and PDU session termination.

Editor’s note: Whether there are more C-Plane functions to consider as a common C-Plane function for multiple Core Network Instances or a dedicated C-Plane function for each Core Network Instance is FFS.

Figure 6.1.3.1-1 Sharing a set of common C-plane functions among multiples Core Network Instances

Editor’s note: It is FFS, whether all CP functions in the dedicate Common CP function and in the default Common CP function should be exactly the same as depicted in Figure 6.1.3.1-1, e.g., whether CCPSF should be available in the dedicate Common CP function.

The direct CP interface between RAN and CP-functions of CN instance is not needed, since the encrypted NAS signalling message has to be decrypted by a common C-Plane function, which has exchanged the encrypted key with the UE, for example, the AU function.

NOTE 1:	In case of UE accessing a single Core Network Instance, RAN can have a direct CP interface to the specific CP function located within the Core Network Instance. In such case, those CP functions depicted as a part of common CP function are considered to be CP functions resided within the Core Network Instance, and hence, there is no common CP function.

The principles of the solution depicted in Figure 6.1.3-1 are described as following:

-	A Core Network Instance consists of a single set of C-Plane Functions and a single set of U-Plane Functions.

-	A Core Network Instance is dedicated for the UEs that are belonging to the same UE type. Identifying the UE type is done by using a specific parameter, e.g. the UE Usage Type, and/or an information from the UE's subscription.

-	A set of C-Plane functions is responsible, for example, for supporting UE mobility if demanded or for admitting the UE into the network by performing authentication and subscription verification.

-	All C-Plane Functions that are common to multiple Core Network Instances, are not necessary to be created multiple times.

-	Other C-Plane Functions that are not in common with other Core Network Instances are only used by its own Core Network Instance.

-	A set of U-Plane Functions in a Core Network Instance is responsible for providing a specific service to the UE and for transports the U-Plane data of the specific service. For example, one set of U-Plane functions in Core Network Instance#1 provides an enhanced mobile broadband service to the UE, whereas another set of U-Plane functions in Core Network Instance#2 provides a critical communication service to the UE.

-	Each UE can have multiple U-Plane connections to different sets of U-Plane Function that are available at different Core Network Instances simultaneously.

-	The Network Slice Selection Function (NSSF) is responsible for selecting which Core Network Instance to accommodate the UE by taking into account the UE's subscription and the specific parameter, e.g. the UE Type, the Service Type or Domain Network Name (DNN) that can be mapped to represent the Service Type that the CNI supports.

-	The RAN communicates with the common CP via a single CP interface regardless of which CP function within the common CP that the RAN is communicating with. Hence, a common CP is considered as a black box.

-	The Common C-Plane Selection Function (CCPSF) is considered to be a part of common CP function, i.e. the CCPSF is assumed to be located in the common CP function. The CCPSF is responsible for determining which common C-Plane Function that the Base Station should communicate with. Determination of common C-Plane Function is done by checking with the UE’s subscription profile.

NOTE 2:	The CCPSF is not a function to route the NAS signaling message to the right Common C-Plane Function. In this solution, it is assumed that the RAN has a function to route the NAS signalling message to a proper Common C-Plane Function by taking the information that is sent by the CCPSF in case of the first initial connection request or by the UE in case of any subsequent NAS messages.

In this solution, the “UE Usage Type”, “DCN-ID”, “Service Type” and “DNN” are used for the purposes as described below:

- “UE Usage Type” is used for identifying the type of UE, e.g., Car UE Usage Type, Smartphone UE Usage Type.

- “DCN-ID” is used for identifying a dedicate CN that is specific for each UE Usage Type. In addition, an operator may consider to use the DCN-ID to identify specific a dedicate CN for the enterprise of the same UE Usage Type or even for the different variants of dedicated CN. An example for the latter case could be to have different variants of dedicate CN for CIoT UEs, i.e. supporting different CIoT optimization features as specified in TS23.401. In other words, DCN-ID is used to determine which common CP function that is specific for this UE Usage Type and (optionally) for a specific UE enterprise of this UE Usage Type. For instance, one could think of having a dedicate CN for different car enterprises, although they belong to the same type of UE.

- “Service Type” is used for identifying what type of service that the Core Network Instance (CNI) is supposed to support, e.g., the car UE may want to access multiple CNIs simultaneously that supports eMBB service and V2X service

- “Domain Network Name (DNN)” is used for identifying the Packet Data Network (PDN) that a UE wants to communicate with for a certain service type, e.g., DNN#1 for Internet service that requires Mobile Broadband service.

- Temporary UE identity is a temporary identifier provided by the CN to the UE. This is similar to GUTI in EPS case. This temporary UE identity consists of two parts: 1) identifier of Common CP function, 2) UE specific identifier

Taking the above explanation into account, selecting a proper CNI for the service type that is requested by the UE can be done by using the DNN parameter.

Editor's note: How this solution and the above parameters (“UE Usage Type”, “DCN-ID”, “Service Type” and “DNN”) supports the roaming scenario is FFS.

#### 6.1.3.2	Function description

Figure 6.1.3-2 Signalling flow for supporting connections with multiple Core Network Instances

1.	When a UE first connects to the operator's network or registers to the PLMN for the first time and there is no sufficient information for the RAN to route this network connection request to an appropriate Common C-Plane Function, the UE sends a network connection request to the RAN, which is then further forwarded to the default Common C-Plane Function. The flow continues in step 2.

If the UE provides the DCN-ID along with this network connection request, but does not provide a Temporary UE identity, the RAN uses this DCN-ID to determine an appropriate Common CP that supports this DCN-ID, and the flow continues in step 4. In addition, the UE may provide other information, e.g., Service Type and/or DNN along with this network connection request.

If the UE provides the Temporary UE identity, the RAN uses the Temporary UE identity to determine a dedicate Common CP function and, the flow continues in step 4. In addition, the UE may provide other information, e.g., DCN-ID, Service Type and/or DNN along with this network connection request.

NOTE 1:	When the UE sends a request to connect to an operator’s network, a UE may request to establish a session for a particular service by sending the DNN along with this network connection request. If this is the case, after the authentication and authorization in step 6 has been performed, the dedicate C-CPF will establish the session for the request service like similar to step 9, 10 and 11.

2.	The CCPSF located in the default C-CPF determines which Common C-Plane function to be connected to by taking into account information in the network connection request from a UE in step#1, e.g., DCN-ID. In addition, other information from the subscription database may be also considered. For example, the UE’s subscription may indicate that for this UE, the operator should set up a session with the subscribed CNI. In this signalling flow example depicted in Figure 6.1.3-2, this is the Core Network Instance#1.

In case, the default C-CPF determines that it will serve this UE’s network connection request, it either continues with the authentication and admitting the UE to attach/connect to operator’s network or reject this UE’s network connection request and hence this procedure ends in this step. It is to note that this specific case is not depicted in Figure 6.1.3-2 for simplicity of the signalling flow.

Editor's note: It is FFS, if authentication of UE to access operator's network should occur before the NSSF/CPSF selects Core Network Instance.

3.	The default C-CPF sends a response to the RAN node with the Common C-Plane function for the UE to attach. Same content of UE’s network connection request in step#1 is also sent back to the RAN.

Editor's note: whether a default CCPF will communicate directly with the dedicate CCPF is FFS, i.e. without redirecting the “network connection request” to the dedicate CCPF.

4.	If the UE provides a Temporary UE identity, this means that the UE has been once registered at the PLMN, and the CN has decided to which DCN-ID the UE should be connected. Hence, the RAN uses only the Temporary UE identity to route the NAS message to the appropriate Common CP function.

In case there is a pool of Common C-Plane Functions that are dedicated for this UE Usage Type and/or for this DCN-ID provided by the UE or by the default Common C-Plane Function in step#3, the RAN node performs NAS Node Selection Function (NNSF) similar to what we have for the NNSF in eNB.

5.	The RAN node routes the UE's network connection request to the dedicated Common C-Plane Function (dedicate C-CPF). Along with this request, it may contain other information like the DNN to enable the dedicate C-CPF to select a CNI that is specific for a certain Service Type.

Editor’s note: It is FFS, when the dedicate C-CPF rejects the UE’s network connection request.

6.	Authentication and admitting the UE to attach/connect to operator’s network is performed.

NOTE 2:	In this step, the key for decrypting NAS message between the UE and the dedicate Common CP function is also provided. Exact details of UE authentication and UE's subscription verification for step 7 are out of scope of this solution.

7.	The dedicated Common CP function sends a network connection accept response to the UE. In this response, it contains the Temporary UE identity and the information, for which the UE is to be configured, e.g., which DCN-ID, its corresponding Service Type and/or corresponding DNN that the UE is allowed to connect. In case, the DCN-ID newly provided does not match to the ones that the UE already has, the DCN-ID(s) will be configured at the UE.

8.	UE requests for establishment of a session for a communication service (e.g. service#1 that is provided by the Core Network Instance#1) by sending an SM request for a new PDU session. In this PDU session request, the UE provides the DCN-ID and DNN.

9.	The RAN forwards the PDU session request to an appropriate dedicate Common C-Plane function (dedicated  C-CPF) by using the DCN-ID information sent by the UE.

10.	The dedicated C-CPF selects C-Plane Function of the Core Network Instance#1 by using the DNN information in the PDU session request, and forwards the UE's PDU session request for the service#1 to the CPF-1 in Core Network Instance#1 (CNI-1 CPF-1), which is responsible for session management in CNI#1. Note that this forwarded PDU session request still contains the information that has been sent by the UE such as the UE Usage Type, DCN-ID and DNN.

Editor's note: It is FFS, whether additional authorization and/or additional authentication to admit the UE to access Core Network Instance#1 is needed.

11.	After a successful session establishment, the CPF-1 in Core Network Instance#1 sends the session response back to the dedicated C-CPF.

12.	The dedicated C-CPF sends a new service response back to the UE via the RAN.

13.	UE requests for establishment another session for a new communication service that is of a different service type than the previous service. In this PDU session request, the UE provides the Temporary UE identity, UE Usage Type, DCN-ID, and Service Type and/or DNN.

14.	The RAN determines the dedicate Common C-Plane Function (dedicate C-CPF) by using the Temporary UE identity sent by the UE and forwards the PDU session request to the dedicate Common C-Plane Function (dedicated C-CPF).

15.	The dedicated C-CPF selects C-Plane Function of the Core Network Instance#2 by using the DNN information in the PDU session request, and forwards the UE's service request for the new service to the CPF-1 in Core Network Instance#2 (i.e. CNI-2 CPF-1), which is responsible for session management in CNI#2. Note that this forwarded PDU session request still contains the information that has been sent by the UE such as the UE Usage Type, DCN-ID, and Service Type and/or DNN.

Editor's note: It is FFS, whether additional authorization and/or additional authentication to admit the UE to access Core Network Instance#2 is needed.

16.	After a successful session establishment, the CPF-1 in Core Network Instance#2 sends the session response back to the dedicated C-CPF.

17.	The dedicated C-CPF sends a new service response back to the UE via the RAN.

#### 6.1.3.3	Solution evaluation

Editor's note: This clause will contain evaluation on the system impacts, e.g. UE, access network and non-access network.

* * * * End of changes * * * *