SA WG2 Meeting #119	S2-172067

(revision of S2-17xxxx)

Source:	Huawei, Hisilicon

Title:	TS 23.502: Update the PDU session establishment procedure with uplink classifier UPF

Document for:	Approval

Agenda Item:	6.5.3

Work Item / Release:	5GS_ph1/ Rel-15

Abstract of the contribution:

This paper proposes to update the PDU session establishment procedure with Uplink Classifier UPF supporting.

# Proposal

It is proposed to make the following changes to the TS 23.502 (procedure).

*************** Start of 1st changes *********************

##### 4.3.2.2.1	Non-roaming and Roaming with Local Breakout

In case of roaming, the AMF determines if a PDU session is to be established in LBO or Home Routing. In the case of LBO, the procedure is as in the case of non-roaming with the difference that the SMF, the UPF and the PCF are located in the visited network.

Figure 4.3.2.2.1-1: UE-requested PDU Session Establishment for non-roaming and roaming with local breakout

The procedure assumes that the UE has already registered on the AMF thus the AMF has already retrieved the user subscription data from the UDM.

1.	From UE to AMF: NAS Message (S-NSSAI, DNN, PDU Session ID, N1 SM information).

In order to establish a new PDU session, the UE generates a new PDU Session ID.

The UE initiates the UE Requested PDU Session establishment procedure by the transmission of a NAS message containing a PDU Session Establishment Request within the N1 SM information. The PDU Session Establishment Request may include a PDU Type, SSC mode, Protocol Configuration Options.

The NAS message sent by the UE is encapsulated by the AN in a N2 message that should include User location information and Access Technology Type Information.

The SM information may contain SM PDU DN Request Container containing information for the PDU session authorization by the external DN.

2.	The AMF determines that the message corresponds to a request for a new PDU Session based on the PDU Session ID that is not used for any existing PDU Session(s) of the UE. The AMF selects an SMF as described in TS 23.501 [2], clause 6.3.2.

3.	From AMF to SMF:  SM Request (Subscriber Permanent ID, DNN, S-NSSAI, PDU Session ID, AMF ID, N1 SM information, User location information, Access Technology Type).

The AMF ID uniquely identifies the AMF serving the UE. The N1 SM information contains the PDU Session Establishment Request received from the UE.

4a.	SMF to UDM: Subscription Data Request (Subscriber Permanent ID, DNN).

If the SMF has not yet retrieved the SM-related subscription data for the UE related with the DNN, the SMF requests this subscription data.

4b.	UDM to SMF: Subscription Data Response.

Subscription data includes the authorized PDU type(s), authorized SSC mode(s), Default QoS profile.

Editor's note:	Further detailing of SM-related subscription data is FFS.

The SMF checks whether the UE request is compliant with the user subscription and with local policies. If that is not the case the SMF rejects the UE request via NAS SM signalling (including a relevant SM rejection cause) relayed by the AMF, the SMF indicates to the AMF that the PDU session ID is to be considered as released and the rest of the procedure is skipped.

5.	SMF to DN via UPF

If the SMF needs to authorize/authenticate the establishment of the PDU session as described in clause 5.6.6 of TS 23.501 [2], the SMF selects an UPF as described in TS 23.501 [2] clause 6.3.3 and triggers the PDU session establishment authentication/authorization as described in section 4.3.2.X.

If the PDU session establishment authentication/authorization fails, the SMF terminates the PDU session establishment procedure and indicates a rejection to the UE.

6a.	If dynamic PCC is deployed, the SMF performs PCF selection. The SMF retrieve the traffic routing information for the UE. SMF use the traffic routing information to do the UPF selection and allow routing user traffic to a local access to a Data Network.

6b.	The SMF may initiate PDU-CAN Session Establishment towards the PCF to get the default PCC Rules for the PDU Session.

Editor's note:	The purpose of step 5 is to receive PCC rules before selecting UPF. If PCC rules are not needed as input for UPF selection, step 5 can be skipped.

7.	The SMF selects an SSC mode for the PDU Session. If step 5 is not performed, the SMF also selects an UPF as described in TS 23.501 [2] clause 6.3.3. In case of the UE can locally access to the Data Network, an UL CL should be selected. In case of PDU Type IPv4 or IPv6, the SMF allocates an IP address/prefix for the PDU Session as described in TS 23.501 [2] clause 5.8.1.

8.	If dynamic PCC is deployed and the PDU-CAN Session Establishment was not done in step 5, the SMF initiate PDU-CAN Session Establishment towards the PCF to get the default PCC Rules for the PDU Session. Otherwise, if dynamic PCC is deployed and PDU Type is IPv4 or IPv6, SMF initiates PDU-CAN Session Modification and provides the allocated UE IP address/prefix to the PCF.

9.	If step 5 was not performed, the SMF initiates an N4 Session Establishment procedure with the selected UPF, otherwise it initiates an N4 Session Modification procedure with the selected UPF:

9a.	The SMF sends an N4 Session Establishment/Modification Request to the UPF and provides Packet detection, enforcement and reporting rules to be installed on the UPF for this PDU Session. If UPF Tunnel Info is allocated by the SMF, the UPF Tunnel Info is provided to UPF in this step. If an UL CL is selected and ULCL tunnel Info is allocated by the SMF, the UL CL Tunnel Info is provided to the UPF in this step.

9b.	The UPF acknowledges by sending an N4 Session Establishment/Modification Response. If UPF Tunnel Info is allocated by the UPF, the UPF Tunnel Info is provided to SMF in this step.

9c. If an UL CL is selected, the SMF sends an N4 Session Establishment/Modification Request to the UL CL and provides traffic routing information for the UL CL to decide if the traffic flow can be locally access to the data network. The SMF should provide the UPF tunnel info to the UL CL for uplink traffic data. If UL CL Tunnel Info is allocated by the SMF, the UL CL Tunnel info is provided to UL CL in this step.

9d.	The UPF acknowledges by sending an N4 Session Establishment/Modification Response. If ULCL Tunnel Info is allocated by the UL CL, the ULCL Tunnel info is provided to SMF in this step.

10.	SMF to AMF:  SM Request Ack (N2 SM information (PDU Session ID, QoS Profile, CN Tunnel Info), N1 SM information (PDU Session Establishment Accept (Authorized QoS Rule, SSC mode))) to the AMF.

The N2 SM information carries information that the AMF shall provide to the (R)AN.

The CN Tunnel Info corresponds to the Core Network address of the N3 tunnel corresponding to the PDU session.

The QoS Profile provides the AN with the mapping between QoS parameters and QoS Flow Identifiers. This is further described in TS 23.501 [2] clause 5.7.

The PDU Session ID may be used by AN signalling with the UE to indicate to the UE the association between AN resources and a PDU session for the UE.

The N1 SM information contains the PDU Session Establishment Accept that the AMF shall provide to the UE.

Multiple Authorized QoS Rules may be included in the PDU Session Establishment Accept within the N1 SM information and in the N2 SM information.

The SM Request Ack further contains information allowing the AMF to know which UE is the target of the SMF request as well to determine which access towards the UE to use.

NOTE:	The access information is to deal with the case where a UE is simultaneously connected over 3GPP and Non 3GPP access.

Editor's note:	Further studies are needed to determine the nature of the information allowing the AMF to know which UE is the target of the SMF request as well to determine which access towards the UE to use.

11.	AMF to (R)AN: N2 PDU Session Request (N2 SM information, , PDU Session Establishment Accept).

The AMF sends the PDU Session Establishment Accept and the N2 SM information received from the SMF within the N2 PDU Session Request to the (R)AN.

12.	(R)AN to UE: The (R)AN may issue AN specific signalling exchange with the UE that is related with the information received from SMF. For example, in case of a 3GPP RAN, an RRC Connection Reconfiguration may take place with the UE establishing the necessary RAN resources related to the Authorized QoS Rules for the PDU Session request received in step 10.

(R)AN also allocates (R)AN tunnel information for the PDU Session. (R)AN forwards the NAS message (PDU Session Establishment Accept) provided in step 10 to the UE. (R)AN shall only provide the NAS message to the UE if the necessary RAN resources are established and the allocation of (R)AN tunnel information are successful.

13.	(R)AN to AMF: N2 PDU Session Request Ack ((R)AN Tunnel Info).

The (R)AN Tunnel Info corresponds to the Access Network address of the N3 tunnel corresponding to the PDU session.

14.	AMF to SMF: SM Request (N2 SM information).

The AMF forwards the N2 SM information received from (R)AN to the SMF.

Editor's note:	It is FFS if steps are needed to indicate from the UE to the core network that the UE has successfully established the PDU Session, or whether it is sufficient with a successful establishment in (R)AN indicated in step 12. For example, it is FFS if the UE shall send a NAS PDU Session Establishment Complete message to indicate that the UE has successfully established the PDU Session.

15a. If SMF receives the UL CL Tunnel Info in step 9d, the information should be sent to the UPF. In case the UL CL is not selected, if the N4 session for this PDU Session was not established already, the SMF initiates an N4 Session Establishment procedure with the UPF. Otherwise, the SMF initiates an N4 Session Modification procedure with the UPF. The SMF provides AN Tunnel Info and UPF Tunnel Info. The UPF Tunnel Info only needs to be provided if the SMF selected CN Tunnel Info in step 8.

15b.	The UPF provides a N4 Session Establishment/Modification Response to the SMF.

15c. In case the UL CL is selected, the SMF initiates an N4 Session Modification procedure with the UL CL. The SMF provides AN Tunnel Info to the UL CL.

15d. The UL CL provides a N4 Session Modification Response to the SMF.

The step 15a and 15c can be performed simultaneously.

16.	After this step, the AMF forwards relevant events to the SMF, e.g. at handover where the (R)AN Tunnel Info changes or the AMF is relocated.

Editor's note:	It is FFS if the SMF has to explicitly subscribe to these events or whether the subscription is implicit.

17.	SMF to UE, via UPF: In case of PDU Type IPv6, the SMF generates an IPv6 Router Advertisement and sends it to the UE via N4 and the UPF.

During the lifetime of the PDU session, the AMF stores an association of the PDU session ID and the SMF ID.

*************** End of changes *********************