# 3GPP TSG SA WG 2 Meeting 120	TDoc S2-172744

27 - 31 March 2017, Busan, S. Korea

Title:	LS on Applying Extended NAS timers based on UE's operation in CE Mode B

Response to:

Release:	Rel-14

Work Item:	CIoT

Source:	SA2

To:	CT1, RAN3

Cc:	CT4

Contact person:	Puneet Jain

puneet.jain@intel.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

It is SA2 understanding that CT1 has specified (TS 24.301 Section 4.8) that MME applies extended NAS timer for the UEs indicating support of CE mode B, and CT4 has plan to work on a solution that an MME will reject the Create/Update Bearer Request if the UE is using extended NAS timers due to possibility of GTP timer expiry. As a result dedicated bearer services (e.g. VoLTE) are not possible for UE that support CE mode B even if it is operating in CE mode A.

For CIoT Location Services work in Rel-14, SA2 has agreed in CR3163R1 (S2-166947) that provides CE level from eNB to MME at RRC/S1 connection establishment. SA2 is discussing the possibility of sending the CE level with every S1-AP UL NAS transport message.

SA2 would like to ask CT1 if they see any issue in using CE level information at MME to determine if the UE is operating in CE mode B and apply extended NAS timer at MME only if CE level corresponds to CE mode B.

SA2 would like CT1 to avoid any unexpected expiry of NAS timer due to transition between CE mode B and CE mode A.

# 2	Actions

To CT1:

ACTION: 	SA2 kindly ask to provide response to the above question.

To RAN3:

ACTION: 	It is SA2 understanding that CE levels are provided in the S1-AP Initial UE message and not at each S1-AP UL NAS Transport message. Does RAN3 see any issue with providing CE level with each S1-AP UL NAS Transport message?

# 3	Dates of next TSG SA WG2 meetings

TSG SA WG2 Meeting 121	15 - 19 May 2017	Hangzhou, CN

TSG SA WG2 Meeting 122	26 - 30 Jun 2017    	Cabo, Mexico