SA WG2 Meeting #122Bis	S2-175508

21 - 25 August 2017, Sophia Antipolis, France

Source:	China Mobile

Title:	23.502 on NF service discovery procedures by NRF

Document for:	Discussion and Approval

Agenda Item:	6.5.11

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: this contribution propose to move the NF/NF service discovery procedures from clause 5.2.6 to clause 4 and clarifies further on the procedures.

- Introduction

In SA2#121, the NF service discovery were approved in section 6.3.1 of TS23.501.

This proposal updates the system procedure and NRF services to support NF/NF service discovery. Some part of this proposal also motivated from S2-175060

- Proposal

* * * First Change * * *

4.x	Network Function Service Framework Procedure

4.x.1	Non-roaming scenario NF Service Discovery

Figure 4.x.1-1: NF/NF Service Discovery in the same PLMN

1.	NF service consumer invokes Nnrf_NFDiscovery_Request (Target NF Service Name, NF type of the target NF, NF type of the NF service consumer) to discovery the expected NF or NF service in the same PLMN. The NF service consumer may also include parameters such as S-NSSAI, other services related information.

2. The NRF authorizes the NF Service Discovery Request. Based on the service authorization information in the profile of the expected NF, the NRF check whether the NF service consumer is authorized to access the expected NF instance(s) or the specific NF service(s).

3. If allowed, the NRF provides the information of a set of discovered NF instance(s) or a specific NF service to the NF service consumer via NF Service Discovery response message. The information include: FQDN or IP address.

4.x.2	Roaming scenario NF Service Discovery Procedure

Figure 4.x.2-1: NF/NF Service Discovery across PLMNs

The NF service consumer intends to discover the NF instance(s) in another PLMN. For example, the AMF/AMF service requests to discover the SMF instance(s) or SMF service in the remote PLMN.

1.	NF service consumer invokes Nnrf_NFDiscovery_Request (Target NF Service Name, NF type of the target NF, NF type of the NF service consumer, the Remote PLMN ID) to discovery the expected NF or NF service in the same PLMN. The NF service consumer may also include parameters such as S-NSSAI, other services related information.

2. The NRF in serving PLMN identifies NRF based on the Remote PLMN ID, and it invokes "NF Discovery" service from the NRF of the remote PLMN according the procedure in Figure 4.x.1-1 to get the expected NF instance (s) or NF service deployed in the remote PLMN. As the NRF in the serving PLMN triggers the "NF Discovery" on behalf of the NF service consumer, the NRF in the serving PLMN shall not replace the information of the NF service consumer, i.e. NF service consumer ID, in the Discovery Request message it sends to the NRF in remote PLMN. Due to roaming agreements and operator policies, the NRF in the remote PLMN authorize the request and may provide information of the IP address or the FQDN of a set of the discovered NF instance(s) or of the specific NF service to the NRF in the serving PLMN.

NOTE: Due to network topology hiding or network configuration, the remote NRF may provide information of the IP address or the FQDN of a proxy function of the discovered NF/NF service to the serving NRF.

3.	The NRF in serving PLMN provides the information e.g. FQDN and IP address, of a set of the discovered NF Instance(s) or a specific NF service in NF Service Discovery Response message.

* * * Next Change * * *

5.2.6	NRF services

5.2.6.1	General

The following table shows the NRF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NRF

| Service Name | Service Operations | Known Consumer(s) | Operation Semantic |
| --- | --- | --- | --- |
| Nnrf_NFDiscovery | Request | AMF, SMF, PCF, NEF, NRF, NSSF, UDM, AUSF | Request/ Response |

5.2.6.2	Nnrf_NFDiscovery service

5.2.6.2.1	General

Service description: This service enables one NF to discover a set of NF instances with specific NF service or a target NF type. The service also enables one NF service to discover a specific NF service. The service operations defined below allow the NF/NF services to communicate with NRF.

5.2.6.2.2	Nnrf_NFDiscovery service operation

Service operation name: Nnrf_NFDiscovery_Request

Description: provides the IP address or FQDN of the expected NF instance(s) to the NF service consumer.

Known NF Service Consumers: AMF, SMF, PCF, NEF, NRF in Serving PLMN, NSSF, UDM, AUSF

Inputs, Required: target NF Service Name, NF type of the target NF, NF type of the NF service consumer.

Inputs, Optional: S-NSSAI, target NF/NF service PLMN ID, Serving PLMN ID, the NF service consumer ID.

Outputs, Required: FQDN and IP address of a set of requested target NF instances, or FQDN and IP address of a NF service.

Outputs, Optional: none.

See clause 4.x.1/4.x.2 for details on the usage of this service operation.

* * * End of Changes * * *