SA WG2 Meeting #122	S2-176357

21 – 25 August 2017, Sophia Antipolis, France

Source:	NICT, ATR, Sandvine Inc.

Title:	TS 23.501: Proposed ENs resolutions for 5.18 Network sharing

Document for:	Approval

Agenda Item:	6.5.1

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: This P-CR proposes resolutions to two Editor Notes that are currently in 5.18 Network Sharing of draft TS 23.501. This is a revision of S2-175843.

# Discussion

The first EN in 5.18 of draft TS 23.501 reads “Relation between network slicing and network sharing should be clarified.”

The second EN in 5.18 of draft TS 23.501 reads “Whether core network functions and resources could be also shared is FFS.

Discussion paper S2-174528 submitted to San Jose meeting proposed a clarification of the relationship between network slicing and network sharing. Also, it showed several deployment examples using both network slicing and network sharing, where network sharing was RAN sharing or RAN and CN sharing.

This P-CR proposes modification to 5.18 in draft TS 23.501 inline with recommendations proposed in S2-174528 and taking into account comments received during the meeting in Sophia Antipolis.

# 2	Proposal

It is proposed to update TS 23.501 as described below.

*************** Start of changes *********************

## 5.18	Network Sharing

### 5.18.1	General concepts

A network sharing architecture shall allow multiple participating operators to share resources of a single shared network according to agreed allocation schemes. The shared network includes a radio access network. The shared resources include radio resources.

The shared network operator allocates shared resources to the participating operators based on their planned and current needs and according to service level agreements. The shared resources are allocated separately for each participating operator and for signalling traffic and user data traffic within each participating operator. When there is a need for additional resources for a participating operator, the shared network operator may allocate available shared resource to the participating operator according to the service level agreement.

A UE that has a subscription to a participating network operator shall be able to select this participating network operator while within the coverage area of the shared network and to receive subscribed services from the participating network operator.

Network sharing and network slicing technologies do not have direct impact at each other. A Network Slice is defined within a PLMN, while network sharing is performed among different PLMNs.

*************** End of changes *********************