3GPP TSG-SA WG2 Meeting #127bis 	S2-185352

Newport Beach, the US, 28 May –1 June 2018			       					      revision of S2-184469

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0268 | rev | 4 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of downlink NAS signalling for UE in CM-IDLE on Non-3GPP access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2018-05-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current specification TS23.502 doesn’t provide the network requested PDU session Release procedure via untrusted non-3GPP access. This signalling procedure needs to be added because the Network is allowed to initiate the release of a PDU session over non-3GPP access. In case the UE simultaneously registered to a 3GPP access and a non-3GPP access and the UE is in CM-IDLE state over non-3GPP access, the UE is not reachable on non-3GPP , but it may be reachable on 3GPP, however the PDU session release signaling should not be notified to the UE via 3GPP access. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revise the existing clause 4.12.7 to describe the signalling procedure of the network requested PDU session Release via untrusted non-3GPP access. The core network handling refers the network requested PDU session release for Non-Roaming and Roaming with Local Breakout in clause 4.3.4.2. The N3IWF handling refers the UE requested PDU SessionRelease via untrusted non-3GPP access in clause 4.12.7. The new clause describes the difference in respect the existing UE or Network PDU session release procedure for 3GPP. New paragraph in step 3 of UE or network Requested PDU Session Release via Untrusted non-3GPP access clarifying that the downlink signalling over non-3GPP access shall not be notified to the UE by AMF via 3GPP access type if the UE is in CM-IDLE state over non-3GPP access even though the SMF instructs the AMF to notify the UE without the “skip indicator”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete. Not correct handling of downlink signaling over non-3GPP access in case the UE is simultaneously registered to a 3GPP access and a non-3GPP access in the same PLMN and the UE is in CM-IDLE over non-3GPP access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.12.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** Start of Change *****

### 4.12.7	UE or network Requested PDU Session Release via Untrusted non-3GPP access

Clause 4.12.7 specifies how a UE or network can release a PDU Session via an untrusted non-3GPP access network. The UE requested PDU Session Release procedure via Untrusted non-3GPP access applies in non-roaming, roaming with LBO as well as in home-routed roaming scenarios.

For non-roaming and LBO scenarios, if the UE is simultaneously registered to a 3GPP access in a PLMN different from the PLMN of the N3IWF, the functional entities in the following procedures are located in the PLMN of the N3IWF. For home-routed roaming scenarios, the AMF, V-SMF and associated UPF in VPLMN in the following procedure is located in the PLMN of the N3IWF.

NOTE:	If the UE is simultaneously registered to 3GPP access in the same PLMN as non-3GPP access, when non-3GPP access is not available to the UE (e.g. due to out of non-3GPP coverage) or UE is in CM-IDLE for non-3GPP access, the UE may perform the PDU Session Release procedure via 3GPP access as described in clause 4.3.4.

Figure 4.12.7-1: UE Requested PDU Session Release via Untrusted non-3GPP access

1.	One or more PDU Sessions are already established for the UE using the procedure described in clause 4.12.2.

2.	The UE sends a PDU Session Release Request message to the SMF via the N3IWF as defined in clause 4.3.4.

3.	For non-roaming and roaming with LBO, the steps 1a (from AMF) to 3 according to the PDU Session Release procedure defined in clause 4.3.4.2 are executed. For home-routed roaming, the steps 1a (from AMF) to step 5 according to the PDU Session Release procedure defined in clause 4.3.4.3 are executed.

4.	This step is the same as step 4 in clause 4.3.4.2 (non-roaming and LBO) and step 6 in clause 4.3.4.3 (home-routed roaming).

5.	Upon receiving AN session release request message from the AMF, the N3IWF triggers the release of the corresponding Child SA by sending INFORMATIONAL EXCHANGE (Delete Payload) to the UE. Delete payload is included in the message listing the SPIs of the Child SAs to be deleted to this PDU Session as described in RFC 7296 [3].

6.	The UE responds with INFORMATIONAL EXCHANGE (Delete Payload) message. Delete payload is included for the paired SAs going in the other direction as described in RFC 7296 [3].

7.	This step is the same as step 6 in 4.3.4.2 (non-roaming and LBO) and step 8 in clause 4.3.4.3 (home-routed roaming).

8.	For non-roaming and roaming with LBO, steps 7 according to the PDU Session Release procedure defined in clause 4.3.4.2 are executed. For home-routed roaming, step 9-10 according to the PDU Session Release procedure defined in clause 4.3.4.3 are executed.

9.	The N3IWF delivers the NAS message (PDU Session Release Command) to the UE.

10.	The UE sends a NAS message (PDU Session Release Ack) to the N3IWF.

11.	This step is the same as step 9 in 4.3.4.2 (non-roaming and LBO) and step 11 in clause 4.3.4.3 (home-routed roaming).

Steps 5 and 9 may happen consecutively. Step 10 may happen before step 6.

12.	For non-roaming and roaming with LBO, all steps after step 10 in the PDU Session Release procedure defined in clause 4.3.4.2 are executed. In case of home-routed roaming, all steps after step 12 in the PDU Session Release procedure defined in clause 4.3.4.3 are executed.

Editor's note:	The details of the NAS messages (i.e. PDU Session Release Request and PDU Session Release Response) including message name and parameters, are FFS and should be align with PDU Session Release procedure via 3GPP access.

The network requested PDU Session Release procedure via Untrusted non-3GPP access is the same as the network requested PDU Session Release Procedure specified in clause 4.3.4.2 (for Non-Roaming and Roaming with Local Breakout) with the following differences:

-	The (R)AN corresponds to an N3IWF.

- 	In step 5 the N3IWF upon receiving N2 SM request to release the AN resources associated with the PDU Session from the AMF, the N3IWF triggers the release of the corresponding Child SA to the UE as specified in step 5 and 6, in Figure 4.12.7 clause 4.12.7.

-	User Location Information is not included in the step 6, 7a, 9, 10a and 12 of the procedure.

-	In the step3 in clause 4.3.4.2, if the UE is in the CM-IDLE state over non-3GPP access and even “skip indicator” is not included in the Namf_Communication_N1N2MessageTransfer service operation, the AMF shall not send the N1 container to the UE. And the AMF shall acknowledges the step 3b by sending a Namf_Communication_N1N2MessageTransfer Response message ("N1 SM Message Not Transferred") with corresponding cause value, to SMF and steps 4 to 10 are skipped.

***** End of Changes *****