SA WG2 Meeting #128	S2-187915

Sophia Antipolis, France, August 20-24, 2018	(updated revision of S2-18xxxx)

Source:	Nokia, Nokia Shanghai Bell

Title:	update of solution 6.3 on UE awareness of coexistence rules.

Document for:	Agreement

Agenda Item:	6.21

Work Item / Release:	FS_eNS / Rel-16

Abstract of the contribution: This contribution proposes a solution update to for the solution in clause 6.3

# 1.	Proposal

It is proposed the following solution update to the solution in clause 6.3 is added to TR 23.740

***********************PROPOSED TEXT********************************

### 6.3.3	Procedures

This solution impacts existing procedures by inclusion of the Mutual Exclusion Class Information in association of every S-NSSAI in the configured NSSAI and Allowed NSSAI when these are provided to the UE by the AMF in Registration Accept or in the UE configuration update. Other than this, there is no impact except behavioural in the nodes which have to evaluate e.g. the allowed NSSAI.

Mutual Exclusion Class Information is also provided alongside subscriber information in the interaction between UDM and AMF, and between AMF and NSSF, for the determination of suitable allowed NSSAI which complies to Mutual exclusion constraints that are part of Subscriber data.

***********************END of PROPOSED TEXT***************************