SA WG2 Meeting #131 	S2-1901539

# 25 February – 1 March 2019, Santa Cruz - Tenerife, Spain		(revision of S2-19xxxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1012 | rev | - | Current version: | 15.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Access Network Performance Measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2019-02-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR introduces the Access Performance Measurements related call flows for ATSSS. It includes the two agreed measurements i.e. RTTmeasurements and the reporting of the UE initiated Access Availability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Under the new clause on Access Network Performarnce Measurements, two subclauses are added: - A subclause describing how RTT is measured via using Echo Request / Echo Response between UE and UPF PMF functions. - A subsclause describing the reporting of the Access Availaibility/Unavailability, when detected by the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No RTT measurements would be possible, and this would prevent the UPF and the UE to switch the traffic between accesses with steering mode "smallest delay". No Access Availability indication would be possible from the UE, which would result in too late switching of traffic by the UPF in case of loss of an access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 4.x.4 with subclauses 4.x.4.1 and 4.x.4.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 4.x.4	Access Network Performance Measurements

#### 4.x.4.1	Round Trip Time measurement procedure

The RTT is measured independently by the UE or by the UPF according to the procedure described below.

Figure 4.x.4.1-1:	RTT measurement procedure initiated by the UE

- The UE has received ATSSS rules that define the periodicity P of such measurement and how a RTT measure is performed (i.e. one message every S seconds for N times).

Editor's Note:	Whether the 5QI value used to convey the Echo Request / Echo Response messages is determined by an ATSSS rule or is pre-configured in both UE and UPF is FFS.

1.	Based on the ATSSS rules related to RTT, the UE decides to perform RTT measurements. It sends a PMF Echo Request message and the UPF replies with Echo Response. This is repeated N times, every S seconds. RTT is derived at the UE side.

2.	Based on the rules received over N4, the UPF may decide to wait for the UE for starting RTT measurements (to avoid paging the UE, thus saving battery) or to perform these measurements at any time. The procedure is same as in step 1.

NEXT CHANGE

#### 4.x.4.2	Access Availability/Unavailability reporting

Figure 4.x.4.2-1:	Access availability reporting by the UE

1.	Upon detection of the access unavailability (e.g. WLAN radio conditions, Fixed access failure), the UE decides to send an indication that the access has become unavailable by sending a Measurement Report (Measurement Type = Access Availability, Status = Unavailable, Access Type = non-3GPP). The Measurement Report is sent over the 3GPP access. The UPF may take this indication into account to decide whether to switch the traffic to the 3GPP access.

2.	Upon detection of the access availability, the UE decides to send an indication that the access has become available by sending a Measurement Report (Measurement Type = Access Availability, Status = Available, Access Type = non-3GPP). The Measurement Report is sent over the non-3GPP access. The UPF may take this indication into account to decide whether to switch the traffic to the 3GPP access

END OF CHANGES