3GPP TSG-SA WG2 Meeting #132	S2-1903948

8th April – 12th April, 2019, Xi’an, China	(revision of S2-1903686)

Source:	Samsung

Title:	Update on 6.11 UE Communication Analytics

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA/ Rel-16

Abstract of the contribution: This paper proposes the text changes for clause 6.11 of TS 23.288.

1. Introduction

This contribution provides text update for 6.11 UE Communication Analytics to propose the following changes:

1. The support for UE Communication analytics by the NWDAF should not be mandated

2. Input data from AF’s are updated to resolve the EN.

Editor's Note:	The exact information carried in the UE communication is FFS.

3. Editorial clean-up to have consistent use of terminology in the document

2. Proposal

It is proposed to adopt the following text changes in TS 23.288.

**** Start of 1st Changes ****

6.11	UE Communication Analytics

6.11.1	General

In order to support some optimized operations, e.g. customized mobility management, in 5GS, an NWDAF may perform data analytics on UE communication pattern and provide the analytics results (i.e. UE communication statistics or prediction) to NFs in the 5GC.

An NWDAF supporting UE Communication Analytics collects per-application communication description from AFs.

6.11.2	Input Data

The detailed information collected by the NWDAF includes service data related to UE communication as defined in the Table 6.11.2-1.

Table 6.11.2-1: Service Data from AF related to UE communication

| Information | Description |
| --- | --- |
| UE ID | Could be GPSI or external UE ID |
| Application ID | Identifying the application providing this information |
| UE communication (1..max) | Communication description per application |

Editor's Note:	The exact information carried in the UE communication is FFS.

6.11.3	Output Analytics

The NWDAF sup	orting UE Communication Analytics provides the analytics results to consumer NFs. The analytic results provided by the NWDAF includes the UE communication statistics or predictions as defined in the Table 6.11.3-1;

Table 6.11.3-1: UE Communication Analytics

| Information | Description |
| --- | --- |
| UE ID | Internal UE ID, i.e., SUPI |
| UE communications (1..max) |  |
| > Periodic communication indicator | Identifies whether the UE communicates periodically or not |
| > Periodic time | Interval Time of periodic communication |
| > Scheduled communication time | Time zone and Day of the week when the UE may be available for communication |
| > Communication duration time | Duration interval time of periodic communication or the scheduled communication |
| > Traffic Profile | Identifies the traffic type: MO/MT, single packet or multiple packets, etc. |
| > Confidence | Confidence of the prediction if this is a communication pattern prediction |

### 6.11.4	Procedures

The NWDAF can provide UE communication related analytics, in the form of statistics or predictions or both, to a 5GC NF.

Figure 6.11.4-1: Procedure for UE communication analytics

1.	5GC NF to NWDAF: Nnwdaf_AnalyticsInfo_Request (Analytic ID=UE communication, Analytic Filter=SUPI) or Nnwdaf_ AnalyticsSubscription_Subscribe (Analytic ID=UE communication, Analytic Filter=SUPI).

5GC NF sends a request to the NWDAF for analytics on a specific UE(s), using either Nnwdaf_AnalyticsInfo or Nnwdaf_ AnalyticsSubscription_Subscribe service. The analytic type indicated by "Analytic ID" is set to UE communication. The analytic target indicated by Analytic Filter is set to SUPI or a list of SUPI.

2.	NWDAF to AF (Optional): Naf_ EventExposure_Subscribe (Event ID, external UE ID).

In order to provide the requested analytics, the NWDAF may subscribe per application communication information from AFs for the UE. The Event ID indicates communication report for the UE which is requested by the 5GC NF in the step 1. The external UE ID is obtained by the NWDAF based on UE internal ID, i.e., SUPI.

This step is skipped if the NWDAF already has the requested analytics available or has subscribed to the AF.

3.	The NWDAF derives requested analytics, in the form of UE communication statistics or predictions or both.

4.	NWDAF to 5GC NF: Nnwdaf_AnalyticsInfo_Response or Nnwdaf_ AnalyticsSubscription_Notify.

The NWDAF provides requested UE communication analytics to the NF, using either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_ AnalyticsSubscription _Notify, depending on the service used in step 1.

5-6. If the NF subscribed UE communication analytics at step 1, when the NWDAF generates new analytics, it provides the new generated analytics to the NF.

**** End of 1st Changes ****